MATERIAL SAFETY DATA SHEET

ACETATE BUFFER SOLUTION pH 4.6

MSDS CAS:

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: ACETATE BUFFER SOLUTION pH 4.6 
CAS#: -

Synonym: Not Available

Chemical Name: Not Available

Chemical Formula: Not Available

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Substance / Preparation			:  Preparation.

Hazardous component(s) : This product is not hazardous but contains hazardous components.

| Substance name | CAS No | % |
| --- | --- | --- |
| Water | 7732-18-5 | >=99 |
| Sodium acetate | 127-09-3 | 0.1-1 |
| Acetic acid | 64-19-7 | 0.01-1 |

Section 2: Composition and Information on Ingredients (Continued)

Section 3: Hazards Identification

Classification of the substance or mixture

Classification EC 67/548 or EC 1999/45: Not classified.

Hazard Class and Category Code(s), Regulation (EC) No 1272/2008 (CLP): Not classified.

Label elements

Labelling EC 67/548 or EC 1999/45: Not classified.

Symbol(s)		 : None.

R Phrase(s) 		: None.

S Phrase(s) 		: None.

Contains		: Water - Sodium acetate - Acetic acid

Labelling Regulation EC 1272/2008 (CLP): Not classified.

Contains		: Water - Sodium acetate - Acetic acid.

Other hazards: The substance does not fulfil the criteria to be identified as PBT substance or vPvB

substance according to Annex XIII of Regulation REACH.

Section 4: First Aid Measures

Description of first aid measures

Inhalation 		: Assure fresh air breathing. Allow the victim to rest.

Skin contact 	: Remove affected clothing and wash all exposed skin area with mild soap and water, followed by 			   warm water rinse.

Eye contact		 : Rinse immediately with plenty of water. Obtain medical attention if pain, blinking or

redness persist.

Ingestion 		: Obtain emergency medical attention. Do NOT induce vomiting. Rinse mouth.

Most important symptoms and effects, both acute and delayed

Symptoms relating to use: Not expected to present a significant hazard under anticipated conditions of 				     normal use.

Indication of any immediate medical attention and special treatment needed

General information: Never give anything by mouth to an unconscious person. If you feel unwell, seek

medical advice (show the label where possible).

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media 		: Foam. Dry powder. Carbon dioxide. Water spray. Sand.

Unsuitable extinguishing media 	: Do not use a heavy water stream.

Surrounding fires				 : Use water spray or fog for cooling exposed containers.

Special hazards arising from the substance or mixture

Hazardous combustion products 	: Under fire conditions, hazardous fumes will be present.

Advice for fire-fighters

Protection against fire : Do not enter fire area without proper protective equipment, including respiratory

protection.

Special procedures:  Exercise caution when fighting any chemical fire. Avoid (reject) fire-fighting water to

enter environment.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures

For emergency responders	 : Equip cleanup crew with proper protection. Ventilate area.

For non-emergency personnel : Evacuate unnecessary personnel.

Environmental precautions:   Prevent entry to sewers and public waters. Notify authorities if product 				            enters sewers or public waters.

Methods and material for containment and cleaning up

Clean up methods		: Soak up spills with inert solids, such as clay or diatomaceous earth as 					soon as possible. Store away from other materials. Collect spillage.

Section 7: Handling and Storage

Precautions for safe handling

Handling 		: Wash hands and other exposed areas with mild soap and water before eat, drink or

smoke and when leaving work.

Technical protective measures: Provide good ventilation in process area to prevent formation of vapour.

Conditions for safe storage, including any incompatibilities

Storage		 : Keep only in the original container in a cool, well ventilated place. Keep container 				   closed when not in use.

Storage - away from : Strong bases. Strong acids. Sources of ignition. Direct sunlight.

Specific end use(s)    : None.

Section 8: Exposure Controls/Personal Protection

Exposure controls

Personal protection 	: Avoid all unnecessary exposure.

• Respiratory protection : Wear approved mask.

Section 8: Exposure Controls/Personal Protection (Continued)

• Hand protection	          : Wear protective gloves.

• Eye protection 		: Chemical goggles or safety glasses.

• Others 			: When using, do not eat, drink or smoke.

Control parameters

Occupational Exposure Limits : No data available.

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Physical state at 20 °C 			: Liquid.

Colour 					: Colorless

Odour					: Characteristic.

Odour threshold				: No data available.

pH value 			          		: No data available.

Melting point [°C]			: No data available.

Decomposition point [°C]		: N/A

Critical temperature [°C] 		: No data available.

Auto-ignition temperature [°C] 	: No data available.

Flammability (solid, gas) 		: N/A

Flash point [°C] 			  	: No data available.

Boiling point [°C]			  	: N/A

Initial boiling point [°C] 	  	: No data available.

Final boiling point [°C] 		  	: No data available.

Evaporation rate			   	: N/A

Vapour pressure [20°C]		  	: No data available.

Vapour pressure mm/Hg	    	: N/A

Vapour density			     	: N/A

Density [g/cm3] 			     	: 1

Relative density, gas (air=1)	     	: N/A

Relative density, liquid (water=1) 	: N/A

Solubility in water [% weight]      	: N/A

Solubility in water		       	: No data available.

Section 9: Physical and Chemical Properties (Continued)

Log Pow octanol / water at 20°C    	: No data available.

Solubility 					: 4.7 - 4.9

Viscosity at 40°C [mm2/s]        	: No data available.

Other information

Explosive properties 			: N/A

Explosion limits - upper [%] 		: No data available.

Explosion limits - lower [%] 		: No data available.

Oxidising properties 			: No data available.

Section 10: Stability and Reactivity Data

Reactivity

Reactivity 			: Not established.

Chemical stability

Chemical stability 	: Stable under recommended storage conditions.

Possibility of hazardous reactions

Hazardous reactions	 : Not established.

Conditions to avoid 	:Direct sunlight. Extremely high or low temperatures.

Materials to avoid 	: Strong acids. Strong bases.

Hazardous decomposition products: Fumes. Carbon monoxide. Carbon dioxide.

Section 11: Toxicological Information

Information on toxicological effects

Toxicity information 	:The product has been not fully tested. The calculated risk has been done under the

requirements of the EU regulations.

Section 11: Toxicological Information (Continued)

Acute toxicity

• Inhalation 		: Based on available data, the classification criteria are not met.

• Dermal 			: Based on available data, the classification criteria are not met.

• Ingestion 			: Based on available data, the classification criteria are not met.

Corrosion 			: Based on available data, the classification criteria are not met.

Irritation 			: Based on available data, the classification criteria are not met.

Sensitization 		: Based on available data, the classification criteria are not met.

Mutagenicity		 : Based on available data, the classification criteria are not met.

Carcinogenicity 		 : Based on available data, the classification criteria are not met.

Toxic for reproduction 	 : Based on available data, the classification criteria are not met.

STOT-single exposure	 : Based on available data, the classification criteria are not met.

STOT-repeated exposure : Based on available data, the classification criteria are not met.

Aspiration hazard 	 : Based on available data, the classification criteria are not met.

Section 12: Ecological Information

Toxicity

Toxicity information : The product has been not fully tested. The calculated risk has been done under the

requirements of the EU regulations.

Persistence - degradability

Persistence - degradability : Biodegradable.

Bioaccumulative potential   : Not established.

Mobility in soil		      : Not established.

Results of PBT and vPvB assessment: The substance does not fulfil the criteria to be identified as PBT substance or vPvB substance according to Annex XIII of Regulation REACH.

Other adverse effects

Environmental precautions : Avoid release to the environment.

Section 13: Disposal Considerations

Waste treatment methods

General 		: Avoid release to the environment. Dispose in a safe manner in accordance with 				   local/national regulations.

Section 14: Transport Information

Land transport (ADR-RID)

General information : Not regulated.

Sea transport (IMDG) [English only]

General information : Not regulated.

Air transport (ICAO-IATA) [English only]

General information : Not regulated.

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture

Safety, health and environmental 	: Ensure all national/local regulations are observed.

regulations/legislation specific for the

substance or mixture

REACH Restrictions - Annex XVII : The components of this product are not subject to restrictions.

REACH Authorisation - Annex XIV : The components of this product are not subject to authorization.

Chemical Safety Assessment : It has not been carried out.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.