MATERIAL SAFETY DATA SHEET

ACETONE CYANOHYDRIN

(For Synthesis)

MSDS CAS: 75-86-5

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: ACETONE CYANOHYDRIN

CAS#: 75-86-5

Synonym: Not Available.

Chemical Name: ACETONE CYANOHYDRIN.

Chemical Formula: C4H7NO

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | EC no | Annex I Index Number | Concentration % |
| --- | --- | --- | --- | --- |
| ACETONE CYANOHYDRIN | 75-86-5 | 200-909-4 | 608-004-00-X | 100 |

Toxicological Data on Ingredients: See Section No. 11.

Section 3: Hazards Identification

SPECIAL INDICATION OF HAZARDS TO HUMANS AND THE ENVIRONMENT

- Very toxic by inhalation, in contact with skin and if swallowed.

- Very toxic to aquatic organisms.

Section 4: First Aid Measures

AFTER INHALATION

- If inhaled, remove to fresh air. If not breathing give

- artificial respiration. If breathing is difficult, give oxygen.

Section 5: Fire and Explosion Data

EXTINGUISHING MEDIA

- Suitable: Carbon dioxide, dry chemical powder, or appropriate foam.

SPECIAL RISKS

- Specific Hazard(s): Emits toxic fumes under fire conditions.

SPECIAL PROTECTIVE EQUIPMENT FOR FIREFIGHTERS

- Wear self-contained breathing apparatus

Section 6: Accidental Release Measures

PERSONAL PRECAUTION PROCEDURES TO BE FOLLOWED IN CASE OF LEAK OR SPILL

Evacuate area.

PROCEDURE(S) OF PERSONAL PRECAUTION(S)

- Wear self-contained breathing apparatus, rubber boots, and heavy  rubber gloves.

Section 6: Accidental Release Measures (Continued)

METHODS FOR CLEANING UP

- Absorb on sand or vermiculite and place in closed containers for disposal.

Section 7: Handling and Storage

HANDLING

- Directions for Safe Handling: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

STORAGE

- Conditions of Storage: Keep tightly closed. Keep away from heat  and open flame. Store in a cool dry place.

Section 8: Exposure Controls/Personal Protection

Engineering Controls:

- Safety shower and eye bath. Use only in a chemical fume hood.

PERSONAL PROTECTIVE EQUIPMENT:

- Special Protective Measures: Wear appropriate government approved respirator, chemical-resistant gloves, safety goggles, other  protective clothing.
.

GENERAL HYGIENE MEASURES

- Wash thoroughly after handling. Wash contaminated clothing before reuse.

Exposure Limits:

Section 8: Exposure Controls/Personal Protection (Continued)

DENMARK

Source 	Type 		Value

OEL 		TWA 		3.5 mg/m3

1 ppm

Remarks: H

Section 9: Physical and Chemical Properties

Physical state and appearance	: Liquid.

Form						: Clear liquid.
Taste						: Not available.

Molecular Weight			: Not available.

Color						: Light yellow.

pH (1% soln/water)			: Not available.

Boiling Point				: 68 - 70 °C

Melting Point				: -19 °C

Critical Temperature			: Not available.

Specific Gravity				: Not available

Vapor Pressure				: Not available.

Vapor Density				: Not available.

Volatility					: Not available.

Odor Threshold				: Not available.

Water/Oil Dist. Coeff.			: Not available.

Ionicity (in Water)			: Not available..
Dispersion Properties			: Not available.

Solubility					: Not available.

Explosion Limits 				:Lower : 2.25 %

Upper: 11 %

SG/Density					: 0.93 g/cm3

Section 10: Stability and Reactivity Data

STABILITY

- Materials to Avoid: Strong acids, Strong bases, Strong oxidizing agents, Strong reducing agents.

Section 10: Stability and Reactivity Data (Continued)

HAZARDOUS DECOMPOSITION PRODUCTS

- Hazardous Decomposition Products: Carbon monoxide, Carbon dioxide, Acetone.

HAZARDOUS POLYMERIZATION

- Hazardous Polymerization: Will not occur

Section 11: Toxicological Information

RTECS NUMBER: OD9275000

ACUTE TOXICITY

LD50

Oral

Rat

18.65 mg/kg

LD50

Subcutaneous

Rat

8500 UG/KG

Remarks: Blood:Other changes.

LD50

Oral

Mouse

1.898 mg/kg

LD50

Intraperitoneal

Mouse

1 MG/KG

LD50

Oral

Rabbit

13.5 mg/kg

Section 11: Toxicological Information (Continued)

LD50

Skin

Rabbit

17 UL/KG

LD50

Oral

Guinea pig

9 mg/kg

LD50

Skin

Guinea pig

150 mg/kg

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing, wheezing, laryngitis, shortness of breath, headache, nausea, and vomiting. May cause cyanosis (blue-gray coloring of skin and lips caused by lack of oxygen). Exposure can cause: Convulsions.

ROUTE OF EXPOSURE

Multiple Routes: May be fatal if inhaled, swallowed, or absorbed through skin. High concentrations are extremely destructive to tissues of the  mucous membranes and upper respiratory tract, eyes, and skin.

TARGET ORGAN INFORMATION

Central nervous system. Blood. Heart. Liver. Kidneys.

Section 12: Ecological Information

Not available.

Section 13: Disposal Considerations

SUBSTANCE DISPOSAL: This combustible material may be burned in a chemical incinerator equipped with an afterburner and scrubber. Observe all federal,  state, and local environmental regulations

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name		   : Acetone cyanohydrin, stabilized

UN N° 		       		   : 1541

ADR - Class	         		   : 6.1

PG				                : I

Sea transport (IMDG) [English only]

Proper shipping name 		   : ACETONE CYANOHYDRIN, STABILIZED
UN N°		      		   : 1541

Class					   : 6.1
PG					   : I

Marine Pollutant			   : Yes

Severe Marine Pollutant	   : No

Air transport (ICAO-IATA) [English only

Proper shipping name 		   : Acetone cyanohydrin, stabilized.

UN N°		      		   : 1541
Class					   : 6.1
PG					   : I

Inhalation Packing Group I	   : Yes

Section 15: Other Regulatory Information

CLASSIFICATION AND LABELING ACCORDING TO EU DIRECTIVES

ANNEX I INDEX NUMBER: 608-004-00-X

INDICATION OF DANGER: T+ N

Very toxic. Dangerous for the environment.

R-PHRASES: 26/27/28 50

Very toxic by inhalation, in contact with skin and if swallowed. Very toxic to aquatic organisms.

S-PHRASES: 7/9 27 45 60 61

Keep container tightly closed and in well-ventilated place. Take off immediately all contaminated clothing. In case of

accident or if you feel unwell, seek medical advice immediately (show the label where possible). This material and its

Section 15: Other Regulatory Information (Continued)

container must be disposed of as hazardous waste. Avoid release to the environment. Refer to special instructions/safety data sheets.

COUNTRY SPECIFIC INFORMATION

Germany

- WGK: 3

SWITZERLAND

- SWISS POISON CLASS: 1

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.