MATERIAL SAFETY DATA SHEET

ACETYL ACETONE 99.5% AR

MSDS CAS: 123-54-6

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: ACETYL ACETONE 99.5% AR

CAS#:  123-54-6

Synonym: Acetoacetone; diacetylmethane; 2,4-pentanedione

Chemical Name:

Chemical Formula: CH3COCH2COCH3

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight | Hazardous |
| --- | --- | --- | --- |
| ACETYL ACETONE 99.5% AR | 123-54-6 | 99.5 | Yes |

Section 3: Hazards Identification

Emergency Overview

--------------------------

WARNING! FLAMMABLE LIQUID AND VAPOR. HARMFUL IF SWALLOWED, INHALED OR

ABSORBED THROUGH SKIN. CAUSES IRRITATION TO SKIN, EYES AND RESPIRATORY

TRACT. AFFECTS CENTRAL NERVOUS SYSTEM.

(tm) SAF-T-DATA Ratings (Provided here for your convenience)

-----------------------------------------------------------------------------------------------------------

Health Rating: 2 - Moderate (Poison)

Flammability Rating: 2 - Moderate

Reactivity Rating: 1 - Slight

Contact Rating: 3 - Severe

Lab Protective Equip: GOGGLES & SHIELD; LAB COAT & APRON; VENT HOOD; PROPER GLOVES;

CLASS B EXTINGUISHER

Storage Color Code: Red (Flammable)

Potential Health Effects

2,4-Pentanedione produces a clinically, anatomically, and morphologically distinct neurotoxic response which

differs from that produced by the other ketones. (Patty, et al)

Inhalation:

Inhalation of vapors produces a narcotic effect. Symptoms of exposure may include drowsiness and irritation of  the respiratory passages. Greater exposure may cause headache, dizziness, nausea, vomiting, and loss of

consciousness.

Ingestion:

Symptoms may include irritation to the gastrointestinal tract and other symptoms paralleling inhalation

exposure. May produce irreversible neurological impairment and possibly death.

Skin Contact:

Causes irritation. May be absorbed through the intact skin in harmful amounts.

Eye Contact:

Vapors are irritating to the eyes. Splashes are severely irritating with pain, redness.

Chronic Exposure:

Repeated exposure to low concentrations may be cumulative.

Aggravation of Pre-existing Conditions:

Persons with pre-existing neurological disorders may be more susceptible to the effects of the substance.

Section 4: First Aid Measures

Inhalation: Remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen.  Call a physician.

Ingestion: Give large amounts of water to drink. Never give anything by mouth to an unconscious person. Get medical attention.

Skin Contact:

Remove any contaminated clothing. Wash skin with soap or mild detergent and water for at least 15 minutes.  Wash clothes before reuse. Get medical attention if irritation develops or persists.

Eye Contact:

Immediately flush eyes with plenty of water for at least 15 minutes, lifting lower and upper eyelids occasionally.  Get medical attention immediately.

Section 5: Fire and Explosion Data

Fire:

Flash point: 34C (93F) CC

Autoignition temperature: 335C (635F)

Flammable limits in air % by volume:

lel: 2.4; uel: 11.6

Flammable Liquid and Vapor!

Explosion:

Above flash point, vapor-air mixtures are explosive within flammable limits noted above. Vapors can flow  along surfaces to distant ignition source and flash back. Sensitive to static discharge.

Fire Extinguishing Media:

Dry chemical, alcohol foam or carbon dioxide. Water spray may be used to keep fire exposed containers cool.

Special Information:

In the event of a fire, wear full protective clothing and NIOSH-approved self-contained breathing apparatus with full facepiece operated in the pressure demand or other positive pressure mode. This highly flammable liquid must be kept from sparks, open flame, hot surfaces, and all sources of heat and ignition.

Section 6: Accidental Release Measures

Ventilate area of leak or spill. Remove all sources of ignition. Wear appropriate personal protective equipment  as specified in Section 8. Isolate hazard area. Keep unnecessary and unprotected personnel from entering.  Contain and recover liquid when possible. Use non-sparking tools and equipment. Collect liquid in an  appropriate container or absorb with an inert material (e. g., vermiculite, dry sand, earth), and place in a chemical  waste container. Do not use combustible materials, such as saw dust. Do not flush to sewer! If a leak or spill has not ignited, use water spray to disperse the vapors, to protect personnel attempting to stop leak, and to flush  spills away from exposures.

Section 7: Handling and Storage

Protect against physical damage. Store in a cool, dry well-ventilated location, away from any area where the fire  hazard may be acute. Outside or detached storage is preferred. Separate from incompatibles. Containers should be bonded and grounded for transfers to avoid static sparks. Storage and use areas should be No Smoking areas. Use non-sparking type tools and  equipment, including explosion proof ventilation. Containers of this material may be hazardous when empty since they retain product residues (vapors, liquid); observe all warnings and precautions listed for the product.

Section 8: Exposure Controls/Personal Protection

Airborne Exposure Limits:

None established.

Ventilation System:

A system of local and/or general exhaust is recommended to keep employee exposures as low as possible. Local  exhaust ventilation is generally preferred because it can control the emissions of the contaminant at its source,  preventing dispersion of it into the general work area. Please refer to the ACGIH document, Industrial Ventilation, A Manual of Recommended Practices, most recent edition, for details.

Personal Respirators (NIOSH Approved):

For conditions of use where exposure to the substance is apparent, consult an industrial hygienist. For

emergencies, or instances where the exposure levels are not known, use a full-facepiece positive-pressure,

air-supplied respirator. WARNING: Air purifying respirators do not protect workers in oxygen-deficient

atmospheres.

Skin Protection:

Rubber or neoprene gloves and additional protection including impervious boots, apron, or coveralls, as needed in areas of unusual exposure.

Section 8: Exposure Controls/Personal Protection (Continued)

Eye Protection:

Use chemical safety goggles and/or a full face shield where splashing is possible. Maintain eye wash fountain

and quick-drench facilities in work area.

Section 9: Physical and Chemical Properties

Physical state and appearance: Colorless to yellowish liquid.

Odor					: Pleasant odor.

Taste					: Not available

Molecular Weight		: 120.16 g/mole.

Color					: Colorless

pH (1% soln/water)		: Not available

Boiling Point			: 140C (284F)

Melting Point 			: -23C (-9F)

Specific Gravity			: 1.03 (Water = 1)

Vapor Pressure			: 7.0 @ 20C (68F)

Vapor Density			: 0.975 @20/20C (68F)

Volatility				: 100.

Odor Threshold			: Not available.

Water/Oil Dist. Coeff.		: Not available.

Ionicity (in Water)		: Not available.

Dispersion Properties		: Not available.

Solubility				: Soluble in water, 12% w/w

Section 10: Stability and Reactivity Data

Stability			 : Stable under ordinary conditions of use and storage.

Hazardous Decomposition Products: Carbon dioxide and carbon monoxide may form when heated to decomposition.

Hazardous Polymerization: Will not occur.

Incompatibilities: Oxidizing agents, zinc, copper, nickel and related alloys.

Conditions to Avoid: Heat, flame, ignition sources, air, light and incompatibles.

Section 11: Toxicological Information

Oral rat LD50: 55 mg/kg; skin rabbit LD50: 810 mg/kg; Irritation skin rabbit 488 mg open: mildly irritating; irritation eye rabbit 20 mg: severely irritating; inhalation rat LCLo: 1000 ppm/4H; investigated as a mutagen,  reproductive effector.

--------\Cancer Lists\------------------------------------------------------

---NTP Carcinogen---

Ingredient 				Known    Anticipated		 IARC Category

------------------------------------	 ----- ----------- -------------

2,4-Pentanedione (123-54-6)	 No		 No 			          None

Section 12: Ecological Information

Environmental Fate:

When released into the soil, this material may biodegrade to a moderate extent. When released into the soil, this  material is expected to leach into groundwater. When released into the soil, this material is expected to quickly evaporate. When released into water, this material may biodegrade to a moderate extent. This material has a log octanol-water partition coefficient of less than 3.0. This material is not expected to significantly bioaccumulate. When released into the air, this material may be moderately degraded by reaction with photochemically produced hydroxyl radicals. When released into the air, this material is expected to be readily removed from the atmosphere by wet deposition. When released into air, this material is expected to have a half-life between 10 and 30 days.

Environmental Toxicity:

96 Hr LC50 Pimephales promelas: 104 mg/L [flow-through];

96 Hr LC50 Lepomis macrochirus: 29 mg/L;

48 Hr EC50 Daphnia magna: 34.4 mg/L.

Section 13: Disposal Considerations

Whatever cannot be saved for recovery or recycling should be handled as hazardous waste and sent to a RCRA approved incinerator or disposed in a RCRA approved waste facility. Processing, use  or contamination of this product may change the waste management options. State and local disposal regulations

may differ from federal disposal regulations. Dispose of container and unused contents in accordance with

federal, state and local requirements.

Section 14: Transport Information

Land transport (ADR-RID

Proper shipping name : PENTANE-2,4-DIONE

UN N°		        : 2310

H.I. nr		         : 36

ADR - Class	         : 3

Sea transport (IMDG) [English only]

Proper shipping name 	: PENTANE-2,4-DIONE

UN N°			 : 2310

IMO-IMDG - Class or division : 3 : Flammable liquid. ( 6.1 : Toxic substances. )

IMO-IMDG - Packing group	  : III

Air transport (ICAO-IATA) [English only]

Proper shipping name 	: PENTANE-2,4-DIONE

UN N°			 : 2310

IATA- Class or division : 3 : Flammable liquid. ( 6.1 : Toxic substances. )

IATA- Packing group	  : III

Section 15: Other Regulatory Information

--------\Chemical Inventory Status - Part 1\---------------------------------

Ingredient 				TSCA EC Japan Australia

-----------------------------------------------	 ---- --- ----- ---------

2,4-Pentanedione (123-54-6) 	Yes        Yes          Yes        Yes

--------\Chemical Inventory Status - Part 2\---------------------------------

--Canada--

Ingredient 					Korea DSL NDSL Phil.

Section 15: Other Regulatory Information (Continued)

2,4-Pentanedione (123-54-6) 		 Yes     Yes     No        Yes

--------\Federal, State & International Regulations - Part 1\----------------

-SARA 302- ------SARA

313------

Ingredient					 RQ TPQ List Chemical Catg.

----------------------------------------- 		--- ----- ---- --------------

2,4-Pentanedione (123-54-6) 		No      No        No         No

--------\Federal, State & International Regulations - Part 2\----------------

-RCRA- -TSCA-

Ingredient 						CERCLA 261.33 8(d)

----------------------------------------- 			------ ------ ------

2,4-Pentanedione (123-54-6) 			No 	No 	No

Chemical Weapons Convention: No TSCA 12(b): Yes CDTA: Yes

SARA 311/312: Acute: Yes Chronic: Yes Fire: Yes Pressure: No

Reactivity: No (Pure / Liquid)

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.