MATERIAL SAFETY DATA SHEET

ALUMINUM IODIDE

MSDS CAS: 7784-23-8

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:ALUMINUM IODIDE

CAS#: 7784-23-8

Synonym:

Chemicals Name: Aluminum Hydroxide Powder

Chemical Formula:AlI3

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Aluminum Iodide | 7784-23-8 | 100 |

Toxicological Data on Ingredients: Not applicable.

Section 3: Hazards Identification

Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008 [EU-GHS/CLP]

Classification according to EU Directives 67/548/EEC or 1999/45/EC

Reacts violently with water. Causes burns. May cause sensitization by inhalation and skin contact.

Label elements

Labelling according Regulation (EC) No 1272/2008 [CLP]

Signal word 			Danger

Hazard statement(s)

H314 				Causes severe skin burns and eye damage.

H317 				May cause an allergic skin reaction.

Precautionary statement(s)

P280 				Wear protective gloves/ protective clothing/ eye protection/ face protection.

P305 + P351 + P338 IF 	IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

P310 				Immediately call a POISON CENTER or doctor/ physician.

Supplemental Hazard information (EU)

EUH014 			Reacts violently with water.

According to European Directive 67/548/EEC as amended.

R-phrase(s)

R14 			Reacts violently with water.

R34 			Causes burns.

R42/43 		May cause sensitization by inhalation and skin contact.

S-phrase(s)

S22 			Do not breathe dust.

S26 			In case of contact with eyes, rinse immediately with plenty of water and seek medical advice.

S36/37/39 		Wear suitable protective clothing, gloves and eye/face protection.

S45 			In case of accident or if you feel unwell, seek medical advice immediately

Section 4: First Aid Measures

Description of first aid measures

General advice: Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled: If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

In case of skin contact: Take off contaminated clothing and shoes immediately. Wash off with soap and plenty of water. Consult a physician.

In case of eye contact: Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed: Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Most important symptoms and effects, both acute and delayed

Material is extremely destructive to tissue of the mucous membranes and upper respiratory tract, eyes, and skin., spasm, inflammation and edema of the larynx, spasm, inflammation and edema of the bronchi, pneumonitis, pulmonary edema, burning sensation, Cough, wheezing, laryngitis, Shortness of breath,

Headache, Nausea

Indication of any immediate medical attention and special treatment needed

no data available

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media

Dry powder

Special hazards arising from the substance or mixture

Hydrogen iodide, Aluminum oxide

Advice for firefighters

Wear self-contained breathing apparatus for firefighting if necessary.

Further information

no data available

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures

Use personal protective equipment. Avoid dust formation. Avoid breathing vapors, mist or gas. Ensure adequate ventilation. Evacuate personnel to safe areas. Avoid breathing dust.

Environmental precautions

Do not let product enter drains.

Methods and materials for containment and cleaning up

Pick up and arrange disposal without creating dust. Sweep up and shovel. Do not flush with water. Keep in suitable, closed containers for disposal.

Section 7: Handling and Storage

Precautions for safe handling

Avoid contact with skin and eyes. Avoid formation of dust and aerosols.

Provide appropriate exhaust ventilation at places where dust is formed.

Conditions for safe storage, including any incompatibilities

Store in cool place. Keep container tightly closed in a dry and well-ventilated place.

Never allow product to get in contact with water during storage.

Light sensitive. Air sensitive.

Specific end use(s)

no data available

Section 8: Exposure Controls/Personal Protection

Control parameters

Components with workplace control parameters

Exposure controls

Appropriate engineering controls: Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday.

Personal protective equipment

Eye/face protection: Face shield and safety glasses Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

Section 8: Exposure Controls/Personal Protection (Continued)

Skin protection: Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices.Wash and dry hands.

Body Protection: Complete suit protecting against chemicals, Flame retardant protective clothing, and the type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace.

Respiratory protection:  Where risk assessment shows air-purifying respirators are appropriate use a full-face particle respirator type N100 (US) or type P3 (EN 143) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards suchas NIOSH (US) or CEN (EU).

Section 9: Physical and Chemical Properties

Physical state and appearance: Powder

Odor					: Not available.

Taste					: Not available.

Molecular Weight		: 407, 70 g/mol

Color					: White. Off-white.

pH (1% soln/water)		: Not applicable.

Boiling Point			: 360 °C - lit.

Melting Point			: 191 °C - lit.

Critical Temperature		: Not available.

Specific Gravity			: Not available.

Vapor Pressure			: 1 hPa at 178 °C

Vapor Density			: 3, 98 g/cm3 at 25 °C

Volatility				: Not available.

Odor Threshold			: Not available.

Water/Oil Dist. Coeff.		: Not available

Ionicity (in Water)		: Not available.

Dispersion Properties		: Not available.

Solubility				: Not available.

Section 10: Stability and Reactivity Data

Reactivity: no data available

Chemical stability: no data available

Possibility of hazardous reactions: Reacts violently with water.

Conditions to avoid: Exposure to moisture.

Incompatible materials: Water

Hazardous decomposition products: Other decomposition products - no data available

Section 11: Toxicological Information

Information on toxicological effects

Acute toxicity: no data available

Skin corrosion/irritation:no data available

Serious eye damage/eye irritation:no data available

Respiratory or skin sensitization: May cause allergic skin reaction.

Germ cell mutagenicity:no data available

Carcinogenicity

IARC: No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

Reproductive toxicity: no data available

Specific target organ toxicity - single exposure: no data available

Specific target organ toxicity - repeated exposure: no data available.

Aspiration hazard:no data available

Potential health effects

Inhalation:May be harmful if inhaled. Material is extremely destructive to the tissue of the mucous membranes and upper respiratory tract.

Ingestion:May be harmful if swallowed. Causes burns.

Skin:May be harmful if absorbed through skin. Causes skin burns.

Eyes: Causes eye burns.

Section 11: Toxicological Information (Continued)

Signs and Symptoms of Exposure

Material is extremely destructive to tissue of the mucous membranes and upper respiratory tract, eyes, and skin., spasm, inflammation and edema of the larynx, spasm, inflammation and edema of the bronchi, pneumonitis, pulmonary edema, burning sensation, Cough, wheezing, laryngitis, Shortness of breath,

Headache, Nausea

Section 12: Ecological Information

Toxicity:no data available

Persistence and degradability:no data available

Bioaccumulative potential:no data available

Mobility in soil:no data available

Results of PBT and vPvB assessment:no data available

Other adverse effects:no data available

Section 13: Disposal Considerations

Waste treatment methods

Product

Offer surplus and non-recyclable solutions to a licensed disposal company. Dissolve or mix the material with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and scrubber.

Contaminated packaging

Dispose of as unused product.

Section 14: Transport Information

UN number

ADR/RID: 3260 			IMDG: 3260 			IATA: 3260

UN proper shipping name

ADR/RID: 		CORROSIVE SOLID, ACIDIC, INORGANIC, N.O.S. (Aluminium iodide)

IMDG: 		CORROSIVE SOLID, ACIDIC, INORGANIC, N.O.S. (Aluminium iodide)

IATA: 		Corrosive solid, acidic, inorganic, n.o.s. (Aluminium iodide)

Section 14: Transport Information (Continued)

Transport hazard class (es)

ADR/RID: 8 				IMDG: 8 			IATA: 8

Packaging group

ADR/RID: II 				IMDG: II 			IATA: II

Environmental hazards

ADR/RID: no 		IMDG Marine Pollutant: no 	IATA: no

Special precautions for user

no data available

Section 15: Other Regulatory Information

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

Safety, health and environmental regulations/legislation specific for the substance or mixture

no data available

Chemical Safety Assessment

no data availabl

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.