MATERIAL SAFETY DATA SHEET

ALUMINIUM STEARATE

(Extra Pure)

MSDS CAS: 637-12-7

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:Aluminium Stearate

CAS#: 637-12-7

Synonym:Octadecanoic acid, aluminum salt.

Chemicals Name: Aluminium Stearate

Chemical Formula: Al(C18H35O2)3 (approx.)

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Aluminium Stearate | 637-12-7 | 100 |

Toxicological Data on Ingredients: Aluminum stearate LD50: Not available. LC50: Not available.

Section 3: Hazards Identification

Potential Acute Health Effects:Hazardous in case of eye contact (irritant), of ingestion. Slightly hazardous in case of skin contact (irritant), of inhalation.

Potential Chronic Health Effects:CARCINOGENIC EFFECTS: Not available. MUTAGENIC EFFECTS: Not available. TERATOGENIC EFFECTS: Not available. DEVELOPMENTAL TOXICITY: Not available. Repeated or prolonged exposure is not known to aggravate medical condition.

Section 4: First Aid Measures

Eye Contact:Check for and remove any contact lenses. In case of contact, immediately flush eyes with plenty of water for at least 15 minutes. Get medical attention.

Skin Contact: If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical attention.

Serious Skin Contact:Not available.

Inhalation:If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical attention.

Serious Inhalation:Not available.

Ingestion: Do NOT induce vomiting unless directed to do so by medical personnel. Never give anything by mouth to an unconscious person. If large quantities of this material are swallowed, call a physician immediately. Loosen tight clothing such as a collar, tie, belt or waistband.

Serious Ingestion: Not available.

Section 5: Fire and Explosion Data

Flammability of the Product:May be combustible at high temperature.

Auto-Ignition Temperature:Not available.

Flash Points:Not available.

Flammable Limits:Not available.

Section 5: Fire and Explosion Data (Continued)

Products of Combustion:These products are carbon oxides (CO, CO2). Some metallic oxides.

Fire Hazards in Presence of Various Substances:Not applicable.

Explosion Hazards in Presence of Various Substances: Risks of explosion of the product in presence of mechanical impact: Not available. Risks of explosion of the product in presence of static discharge: Not available.

Fire Fighting Media and Instructions: SMALL FIRE: Use DRY chemical powder. LARGE FIRE: Use water spray, fog or foam. Do not use water jet.

Special Remarks on Fire Hazards:Not available.

Special Remarks on Explosion Hazards:Not available.

Section 6: Accidental Release Measures

Small Spill: Use appropriate tools to put the spilled solid in a convenient waste disposal container. Finish cleaning by spreading water on the contaminated surface and dispose of according to local and regional authority requirements.

Large Spill: Use a shovel to put the material into a convenient waste disposal container. Finish cleaning by spreading water on the contaminated surface and allow to evacuate through the sanitary system.

Section 7: Handling and Storage

Precautions:Keep away from heat. Keep away from sources of ignition. Empty containers pose a fire risk, evaporate the residue under a fume hood. Ground all equipment containing material. Do not breathe dust. Avoid contact with eyes. Wear suitable protective clothing. If you feel unwell, seek medical attention and show the label when possible.

Storage:Keep container tightly closed. Keep container in a cool, well-ventilated area.

Section 8: Exposure Controls/Personal Protection

Engineering Controls:Use process enclosures, local exhaust ventilation, or other engineering controls to keep airborne levels below recommended exposure limits. If user operations generate dust, fume or mist, use ventilation to keep exposure to airborne contaminants below the exposure limit.

Personal Protection: Splash goggles. Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Gloves.

Section 8: Exposure Controls/Personal Protection (Continued)

Personal Protection in Case of a Large Spill: Splash goggles. Full suit. Dust respirator. Boots. Gloves. A self-contained breathing apparatus should be used to avoid inhalation of the product. Suggested protective clothing might not be sufficient; consult a specialist BEFORE handling this product.

Exposure Limits: Not available.

Section 9: Physical and Chemical Properties

Physical state and appearance: Solid. (Powdered solid.)

Odor					: Not available.

Taste					: Not available.

Molecular Weight		: 877.35 g/mole

Color					: White.

pH (1% soln/water)		: Not applicable.

Boiling Point			: Decomposes.

Melting Point			: 115°C (239°F)

Critical Temperature		: Not available.

Specific Gravity			: 1.07 (Water = 1)

Vapor Pressure			: Not applicable.

Vapor Density			: Not available.

Volatility				: Not available.

Odor Threshold			: Not available.

Water/Oil Dist. Coeff.		: Not available

Ionicity (in Water)		: Not available.

Dispersion Properties		: See solubility in water, n-octanol.

Solubility				: Partially soluble in n-octanol. Insoluble in cold water, hot water, methanol, diethyl ether.

Section 10: Stability and Reactivity Data

Stability:The product is stable.

Instability Temperature: Not available.

Conditions of Instability:Not available.

Incompatibility with various substances:Not available.

Corrosivity:Non-corrosive in presence of glass.

Section 10: Stability and Reactivity Data (Continued)

Special Remarks on Reactivity:Forms gel with aliphatic and aromatic hydrocarbons.

Special Remarks on Corrosively:Not available.

Polymerization:Will not occur.

Section 11: Toxicological Information

Routes of Entry:Eye contact, Ingestion.

Toxicity to Animals: LD50: Not available. LC50: Not available.

Chronic Effects on Humans: Not available.

Other Toxic Effects on Humans: Hazardous in case of ingestion. Slightly hazardous in case of skin contact (irritant), of inhalation.

Special Remarks on Toxicity to Animals: Not available.

Special Remarks on Chronic Effects on Humans: Not available.

Special Remarks on other Toxic Effects on Humans: Nuisance dust.

Section 12: Ecological Information

Ecotoxicity:Not available.

BOD5 and COD:Not available.

Products of Biodegradation: Possibly hazardous short term degradation products are not likely. However, long term degradation products may arise.

Toxicity of the Products of Biodegradation: The products of degradation are more toxic.

Special Remarks on the Products of Biodegradation:Not available.

Section 13: Disposal Considerations

Waste Disposal:

Section 14: Transport Information

Land transport (ADR-RID)

General Information: Regulated

Sea transport (IMDG) [English only]

General Information: Regulated

Air transport (ICAO-IATA) [English only]

General Information: Regulated

Section 15: Other Regulatory Information

Federal and State Regulations:TSCA 8(b) inventory: Aluminum stearate

Other Regulations:Not available.

Other Classifications:

WHMIS (Canada): Not controlled under WHMIS (Canada).

DSCL (EEC):R36- Irritating to eyes.

HMIS (U.S.A.):

Health Hazard:2

Fire Hazard:1

Reactivity:0

Personal Protection:E

National Fire Protection Association (U.S.A.):

Health:2

Flammability:0

Reactivity:0

Specific hazard:

Protective Equipment: Gloves. Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Splash goggles.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.