MATERIAL SAFETY DATA SHEET

ALUMINIUM SULPHATE AR

(Extra Pure)(18-Hydrate)

MSDS CAS: 7784-31-8

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:Aluminium Sulphate AR

CAS#: 7784-31-8

Synonym:Aluminum sulfate octadecahydrate; Cake alum octadecahydrate; Dialuminum sulfate

octadecahydrate; Patent alum; Dialuminumtrisulateoctadecahydrate; Aluminum sulfate (2:3) octadecahydrate; Aluminum trisulfateoctadecahydrate; Dialuminumsulfateoctadecahydrate

Chemicals Name: Sulfuric acid, aluminum salt (3:2),octadecahydrate

Chemical Formula: Al2 (SO4)3.(14-18)H2O

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Aluminum Sulfate, Hydrated (ACS & FCC) | 7784-31-8 | 100 |

Toxicological Data on Ingredients: Aluminum Sulfate, Hydrated (ACS & FCC): ORAL (LD50): Acute: >9000 mg/kg [Mouse]. >9000 mg/kg [Rat].

Section 3: Hazards Identification

Potential Acute Health Effects:Hazardous in case of skin contact (irritant), of eye contact (irritant), of inhalation (lung irritant). Slightly hazardous in case of ingestion.

Potential Chronic Health Effects:CARCINOGENIC EFFECTS: Not available. MUTAGENIC EFFECTS: Not available. TERATOGENIC EFFECTS: Not available. DEVELOPMENTAL TOXICITY: Classified Reproductive system/toxin/female, Reproductive system/toxin/male [SUSPECTED]. The substance may be toxic to the reproductive system, mucous membranes, skin, eyes, Urinary System. Repeated or prolonged exposure to the substance can produce target organs damage.

Section 4: First Aid Measures

Eye Contact:Check for and remove any contact lenses. In case of contact, immediately flush eyes with plenty of water for at least 15 minutes. Cold water may be used. Get medical attention.

Skin Contact: In case of contact, immediately flush skin with plenty of water. Cover the irritated skin with an emollient. Remove contaminated clothing and shoes. Cold water may be used. Wash clothing before reuse. Thoroughly clean shoes before reuse. Get medical attention.

Serious Skin Contact:Wash with a disinfectant soap and cover the contaminated skin with an anti-bacterial cream. Seek medical attention.

Inhalation:If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical attention.

Serious Inhalation:Not available.

Ingestion: Do NOT induce vomiting unless directed to do so by medical personnel. Never give anything by mouth to an unconscious person. Loosen tight clothing such as a collar, tie, belt or waistband. Get medical attention if symptoms appear.

Serious Ingestion: Not available.

Section 5: Fire and Explosion Data

Flammability of the Product:Non-flammable.

Auto-Ignition Temperature:Not applicable.

Section 5: Fire and Explosion Data (Continued)

Flash Points:Not applicable.

Flammable Limits:Not applicable.

Products of Combustion:Not available.

Fire Hazards in Presence of Various Substances:Not applicable.

Explosion Hazards in Presence of Various Substances: Risks of explosion of the product in presence of mechanical impact: Not available. Risks of explosion of the product in presence of static discharge: Not available.

Fire Fighting Media and Instructions: Not applicable.

Special Remarks on Fire Hazards:It may burn, but it will not ignite. Fire may produce irritating, corrosive and/or toxic gases.

Special Remarks on Explosion Hazards:Not available.

Section 6: Accidental Release Measures

Small Spill: Use appropriate tools to put the spilled solid in a convenient waste disposal container. Finish cleaning by spreading water on the contaminated surface and dispose of according to local and regional authority requirements.

Large Spill: Use a shovel to put the material into a convenient waste disposal container. Finish cleaning by spreading water on the contaminated surface and allow to evacuate through the sanitary system. Be careful that the product is not present at a concentration level above TLV. Check TLV on the MSDS and with local authorities.

Section 7: Handling and Storage

Precautions:Do not ingest. Do not breathe dust. Wear suitable protective clothing. In case of insufficient ventilation, wear suitable respiratory equipment. If ingested, seek medical advice immediately and show the container or the label. Avoid contact with skin and eyes. Keep away from incompatibles such as oxidizing agents.

Storage:Keep container tightly closed. Keep container in a cool, well-ventilated area.

Section 8: Exposure Controls/Personal Protection

Engineering Controls:Use process enclosures, local exhaust ventilation, or other engineering controls to keep airborne levels below recommended exposure limits. If user operations generate dust, fume or mist, use ventilation to keep exposure to airborne contaminants below the exposure limit.

Personal Protection: Splash goggles. Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Gloves.

Personal Protection in Case of a Large Spill: Splash goggles. Full suit. Dust respirator. Boots. Gloves. A self-contained breathing apparatus should be used to avoid inhalation of the product. Suggested protective clothing might not be sufficient; consult a specialist BEFORE handling this product.

Exposure Limits: TWA: 2 (mg (Al)/m) from ACGIH (TLV) [United States] TWA: 2 (mg (Al)/m) [United Kingdom (UK)] Consult local authorities for acceptable exposure limits.

Section 9: Physical and Chemical Properties

Physical state and appearance: Solid. (Crystals solid.)

Odor					: Odorless.

Taste					: Sweet. Mildly Astringent.

Molecular Weight		: 342.14 g/mole + (14-18)H2O

Color					: White.

pH (1% soln/water)		: 3-4.

Boiling Point			: Not available.

Melting Point			: 86 °C.

Critical Temperature		: Not available.

Specific Gravity			: Density: 1.69 @ 17 deg. C(Water = 1)

Vapor Pressure			: Not applicable.

Vapor Density			: Not available.

Volatility				: Not available.

Odor Threshold			: Not available.

Water/Oil Dist. Coeff.		: Not available

Ionicity (in Water)		: Not available.

Dispersion Properties		: See solubility in water.

Solubility				: Easily soluble in hot water. Soluble in cold water. It will hydrolyze in water to form sulfuric acid. Insoluble in alcohol. Solubility in water: 86.9 g/ 100 ml @0 deg. C; 1104 g/100 ml @ 100 deg. C

Section 10: Stability and Reactivity Data

Stability:The product is stable.

Instability Temperature: Not available.

Conditions of Instability:Incompatible materials, moisture

Incompatibility with various substances:Reactive with oxidizing agents.

Corrosivity:Non-corrosive in presence of glass.

Special Remarks on Reactivity:It melts when gradually heated; At 250 deg. C, it loses its water.

Special Remarks on Corrosively:May corrode metals in the presence of moisture

Polymerization:Will not occur.

Section 11: Toxicological Information

Routes of Entry:Inhalation. Ingestion.

Toxicity to Animals: Acute oral toxicity (LD50):>9000 mg/kg [Rat].

Chronic Effects on Humans: DEVELOPMENTAL TOXICITY: Classified Reproductive system/toxin/female, Reproductive system/toxin/male [SUSPECTED]. May cause damage to the following organs: the reproductive system, mucous membranes, skin, eyes, Urinary System.

Other Toxic Effects on Humans: Hazardous in case of skin contact (irritant), of inhalation (lung irritant). Slightly hazardous in case of ingestion.

Special Remarks on Toxicity to Animals: May affect genetic material (mutagenic). May cause adverse reproductive effects based on animal test data.

Special Remarks on Chronic Effects on Humans: Not available.

Special Remarks on other Toxic Effects on Humans: Acute Potential Health Effects: Skin: Causes skin irritation, particularly if moisture is present. Symptoms include redness, itching, and pain Eyes: Causes eye irritation. Symptoms include redness and pain. Inhalation: Causes mouth and respiratory tract irritation. Symptoms may include coughing, shortness of breath. It may cause airwayconstrictin in rare instances. Symptoms are usually transient. Ingestion: May cause irritation to the gastrointestinal tract. Symptoms may include cramping, nausea, vomiting, and diarrhea. Ingestion also produces a feeling of dryness and puckering of the mucous membranes of the mouth and throat. It may affect behavior/central nervous system and cause ataxia and seizures. High blood concentrations of aluminum may cause aluminum-induced encephalopathy with confusion, lethargy, respiratory depression, cognitive impairment, dysarthria, asterixis, seizure, coma. It may also affect the liver. Individuals with renal failure may more readily accumulate toxic levels of aluminum which can result in encephalopathy and seizures. Chronic Potential Health Effects: Skin: Repeated or prolonged skin contact may cause irritation, especially if moisture is present. Ingestion: Repeated or prolonged ingestion may affect metabolism, urinary system, blood (changes in serum composition - e.g. TP, bilirubin, cholesterol), skeletal system, and brain (degenerative changes). High blood concentrations of aluminum may cause aluminum to be deposited in the bones. Accumulation of aluminum in the bone appears to reduce the positive effects of vitamin D adn may prevent calcium deposition into the bones. The prevention of calcium deposition leads to the return of the calcium to the blood. This may cause bone/skeletalabnormalties, osteomalacia, painful joints. The elevated serum calcium levels in turn inhibit the release of parathyroid hormone by the parathyroid glands.

Section 12: Ecological Information

Ecotoxicity:Not available.

BOD5 and COD:Not available.

Products of Biodegradation: Possibly hazardous short term degradation products are not likely. However, long term degradation products may arise.

Toxicity of the Products of Biodegradation: The products of degradation are more toxic.

Special Remarks on the Products of Biodegradation:Not available.

Section 13: Disposal Considerations

Waste Disposal: Waste must be disposed of in accordance with federal, state and local environmental control regulations.

Section 14: Transport Information

Land transport (ADR-RID)

General Information: Regulated

Sea transport (IMDG) [English only]

General Information: Regulated

Air transport (ICAO-IATA) [English only]

General Information: Regulated

Section 15: Other Regulatory Information

Federal and State Regulations:Connecticut hazardous material Survey. Aluminum sulfate anhydrous (CAS no. 10043-01-3) Illinois chemical safety act: Aluminum sulfate anhydrous (CAS no. 10043-01-3) New York release reporting list: Aluminum sulfate anhydrous (CAS no. 10043-01-3) Pennsylvania RTK: Aluminum sulfate anhydrous (CAS no. 10043-01-3) Massachusetts RTK: Aluminum sulfate anhydrous (CAS no. 10043-01-3) Massachusetts spill list: Aluminum sulfate anhydrous (CAS no. 10043-01-3) New Jersey:

Section 15: Other Regulatory Information (Continued)

Aluminum sulfate anhydrous (CAS no. 10043-01-3) New Jersey spill list: Aluminum sulfate anhydrous (CAS no. 10043-01-3) Louisiana spill reporting: Aluminum sulfate anhydrous (CAS no. 10043-01-3) California Director's List of Hazardous Substances: Aluminum sulfate anhydrous (CAS no. 10043-01-3)- CERCLA: Hazardous substances.: Aluminum sulfate anhydrous (CAS no. 10043-01-3)-: 5000 lbs. (2268 kg). No Regulatory information found for Aluminum sulfate hydrate

(CAS no. 17927-65-0)

Other Regulations:OSHA: Hazardous by definition of Hazard Communication Standard (29 CFR 1910.1200).

Other Classifications:

WHMIS (Canada): CLASS D-2B: Material causing other toxic effects (TOXIC). Aluminum Sulfate anhydrous (CAS no. 10043-01-3) is on the Canadian DSL. Aluminum Sulfate hydrate (Cas no. 17927-65-0) is not on the Canadian DSL.

DSCL (EEC):R36/37/38- Irritating to eyes, respiratory system and skin. S24/25- Avoid contact with skin and eyes. S36/37/39- Wear suitable protective clothing, gloves and eye/face protection.

HMIS (U.S.A.):

Health Hazard:2

Fire Hazard:0

Reactivity:0

Personal Protection:E

National Fire Protection Association (U.S.A.):

Health:2

Flammability:0

Reactivity:0

Specific hazard:

Protective Equipment: Gloves. Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Wear appropriate respirator when ventilation is inadequate. Splash goggles.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.