MATERIAL SAFETY DATA SHEET

4-AMINO MORPHOLINE

MSDS CAS: 4319-49-7

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:4-AMINO MORPHOLINE

CAS#: 4319-49-7

C.I. No.:Not applicable.

Synonym:N-(2-Hydroxyethyl)ethylenediamine; N-(2-Aminoethyl)ethanolamine.

Chemical Name:

Chemical Formula: C4H10N2O

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| 4-AMINO MORPHOLINE | 4319-49-7 | 100 |

Section 3: Hazards Identification

Risk advice to man and the environment

Harmful by inhalation, in contact with skin and if swallowed. Irritating to eyes, respiratory system and skin.

Section 4: First Aid Measures

General advice: Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled: If breathed in, move person into fresh air. If not breathing give artificial respiration Consult a physician.

In case of skin contact: Wash off with soap and plenty of water. Consult a physician.

In case of eye contact: Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed: Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Section 5: Fire and Explosion Data

Suitable extinguishing media

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special protective equipment for fire-fighters

Wear self-contained breathing apparatus for firefighting if necessary.

Section 6: Accidental Release Measures

Personal precautions: Use personal protective equipment. Avoid breathing vapors, mist or gas. Ensure adequate ventilation.

Environmental precautions:Do not let product enter drains.

Methods for cleaning up: Soak up with inert absorbent material and dispose of as hazardous waste. Keep in suitable, closed containersfor disposal.

Section 7: Handling and Storage

Handling: Avoid contact with skin and eyes. Avoid inhalation of vap our or mist. Keep away from sources of ignition - No smoking. Take measures to prevent the buildup of electrostatic charge.

Storage: Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Containers which are opened must be carefully resealed and kept upright to prevent leakage.

Section 8: Exposure Controls/Personal Protection

Personal protective equipment

Respiratory protection: Where risk assessment shows air-purifying respirators are appropriate use a full-face respirator with multipurpose combination (US) or type ABEK (EN 14387) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH

(US) or CEN (EU).

Hand protection: The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it. Handle with gloves.

Eye protection: Face shield and safety glasses

Skin and body protection: Choose body protection according to the amount and concentration of the dangerous substance at the work place.

Hygiene measures: Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday.

Section 9: Physical and Chemical Properties

Physical state and appearance: Liquid

Odor					: Not available.

Taste					: Not available.

Molecular Weight		: 102.14 g/mole

Color					: colourless

pH (1% soln/water)		: Not available.

Boiling Point			: Not available.

Melting Point			: 168 °C - lit.

Flash Point				:58 °C - closed cup

Section 9: Physical and Chemical Properties (Continued)

Critical Temperature		: Not available.

Specific Gravity			: Not available.

Vapor Pressure			: Not applicable.

Density				: 1,059 g/mL at 25 °C

Volatility				: Not available.

Odor Threshold			: Not available.

Water/Oil Dist. Coeff.		: Not available

Ionicity (in Water)		: Not available.

Dispersion Properties		: Not available.

Solubility				: Not available.

Section 10: Stability and Reactivity Data

Storage stability

Stable under recommended storage conditions.

Materials to avoid

Strong oxidizing agentsacids, Acid chlorides, Acid anhydrides, Strong oxidizing agents, Carbon dioxide (CO2)

Hazardous decomposition products

Hazardous decomposition products formed under fire conditions. - Carbon oxides, nitrogen oxides (NOx)

Section 11: Toxicological Information

Acute toxicity: LD50 Oral - mouse - > 1.000 mg/kg

Inhalation: no data available

Irritation and corrosion: no data available

Sensitisation: no data available

Chronic exposure

IARC: No component of this product present at levels greater than or equal to 0.1% is identified asprobable, possible or confirmed human carcinogen by IARC.

Section 11: Toxicological Information (Continued)

Signs and Symptoms of Exposure:To the best of our knowledge, the chemical, physical, and toxicological properties have not been thoroughly investigated.

Potential Health Effects

InhalationHarmful if inhaled. Causes respiratory tract irritation.

SkinHarmful if absorbed through skin. Causes skin irritation.

EyesCauses eye irritation.

IngestionHarmful if swallowed.

Section 12: Ecological Information

Elimination information (persistence and degradability)

no data available

Ecotoxicity effects

no data available

Further information on ecology

no data available

Section 13: Disposal Considerations

Product: This combustible material may be burned in a chemical incinerator equipped with an afterburner and scrubber.Observe all federal, state, and local environmental regulations. Contact a licensed professional waste disposal service to dispose of this material.

Contaminated packaging

Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name: FLAMMABLE LIQUID, N.O.S. (Morpholin-4-ylamine)

UN-Number: 1993 Class: 3

packing group: III

Sea transport (IMDG) [English only]

Proper shipping name: FLAMMABLE LIQUID, N.O.S. (Morpholin-4-ylamine)

UN-Number: 1993 Class: 3

packing group: III

Air transport (ICAO-IATA) [English only]

Proper shipping name: FLAMMABLE LIQUID, N.O.S. (Morpholin-4-ylamine)

UN-Number: 1993 Class: 3

packing group: III

Section 15: Other Regulatory Information

Labelling according to EC Directives

Hazard symbols

Xn Harmful

R-phrase(s)

R20/21/22 Harmful by inhalation, in contact with skin and if swallowed.

R36/37/38 Irritating to eyes, respiratory system and skin.

S-phrase(s)

S26 In case of contact with eyes, rinse immediately with plenty of water and seek medical advice.

S36 Wear suitable protective clothing.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.