MATERIAL SAFETY DATA SHEET

3-AMINO PYRIDINE

(For Synthesis)  (m-Amino pyridine)

MSDS CAS: 462-08-6

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: 3-AMINO PYRIDINE

CAS#: -462-08-6

Synonym:3-Pyridylamine

Chemical Name: Not available.

Chemical Formula: C5H6 N2

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

| Component | CAS-No. | Concentration |
| --- | --- | --- |
| 3-Pyridylamine | 462-08-8 | <= 100 % |

Section 3: Hazards Identification

Label elements

Labelling according Regulation (EC) No 1272/2008

Signal word 			Danger

Hazard statement(s)

H301 + H311 			Toxic if swallowed or in contact with skin

H315 				Causes skin irritation.

H319 				Causes serious eye irritation.

H335 				May cause respiratory irritation.

Precautionary statement(s)

P261 				Avoid breathing dust.

P280 				Wear protective gloves/ protective clothing.

P301 + P310 IF SWALLOWED: Immediately call a POISON CENTER or doctor/ physician.

P305 + P351 + P338 	IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

P312 				Call a POISON CENTER or doctor/ physician if you feel unwell.

Supplemental Hazard	: none

Statements

Other hazards–none

Section 4: First Aid Measures

Description of first aid measures

General advice:Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled:If breathed in, move person into fresh air. If not breathing, give artificial respiration.Consult a physician.

In case of skin contact:Wash off with soap and plenty of water. Take victim immediately to hospital. Consult a physician.

In case of eye contact:Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed:Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Section 4: First Aid Measures (Continued)

Most important symptoms and effects, both acute and delayed

Indication of any immediate medical attention and special treatment needed

no data available

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media:Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special hazards arising from the substance or mixture:Carbon oxides, nitrogen oxides (NOx).

Advice for firefighters: Wear self-contained breathing apparatus for firefighting if necessary.

Further information:no data available

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Wear respiratory protection. Avoid dust formation. Avoid breathing vapors, mist or gas. Ensure adequate ventilation. Evacuate personnel to safe areas. Avoid breathing dust.

Environmental precautions:Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Methods and materials for containment and cleaning up: Pick up and arrange disposal without creating dust. Sweep up and shovel. Keep in suitable, closed containers for disposal.

Section 7: Handling and Storage

Precautions for safe handling;Avoid contact with skin and eyes. Avoid formation of dust and aerosols.

Provide appropriate exhaust ventilation at places where dust is formed. Normal measures for preventive fire protection.

Section 7: Handling and Storage (Continued)

Conditions for safe storage, including any incompatibilities: Store in cool place. Keep container tightly closed in a dry and well-ventilated place.

Section 8: Exposure Controls/Personal Protection

Control parameters

Components with workplace control parameters

Exposure controls

Appropriate engineering controls

Avoid contact with skin, eyes and clothing. Wash hands before breaks and immediately after handling the product.

Personal protective equipment

Eye/face protection

Face shield and safety glasses Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

Skin protection:Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices. Wash and dry hands.

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

Body Protection:

Complete suit protecting against chemicals, the type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace.

Respiratory protection:

Where risk assessment shows air-purifying respirators are appropriate use a full-face particle respirator type N99 (US) or type P2 (EN 143) respirator cartridges as a backup to engineeringcontrols. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Section 9: Physical and Chemical Properties

Appearance	: Form: Flakes

Odour		: no data available

Color		:Colorless

OdourThreshold :no data available
pH			: no data available
Melting point/freezing point:60 - 63 °C - lit.

Initial boiling point and boiling range:248 °C - lit.

Flash point 		: 88 °C - closed cup

Evaporation rate	:  no data available
Flammability (solid, gas): no data available

Upper/lower flammability or explosive limits: no data available
Vapour pressure	: no data available

Vapour density	: no data available

Relative density	:  no data available

Water solubility	: no data available
Partition coefficient: no data available

Auto ignition Temperature:  no data available

Decomposition temperature:  no data available
Viscosity		: no data available

Explosive properties: no data available.

Oxidizing properties: no data available
Other safety information: no data available

Section 10: Stability and Reactivity Data

Reactivity		: no data available.

Chemical stability:Stable under recommended storage conditions.

Possibility of hazardous reactions: no data available

Conditions to avoid: no data available
Incompatible materials: Strong oxidizing agents

Section 11: Toxicological Information

Information on toxicological effects

Acute toxicity: no data available

Skin corrosion/irritation: no data available

Serious eye damage/eye irritation: no data available

Respiratory or skin sensitization: no data available

Germ cell mutagenicity: no data available

Carcinogenicity

IARC: No components of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

Reproductive toxicity:no data available

Specific target organ toxicity - single exposure:no data available

Specific target organ toxicity - repeated exposure:May cause damage to organs through prolonged or repeated exposure.

Aspiration hazard:no data available

Aspiration hazard: no data available

Potential health effects

Inhalation 		Toxic if inhaled. Causes respiratory tract irritation.

Ingestion		Toxic if swallowed.

Skin			Toxic if absorbed through skin. Causes skin irritation.

Eyes			Causes serious eye irritation.

Section 12: Ecological Information

Toxicity:no data available

Persistence and degradability:no data available

Bio accumulative potential: no data available

Mobility in soil:no data available

Results of PBT and vPvB assessment: no data available

Section 13: Disposal Considerations

Waste treatment methods

Product:Offer surplus and non-recyclable solutions to a licensed disposal company. Contact a licensed professional waste disposal service to dispose of this material. Dissolve or mix the material with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and scrubber.

Contaminated packaging

Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name	: AMINOPYRIDINES (o-, m-, p-)

UN N°			: 2671

H.I. nr			: 60

ADR – Class		: 6.1

Sea transport (IMDG) [English only]

Proper shipping name	: AMINOPYRIDINES (o-, m-, p-)

UN N°			: 2671

IMO-IMDG - Class or division: 6.1: Toxic substances.

IMO-IMDG - Packing group: II

Air transport (ICAO-IATA) [English only]

Proper shipping name	:AMINOPYRIDINES (o-, m-, p-)

UN N°			:2671

IATA - Class or division:6.1: Toxic substances.

IATA - Packing group	 :II

Section 15: Other Regulatory Information

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

Safety, health and environmental regulations/legislation specific for the substance or mixture

no data available

Chemical Safety Assessment

For this product a chemical safety assessment was not carried out

Section 16 - Additional Information

Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.