MATERIAL SAFETY DATA SHEET

AMMONIUM IODATE

MSDS CAS: 13446-09-8

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:Ammonium Iodate

CAS#: 13446-09-8

C.I. No.:Not applicable.

Synonym:

Chemical Name: Not available.

Chemical Formula: H4INO3

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Ammonium Iodate | 13446-09-8 | 100 |

Section 3: Hazards Identification

Classification of the substance or mixture

Classification according to EU Directives 67/548/EEC or 1999/45/EC

Contact with combustible material may cause fire. Irritating to eyes and skin.

Label elements

Labelling according Regulation (EC) No 1272/2008 [CLP]

Signal word			Danger

Hazard statement(s)

H272					May intensify fire; oxidiser.

H315 					Causes skin irritation.

H319 					Causes serious eye irritation.

Precautionary statement(s)

P220 					Keep/Store away from clothing/ combustible materials.

P305 + P351 + P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

Supplemental Hazard

Statements 				none

According to European Directive 67/548/EEC as amended.

R-phrase(s)

R 8		Contact with combustible material may cause fire.

R36/38 	Irritating to eyes and skin.

S-phrase(s)

S17		 Keep away from combustible material.

S26		In case of contact with eyes, rinse immediately with plenty of water and seek medical advice.

S36 		Wear suitable protective clothing.

Other hazards – none

Section 4: First Aid Measures

FIRST AID MEASURES

Description of first aid measures

General advice

Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

In case of skin contact

Wash off with soap and plenty of water. Consult a physician.

In case of eye contact

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed

Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Most important symptoms and effects, both acute and delayed

Nausea, Vomiting, Diarrhoea, Rash

Indication of any immediate medical attention and special treatment needed

no data available.

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special hazards arising from the substance or mixture

nitrogen oxides (NOx), Hydrogen iodide

Advice for firefighters

Wear self-contained breathing apparatus for firefighting if necessary.

Further information

Use water spray to cool unopened containers.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures

Use personal protective equipment. Avoid dust formation. Avoid breathing vapors, mist or gas. Ensure adequate ventilation. Evacuate personnel to safe areas. Avoid breathing dust.

Environmental precautions

Do not let product enter drains.

Methods and materials for containment and cleaning up

Sweep up and shovel. Contain spillage, and then collect with an electrically protected vacuum cleaner or by wet-brushing and place in container for disposal according to local regulations (see section 13). Keep in suitable, closed containers for disposal.

Reference to other sections

For disposal see section 13.

Section 7: Handling and Storage

Precautions for safe handling

Avoid contact with skin and eyes. Avoid formation of dust and aerosols.

Provide appropriate exhaust ventilation at places where dust is formed. Keep away from sources of ignition

- No smoking. Keep away from heat and sources of ignition. Normal measures for preventive fire protection.

Conditions for safe storage, including any incompatibilities

Store in cool place. Keep container tightly closed in a dry and well-ventilated place.

Air, light, and moisture sensitive.

Specific end uses

no data available

Section 8: Exposure Controls/Personal Protection

Control parameters

Components with workplace control parameters

Exposure controls

Appropriate engineering controls

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday.

Personal protective equipment

Eye/face protection: Safety glasses with side-shields conforming to EN166 Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

Skin protection: Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices.

Wash and dry hands.

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

Body Protection:impervious clothing, the type of protective equipment must be selected according to theconcentration and amount of the dangerous substance at the specific workplace.

Respiratory protection: Where risk assessment shows air-purifying respirators are appropriate use a full-face particle respirator type N100 (US) or type P3 (EN 143) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Section 9: Physical and Chemical Properties

Physical state and appearance: Powder with lumps

Odor					: Not available.

Taste					: Not available.

Molecular Weight		: 192, 94 g/mole

Section 9: Physical and Chemical Properties (Continued)

Color					:Not available.

pH (1% soln/water)		: Not available.

Boiling Point			: Not available.

Melting Point			: 150°C

Flash Point				:Not available.

Critical Temperature		: Not available.

Specific Gravity			: 1.28 (Water = 1)

Vapor Pressure			: Not applicable

Vapor Density			: Not available.

Relative density			: 3,309 g/mL at 25 °C

Volatility				: Not available.

Odor Threshold			: Not available.

Water/Oil Dist. Coeff.		: Not available

Ionicity (in Water)		: Not available.

Dispersion Properties		: Not available

Solubility				:Not available

Section 10: Stability and Reactivity Data

Reactivity:no data available

Chemical stability:no data available

Possibility of hazardous reactions: no data available

Conditions to avoid: no data available

Incompatible materials: Strong reducing agents, Powdered metals, Incompatibility: mixtures of iodates with finely divided aluminum, arsenic, copper, carbon, phosphorous (red or white) sulfur; hydrides of alkali and alkaline earth metals; sulfides of antimony, arsenic, copper or tin, metal cyanides, thiocyanates or impure manganese dioxide may react violently or explosively, either spontaneously (especially in the presence of moisture) or on initiation by heat, friction impact, sparks, or addition of sulfuric acid.

Hazardous decomposition products: Other decomposition products - no data available

Section 11: Toxicological Information

Information on toxicological effects

Acute toxicity:no data available

Skin corrosion/irritation: no data available

Serious eye damage/eye irritation: no data available

Respiratory or skin sensitization: no data available

Germ cell mutagenicity: no data available

Carcinogenicity

IARC: No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

Reproductive toxicity: no data available

Specific target organ toxicity - single exposure: no data available

Specific target organ toxicity - repeated exposure: no data available

Aspiration hazard: no data available

Potential health effects

InhalationMay be harmful if inhaled. Causes respiratory tract irritation.

IngestionMay be harmful if swallowed.

SkinMay be harmful if absorbed through skin. Causes skin irritation.

Eyes Causes serious eye irritation.

Signs and Symptoms of Exposure

Nausea, Vomiting, Diarrhoea, Rash

Section 12: Ecological Information

Toxicity: no data available

Persistence and degradability: no data available

Bioaccumulative potential: no data available

Mobility in soil: no data available

Results of PBT and vPvB assessment: no data available

Other adverse effects: no data available

Section 13: Disposal Considerations

Waste treatment methods

Product

Burn in a chemical incinerator equipped with an afterburner and scrubber but exert extra care in igniting as this material is highly flammable. Offer surplus and non-recyclable solutions to a licensed disposal company. Contact a licensed professional waste disposal service to dispose of this material.

Contaminated packaging

Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name	:OXIDIZING SOLID, N.O.S. (Ammonium iodate)

UN N°			: 1479

ADR – Class		: 5.1

Sea transport (IMDG) [English only]

Proper shipping name	:OXIDIZING SOLID, N.O.S. (Ammonium iodate)

UN N°			: 1479

IMO-IMDG - Class or division: 5.1

IMO-IMDG - Packing group: II

Air transport (ICAO-IATA) [English only]

Proper shipping name	:Oxidizing solid, n.o.s. (Ammonium iodate)

UN N°			: 1479

IATA - Class or division: 5.1

IATA - Packing group	: II

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture

no data available

Chemical Safety Assessment

no data available

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.