MATERIAL SAFETY DATA SHEET

AMMONIUM PERSULPHATE AR

(Ammonium PeroxyDisulphate)

MSDS CAS: 7727-54-0

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:Ammonium Persulphate AR

CAS#: 7727-54-0

C.I. No.:Not applicable.

Synonym:Ammonium PeroxyDisulphate

Chemical Name: Ammonium Persulphate

Chemical Formula: (NH4)2S2O8

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Ammonium Persulphate AR | 7727-54-0 | 100 |

Toxicological Data on Ingredients: Ammonium persulfate: ORAL (LD50): Acute: 689 mg/kg [Rat].

Section 3: Hazards Identification

Potential Acute Health Effects: Hazardous in case of skin contact (irritant, sensitizer), of eye contact (irritant), of ingestion, of inhalation (lung irritant, lung sensitizer). Prolonged exposure may result in skin burns and ulcerations. Over-exposure by inhalation may cause respiratory irritation.

Potential Chronic Health Effects: CARCINOGENIC EFFECTS: Not available. MUTAGENIC EFFECTS: Not available. TERATOGENIC EFFECTS: Not available. DEVELOPMENTAL TOXICITY: Not available. The substance may be toxic to upper respiratory tract. Repeated or prolonged exposure to the substance can produce target organs damage.

Section 4: First Aid Measures

Eye Contact:Check for and remove any contact lenses. In case of contact, immediately flush eyes with plenty of water for at least 15 minutes. Cold water may be used. Get medical attention.

Skin Contact:In case of contact, immediately flush skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Cover the irritated skin with an emollient. Cold water may be used. Wash clothing before reuse. Thoroughly clean shoes before reuse. Get medical attention.

Serious Skin Contact:Wash with a disinfectant soap and cover the contaminated skin with an anti-bacterial cream. Seek immediate medical attention.

Inhalation:If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical attention.

Serious Inhalation:Evacuate the victim to a safe area as soon as possible. Loosen tight clothing such as a collar, tie, belt or waistband. If breathing is difficult, administer oxygen. If the victim is not breathing, perform mouth-to-mouth resuscitation. WARNING: It may be hazardous to the person providing aid to give mouth-to-mouth resuscitation when the inhaled material is toxic, infectious or corrosive. Seek medical attention.

Ingestion:Do NOT induce vomiting unless directed to do so by medical personnel. Never give anything by mouth to an unconscious person. If large quantities of this material are swallowed, call a physician immediately. Loosen tight clothing such as a collar, tie, belt or waistband.

Serious Ingestion:Not available.

Section 5: Fire and Explosion Data

Flammability of the Product:May be combustible at high temperature.

Auto-Ignition Temperature: Not available.

Flash Points:CLOSED CUP: Higher than 93.3°C (200°F).

Flammable Limits:Not applicable.

Products of Combustion:Not available.

Fire Hazards in Presence of Various Substances:Not applicable.

Explosion Hazards in Presence of Various Substances:Risks of explosion of the product in presence of mechanical impact: Not available. Risks of explosion of the product in presence of static discharge: Not available. Slightly explosive in presence of reducing materials, of combustible materials, of organic materials, of metals.

Fire Fighting Media and Instructions: Oxidizing material. Do not use water jet. Use flooding quantities of water. Avoid contact with organic materials.

Special Remarks on Fire Hazards:Not available.

Special Remarks on Explosion Hazards:Not available.

Section 6: Accidental Release Measures

Small Spill:Use appropriate tools to put the spilled solid in a convenient waste disposal container.

Large Spill:Oxidizing material. Stop leak if without risk. Avoid contact with a combustible material (wood, paper, oil, clothing...). Keep substance damp using water spray. Do not touch spilled material. Prevent entry into sewers, basements or confined areas; dike if needed. Eliminate all ignition sources. Call for assistance on disposal.

Section 7: Handling and Storage

Precautions:Keep away from heat. Keep away from sources of ignition. Keep away from combustible material.. Empty containers pose a fire risk, evaporate the residue under a fume hood. Ground all equipment containing material. Do not ingest. Do not breathe dust. Wear suitable protective clothing. In case of insufficient ventilation, wear suitable respiratory equipment. If ingested, seek medical advice immediately and show the container or the label. Avoid contact with skin and eyes. Keep away from incompatibles such as reducing agents, combustible materials, organic materials, metals.

Storage:Strong Oxidizer. Moisture Sensitive. Keep container tightly closed. Keep container in a cool, well-ventilated area. Separate from acids, alkalies, reducing agents and combustibles. See NFPA 43A, Code for the Storage of Liquid and Solid Oxidizers. Do not store above 23°C (73.4°F).

Section 8: Exposure Controls/Personal Protection

Engineering Controls:Use process enclosures, local exhaust ventilation, or other engineering controls to keep airborne levels below recommended exposure limits. If user operations generate dust, fume or mist, use ventilation to keep exposure to airborne contaminants below the exposure limit.

Personal Protection:Splash goggles. Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Gloves.

Personal Protection in Case of a Large Spill:Splash goggles. Full suit. Dust respirator. Boots. Gloves. A self-contained breathing apparatus should be used to avoid inhalation of the product. Suggested protective clothing might not be sufficient; consult a specialist BEFORE handling this product.

Exposure Limits:Not available.

Section 9: Physical and Chemical Properties

Physical state and appearance: Solid.

Odor					: Odorless

Taste					: Not available.

Molecular Weight		: 228.2 g/mole

Color					:White to yellowish. (Light.)

pH (1% soln/water)		: 1-2.

Boiling Point			: Not available.

Melting Point			: 120°C (248°F)

Flash Point				:Not available.

Critical Temperature		: Not available.

Specific Gravity			: 1.98 (Water = 1)

Vapor Pressure			: Not applicable

Vapor Density			: Not available.

Volatility				: Not available.

Odor Threshold			: Not available.

Water/Oil Dist. Coeff.		: Not available

Ionicity (in Water)		: Not available.

Dispersion Properties		: See solubility in water.

Solubility				: Soluble in cold water, hot water.

Section 10: Stability and Reactivity Data

Stability:The product is stable.

Instability Temperature:Not available.

Conditions of Instability:Excess heat, ignition sources, incompatible materials.

Incompatibility with various substances:Reactive with reducing agents, combustible materials, organic materials, metals.

Corrosivity:Non-corrosive in presence of glass.

Special Remarks on Reactivity:Strong oxider. Moisture sensitive. May decompose on exposure to moist air or water. May be shock sensitive and thermally unstable. Decomposes when heated. Incompatible with powdered aluminum, iron, sodium peroxide.

Special Remarks on Corrosivity:Not available.

Polymerization:Will not occur.

Section 11: Toxicological Information

Routes of Entry: Inhalation. Ingestion.

Toxicity to Animals:Acute oral toxicity (LD50): 689 mg/kg [Rat].

Chronic Effects on Humans:May cause damage to the following organs: upper respiratory tract.

Other Toxic Effects on Humans:Hazardous in case of skin contact (irritant, sensitizer), of ingestion, of inhalation (lung irritant, lung sensitizer).

Special Remarks on Toxicity to Animals:Not available

Special Remarks on Chronic Effects on Humans:Not available

Special Remarks on other Toxic Effects on Humans:Acute Potential Health Effects: Skin: Causes skin irritation. May cause skin sensitization, an allergic reation, which becomes evident upon re-exposure to this material. Eyes: Causes eye irritation. Ingestion: Causes gastrointestinal (digestive) tract irritation with nausea, vomiting, and diarrhea. May be harmful if swallowed. Inhalation: Causes respiratory tract irritation. May cause chemical pneumonitis and pulmonary edema, inflammation, edema of bronchi and larynx. Chronic Potential Health Effects: Repeated or prolonged skin exposure may cause allergic reactions in sensitive individuals. Repeated or prolonged exposure by inhalation may affect respiration and metabolism.

Section 12: Ecological Information

Ecotoxicity:Not available.

BOD5 and COD:Not available.

Products of Biodegradation: Possibly hazardous short term degradation products are not likely. However, long term degradation products may arise.

Section 12: Ecological Information (Continued)

Toxicity of the Products of Biodegradation:The products of degradation are less toxic than the product itself.

Special Remarks on the Products of Biodegradation:Not available.

Section 13: Disposal Considerations

Waste Disposal:Waste must be disposed of in accordance with federal, state and local environmental control regulations.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name	: AMMONIUM PERSULPHATE

UN N°			: 1444

H.I. nr			: 50

ADR – Class		: 5.1

Sea transport (IMDG) [English only]

Proper shipping name	: AMMONIUM PERSULPHATE

UN N°			: 1444

IMO-IMDG - Class or division: 5.1: Oxidizing substances.

IMO-IMDG - Packing group: III

Air transport (ICAO-IATA) [English only]

Proper shipping name	: AMMONIUM PERSULPHATE

UN N°			: 1444

IATA - Class or division: 5.1: Oxidizing substances.

IATA - Packing group	: III

Section 15: Other Regulatory Information

Federal and State Regulations:Connecticut hazardous material survey.: Ammonium persulfate New Jersey: Ammonium persulfate TSCA 8(b) inventory: Ammonium persulfate

Other Regulations:OSHA: Hazardous by definition of Hazard Communication Standard (29 CFR 1910.1200). EINECS: This product is on the European Inventory of Existing Commercial Chemical Substances.

Other Classifications:

WHMIS (Canada):CLASS C: Oxidizing material.

DSCL (EEC):R8- Contact with combustible material may cause fire. R20/22- Harmful by inhalation and if swallowed. R36/37/38- Irritating to eyes, respiratory system and skin. R42/43- May cause sensitization by inhalation and skin contact. S22- Do not breathe dust. S24- Avoid contact with skin. S26- In case of contact with eyes, rinse immediately with plenty of water and seek medical advice. S37- Wear suitable gloves.

HMIS (U.S.A.):

Health Hazard:2

Fire Hazard:1

Reactivity:3

Personal Protection:E

National Fire Protection Association (U.S.A.):

Health:2

Flammability:1

Reactivity:3

Specific hazard:

Protective Equipment: Gloves. Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Wear appropriate respirator when ventilation is inadequate. Splash goggles.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.