MATERIAL SAFETY DATA SHEET

AMMONIUM PHOSPHATE DIBASIC AR

(Anhydrous)

MSDS CAS: 7783-28-0

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:AMMONIUM PHOSPHATE DIBASIC AR Anhydrous

CAS#: 7783-28-0

C.I. No.:Not applicable.

Synonym:

Chemical Name: Ammonium Persulphate

Chemical Formula: (NH4)2HPO4

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| AMMONIUM PHOSPHATE DIBASIC AR | 7783-28-0 | 98-102 |

Toxicological Data on Ingredients: Ammonium phosphate dibasic: ORAL (LD50): Acute: 3000 mg/kg [Rat].

Section 3: Hazards Identification

Potential Acute Health Effects: Very hazardous in case of eye contact (irritant). Hazardous in case of skin contact (irritant), of ingestion, of inhalation. Slightly hazardous in case of skin contact (permeator). Inflammation of the eye is characterized by redness, watering, and itching.

Potential Chronic Health Effects: CARCINOGENIC EFFECTS: Not available. MUTAGENIC EFFECTS: Not available. TERATOGENIC EFFECTS: Not available. DEVELOPMENTAL TOXICITY: Not available. The substance is toxic to lungs, mucous membranes. Repeated or prolonged exposure to the substance can produce target organs damage.

Section 4: First Aid Measures

Eye Contact:Check for and remove any contact lenses. In case of contact, immediately flush eyes with plenty of water for at least 15 minutes. Cold water may be used. Get medical attention immediately.

Skin Contact:In case of contact, immediately flush skin with plenty of water. Cover the irritated skin with an emollient. Remove contaminated clothing and shoes. Cold water may be used.Wash clothing before reuse. Thoroughly clean shoes before reuse. Get medical attention.

Serious Skin Contact:Wash with a disinfectant soap and cover the contaminated skin with an anti-bacterial cream. Seek medical attention.

Inhalation:If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical attention.

Serious Inhalation:Not available.

Ingestion:Do NOT induce vomiting unless directed to do so by medical personnel. Never give anything by mouth to an unconscious person. Loosen tight clothing such as a collar, tie, belt or waistband. Get medical attention if symptoms appear.

Serious Ingestion:Not available.

Section 5: Fire and Explosion Data

Flammability of the Product:Non-flammable.

Auto-Ignition Temperature: Not applicable.

Flash Points:Not applicable.

Section 5: Fire and Explosion Data (Continued)

Flammable Limits:Not applicable.

Products of Combustion:Not applicable.

Fire Hazards in Presence of Various Substances:Not applicable.

Explosion Hazards in Presence of Various Substances:Risks of explosion of the product in presence of mechanical impact: Not available. Risks of explosion of the product in presence of static discharge: Not available.

Fire Fighting Media and Instructions: Not applicable.

Special Remarks on Fire Hazards:Not available.

Special Remarks on Explosion Hazards:Not available.

Section 6: Accidental Release Measures

Small Spill:Use appropriate tools to put the spilled solid in a convenient waste disposal container. Finish cleaning by spreading water on the contaminated surface and dispose of according to local and regional authority requirements.

Large Spill:Use a shovel to put the material into a convenient waste disposal container. Finish cleaning by spreading water on the contaminated surface and allow to evacuate through the sanitary system.

Section 7: Handling and Storage

Precautions:Do not ingest. Do not breathe dust. In case of insufficient ventilation, wear suitable respiratory equipment. If ingested, seek medical advice immediately and show the container or the label. Avoid contact with skin and eyes.

Storage:Keep container tightly closed. Keep container in a cool, well-ventilated area.

Section 8: Exposure Controls/Personal Protection

Engineering Controls:Use process enclosures, local exhaust ventilation, or other engineering controls to keep airborne levels below recommended exposure limits. If user operations generate dust, fume or mist, use ventilation to keep exposure to airborne contaminants below the exposure limit.

Section 8: Exposure Controls/Personal Protection (Continued)

Personal Protection:Splash goggles. Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Gloves.

Personal Protection in Case of a Large Spill:Splash goggles. Full suit. Dust respirator. Boots. Gloves. A self-contained breathing apparatus should be used to avoid inhalation of the product. Suggested protective clothing might not be sufficient; consult a specialist BEFORE handling this product.

Exposure Limits:Not available.

Section 9: Physical and Chemical Properties

Physical state and appearance: Solid.

Odor					: Odorless.

Taste					: Not available.

Molecular Weight		: 132.06 g/mole

Color					:Colorless crystlas.

pH (1% soln/water)		: Not available.

Boiling Point			: Not available.

Melting Point			: Decomposes.

Flash Point				:Not available.

Critical Temperature		: Not available.

Specific Gravity			: 1.619 (Water = 1)

Vapor Pressure			: Not applicable

Vapor Density			: Not available.

Volatility				: Not available.

Odor Threshold			: Not available.

Water/Oil Dist. Coeff.		: Not available

Ionicity (in Water)		: Not available.

Dispersion Properties		: See solubility in water.

Solubility				: Easily soluble in cold water.

Section 10: Stability and Reactivity Data

Stability:The product is stable.

Instability Temperature:Not available.

Section 10: Stability and Reactivity Data (Continued)

Conditions of Instability:Not available.

Incompatibility with various substances:Not available.

Corrosivity:Non-corrosive in presence of glass.

Special Remarks on Reactivity:Not available.

Special Remarks on Corrosivity:Not available.

Polymerization:Will not occur.

Section 11: Toxicological Information

Routes of Entry: Eye contact. Inhalation. Ingestion.

Toxicity to Animals:Acute oral toxicity (LD50): 3000 mg/kg [Rat].

Chronic Effects on Humans:Causes damage to the following organs: lungs, mucous membranes.

Other Toxic Effects on Humans:Hazardous in case of skin contact (irritant), of ingestion, of inhalation. Slightly hazardous in case of skin contact (permeator).

Special Remarks on Toxicity to Animals:Not available

Special Remarks on Chronic Effects on Humans:Not available

Special Remarks on other Toxic Effects on Humans:Not available

Section 12: Ecological Information

Ecotoxicity:Not available.

BOD5 and COD:Not available.

Products of Biodegradation: Possibly hazardous short term degradation products are not likely. However, long term degradation products may arise.

Toxicity of the Products of Biodegradation:The products of degradation are more toxic.

Special Remarks on the Products of Biodegradation:Not available.

Section 13: Disposal Considerations

Waste Disposal:

Section 14: Transport Information

Land transport (ADR-RID)

General Information: Not Regulated

Sea transport (IMDG) [English only]

General Information: Not Regulated

Air transport (ICAO-IATA) [English only]

General Information: Not Regulated

Section 15: Other Regulatory Information

Federal and State Regulations:Pennsylvania RTK: Ammonium phosphate dibasic Massachusetts RTK: Ammonium phosphate dibasic TSCA 8(b) inventory: Ammonium phosphate dibasic

Other Regulations:OSHA: Hazardous by definition of Hazard Communication Standard (29 CFR 1910.1200).

Other Classifications:

WHMIS (Canada):CLASS D-2A: Material causing other toxic effects (VERY TOXIC).

DSCL (EEC):R38- Irritating to skin. R41- Risk of serious damage to eyes.

HMIS (U.S.A.):

Health Hazard:2

Fire Hazard:0

Reactivity:0

Personal Protection:E

National Fire Protection Association (U.S.A.):

Health:2

Flammability:0

Reactivity:0

Specific hazard:

Protective Equipment: Gloves. Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Wear appropriate respirator when ventilation is inadequate. Splash goggles.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.