MATERIAL SAFETY DATA SHEET

ANILINE SULPHATE

(Extra Pure)

MSDS CAS: 542-16-5

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:Aniline Sulphate

CAS#: 542-16-5

C.I. No.:Not available.

Synonym:Not available.

Chemical Name:Not available.

Chemical Formula:Not available.

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Aniline Sulphate | 542-16-5 | 100 |

Section 3: Hazards Identification

Classification of the substance or mixture

Classification EC 67/548 or EC 1999/45

Classification :T; R23 Xi; R41 R43 N; R50.

Hazard Class and Category Code(s), Regulation (EC) No 1272/2008 (CLP)

Health hazards : Specific Target Organ Toxicity - Single exposure - Category 2 - Warning (CLP : STOT SE 2) H371 Specific Target Organ Toxicity - Repeated exposure - Category 1 - Danger (CLP : STOT RE 1) H372 Acute toxicity, dermal - Category 3 - Danger (CLP : Acute Tox. 3) H311 Acute toxicity, Inhalation - Category 3 - Danger (CLP : Acute Tox. 3) H331 Serious eye damage - Category 1 - Danger (CLP : Eye Dam. 1) H318 Skin sensitisation - Category 1 - Warning (CLP : Skin Sens. 1) H317 Acute toxicity, Oral - Category 3 - Danger (CLP : Acute Tox. 3) H301Environmental hazards : Hazardous to the aquatic environment - Acute hazard - Category 1 - Warning (CLP : Aquatic Acute 1) H400

Other hazards

Other hazards :The substance does not fulfil the criteria to be identified as PBT substance or vPvB

substance according to Annex XIII of Regulation REACH.

Section 4: First Aid Measures

Description of first aid measures

Inhalation :Immediately call a POISON CENTER or doctor. Specific treatment (see on this label). Remove to fresh air and keep at rest in a position comfortable for breathing.

Skin contact :If skin irritation or rash occurs : Get medical advice. Specific treatment (see on this label). Wash contaminated clothing before reuse. Immediately call a POISON CENTER or doctor. Remove/Take off immediately all contaminated clothing. Wash with plenty of soap and water.

Eye contact :Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Immediately call a POISON CENTER or doctor.

Ingestion :Specific treatment (see on this label). Immediately call a POISON CENTER or doctor. Do NOT induce vomiting. Rinse mouth.

Most important symptoms and effects, both acute and delayed

Section 4: First Aid Measures (Continued)

Symptoms relating to use :Repeated exposure to this material can result in absorption through skin causing significant health hazard. Causes damage to organs through prolonged or repeated exposure. Causes serious eye damage. Danger of serious damage to health by prolonged exposure through inhalation. May cause an allergic skin reaction. May cause damage to organs. Swallowing a small quantity of this material will result in serious health hazard.

Indication of any immediate medical attention and special treatment needed

General information :Never give anything by mouth to an unconscious person. If you feel unwell, seek

medical advice (show the label where possible).

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media :Foam. Dry powder. Carbon dioxide. Water spray. Sand.

Unsuitable extinguishing media :Do not use a heavy water stream.

Surrounding fires :Use water spray or fog for cooling exposed containers.

Special hazards arising from the substance or mixture

Hazardous combustion products : Under fire conditions, hazardous fumes will be present.

Advice for fire-fighters

Protection against fire :Do not enter fire area without proper protective equipment, including respiratory

protection.

Special procedures :Exercise caution when fighting any chemical fire. Avoid (reject) fire-fighting water to

enter environment.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures

For emergency responders :Equip cleanup crew with proper protection. Ventilate area,Avoid breathing dust, fume, gas, mist, vapours, spray.

For non-emergency personnel :Evacuate unnecessary personnel.

Section 6: Accidental Release Measures(Continued)

Environmental precautions

Environmental precautions :Prevent entry to sewers and public waters. Notify authorities if product enters sewers or public waters.Avoid release to the environment.

Methods and material for containment and cleaning up

Clean up methods :On land, sweep or shovel into suitable containers. Store away from other materials.

Reference to other sections

See section 8. Exposure controls/personal protection

Section 7: Handling and Storage

Precautions for safe handling

Personal protection :Do not breathe dust, fume, gas, mist, vapours, spray. Wash thoroughly after handling. Do not eat, drink or smoke when using this product.

Handling :Wash thoroughly after handling. Avoid breathing dust, fume, gas, mist, vapours, spray. Do not breathe dust, fume, gas, mist, vapours, spray. Do not eat, drink or smoke when using this product. Wash contaminated clothing before reuse. Contaminated work clothing should not be allowed out of the workplace. Use only outdoors or in a well-ventilated area. Do not eat, drink or smoke when using this product. Wash thoroughly after handling.

Technical protective measures :Provide good ventilation in process area to prevent formation of vapour.

Conditions for safe storage, including any incompatibilities

Storage :Keep only in the original container in a cool, well ventilated place. Keep container tightly closed.

Storage - away from :Strong bases. Strong acids. Sources of ignition. Direct sunlight.

Specific end use(s)

Specific end use(s) :None.

Section 8: Exposure Controls/Personal Protection

Exposure controls

Personal protection :Avoid all unnecessary exposure.

• Respiratory protection :In case of insufficient ventilation, wear suitable respiratory equipment.Where exposure through inhalation may occur from use, respiratory protectionequipment is recommended.

• Hand protection :Wear protective gloves.

• Eye protection :Chemical goggles or safety glasses.

• Others :When using, do not eat, drink or smoke.

Control parameters

Occupational Exposure Limits :No data available.

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Physical state at 20 °C			: Solid.

Colour					: white powder

Odour					: Odorless

Odour threshold				: No data available.

pH value					: Not applicable.

Melting point [°C]			: No data available.

Decomposition point [°C]		: N/A

Critical temperature [°C] 		: N/A

Auto-ignition temperature [°C]	: N/A

Flammability (solid, gas)		: N/A

Flash point [°C]				: N/A

Boiling point [°C]				: N/A

Initial boiling point [°C]		: N/A

Final boiling point [°C]			: N/A

Section 9: Physical and Chemical Properties(Continued)

Evaporation rate				: N/A

Vapour pressure [20°C]			: N/A

Vapour pressure mm/Hg		: N/A

Vapour density				: N/A

Density [g/cm3]				: N/A

Relative density, gas (air=1)		: N/A

Relative density, liquid (water=1)	: N/A

Solubility in water [% weight]	: Soluble in water

Solubility in water			: N/A

Log Pow octanol / water at 20°C	: No data available.

Solubility					: N/A

Viscosity at 40°C [mm2/s]		: N/A

Other information

Explosive properties			: N/A

Explosion limits - upper [%]		: N/A

Explosion limits - lower [%]		: N/A

Oxidising properties			: No data available.

Section 10: Stability and Reactivity Data

Reactivity

Reactivity :Not established.

Chemical stability

Chemical stability :Stable under recommended storage conditions.

Possibility of hazardous reactions

Hazardous reactions :Not established.

Conditions to avoid

Section 10: Stability and Reactivity Data(Continued)

Conditions to avoid :Direct sunlight. Extremely high or low temperatures.

Incompatible materials

Materials to avoid :Strong acids. Strong bases.

Hazardous decomposition products

Hazardous decomposition products :Fumes. Carbon monoxide. Carbon dioxide.

Section 11: Toxicological Information

Information on toxicological effects

Acute toxicity

• Inhalation			: Toxic if inhaled.

• Dermal			: Toxic in contact with skin.

• Ingestion			: Toxic if swallowed.

Corrosion			: Causes serious eye damage.

Irritation			: Based on available data, the classification criteria are not met.

Sensitization		: May cause an allergic skin reaction.

Mutagenicity		: Based on available data, the classification criteria are not met.

Carcinogenicity		: Based on available data, the classification criteria are not met.

Toxic for reproduction	: Based on available data, the classification criteria are not met.

STOT-single exposure 	: Causes damage to organ.

STOT-repeated exposure :Causes damage to organs through prolonged or repeated exposure.

Aspiration hazard 	: Based on available data, the classification criteria are not met.

Section 12: Ecological Information

Toxicity

Toxicity information :Very toxic to aquatic life.

Section 12: Ecological Information(Continued)

Persistence - degradability

Persistence - degradability : Biodegradable.

Bioaccumulative potential

Bioaccumulative potential : Not established.

Mobility in soil

Mobility in soil : Not established.

Results of PBT and vPvB assessment

Results of PBT and vPvBassessment :The substance does not fulfil the criteria to be identified as PBT substance or vPvBsubstance according to Annex XIII of Regulation REACH.

Other adverse effects

Environmental precautions :Avoid release to the environment.

Section 13: Disposal Considerations

Waste treatment methods

General :Avoid release to the environment. Dispose in a safe manner in accordance with local/national regulations. Dispose of this material and its container to hazardous or special waste collection point, in accordance with local, regional, national and/or international regulation.

Special precautions :Hazardous waste due to toxicity.

Section 14: Transport Information

Land transport (ADR-RID)

UN N°					: 2811

H.I. nr					: 60

Section 14: Transport Information(Continued)

ADR – Class				: 6.1

Labelling – Transport	: 6.1 : Toxic substances. 9 : Miscellaneous dangerous substances and articles.

ADR – Group				: III

Sea transport (IMDG) [English only]

UN N°					: 2811

IMO-IMDG - Class or division	: 6.1 : Toxic substances. ( 9 : Miscellaneous dangerous substances and articles. )

IMO-IMDG - Packing group		: III

Air transport (ICAO-IATA) [English only]

UN N°					: 2811

IATA - Class or division	: 6.1 : Toxic substances. ( 9 : Miscellaneous dangerous substances and articles. )

IATA - Packing group 			: III

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture

Safety, health and environmental :Ensure all national/local regulations are observed.

regulations/legislation specific for the substance or mixture

REACH Restrictions - Annex XVII :The components of this product are not subject to restrictions.

REACH Authorisation - Annex XIV :The components of this product are not subject to authorization.

Chemical Safety Assessment

Chemical Safety Assessment :It has not been carried out.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.