MATERIAL SAFETY DATA SHEET

ANTHRACENE 96%

Extra Pure

MSDS CAS: 120-12-7

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:ANTHRACENE

CAS#: 120-12-7

C.I. No.:Not available.

Synonym: Not available.

Chemical Name:ANTHRACENE

Chemical Formula:C14-H10

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| ANTHRACENE | 120-12-7 | 100 |

Toxicological Data on Ingredients: Anthracene: ORAL (LD50): Acute: &gt;5000 mg/kg [Mouse]. DERMAL (LD50): Acute: & gt;1000 mg/kg [Rat]. DUST (LC50): Acute: &gt;1300 mg/m 4 hours [Rat].

Section 3: Hazards Identification

Potential Acute Health Effects:Slightly hazardous in case of skin contact (irritant), of eye contact (irritant), of ingestion, of inhalation.

Potential Chronic Health Effects: Slightly hazardous in case of skin contact (sensitizer). CARCINOGENIC EFFECTS: Not available. MUTAGENIC EFFECTS:Mutagenic for mammalian somatic cells. Mutagenic for bacteria and/or yeast. TERATOGENIC EFFECTS: Not available.DEVELOPMENTAL TOXICITY: Not available.

Section 4: First Aid Measures

Eye Contact: Check for and remove any contact lenses. In case of contact, immediately flush eyes with plenty of water for at least 15minutes. Get medical attention if irritation occurs.

Skin Contact: In case of contact, immediately flush skin with plenty of water. Cover the irritated skin with an emollient. Remove contaminatedclothing and shoes. Wash clothing before reuse. Thoroughly clean shoes before reuse. Get medical attention.

Serious Skin Contact: Not available.

Inhalation: If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medicalattention.

Serious Inhalation: Not available.

Ingestion: Do NOT induce vomiting unless directed to do so by medical personnel. Never give anything by mouth to an unconsciousperson. Loosen tight clothing such as a collar, tie, belt or waistband. Get medical attention if symptoms appear.

Serious Ingestion: Not available.

Section 5: Fire and Explosion Data

Flammability of the Product: May be combustible at high temperature.

Auto-Ignition Temperature: 650°C (1202°F)

Flash Points: CLOSED CUP: 185°C (365°F).

Flammable Limits: Not available.

Products of Combustion: These products are carbon oxides (CO, CO2).

Fire Hazards in Presence of Various Substances: Slightly flammable to flammable in presence of open flames and sparks, of heat. Non-flammable in presence of shocks.

Explosion Hazards in Presence of Various Substances:

Risks of explosion of the product in presence of mechanical impact: Not available. Risks of explosion of the product in

Section 5: Fire and Explosion Data(Continued)

presence of static discharge: Not available.

Fire Fighting Media and Instructions:

SMALL FIRE: Use DRY chemical powder.

LARGE FIRE: Use water spray, fog or foam. Do not use water jet.

Special Remarks on Fire Hazards: Not available.

Special Remarks on Explosion Hazards: Not available.

Section 6: Accidental Release Measures

Small Spill:Use appropriate tools to put the spilled solid in a convenient waste disposal container. Finish cleaning by spreading water onthe contaminated surface and dispose of according to local and regional authority requirements.

Large Spill: Use a shovel to put the material into a convenient waste disposal container. Finish cleaning by spreading water on thecontaminated surface and allow to evacuate through the sanitary system.

Section 7: Handling and Storage

Precautions: Keep locked up.. Keep away from heat. Keep away from sources of ignition. Empty containers pose a fire risk, evaporatethe residue under a fume hood. Ground all equipment containing material. Do not ingest. Do not breathe dust. Wear suitableprotective clothing. If ingested, seek medical advice immediately and show the container or the label. Keep away fromincompatibles such as oxidizing agents.

Storage: Keep container tightly closed. Keep container in a cool, well-ventilated area. Do not store above 25°C (77°F).

Section 8: Exposure Controls/Personal Protection

Engineering Controls: Use process enclosures, local exhaust ventilation, or other engineering controls to keep airborne levels below recommendedexposure limits. If user operations generate dust, fume or mist, use ventilation to keep exposure to airborne contaminantsbelow the exposure limit.

Personal Protection: Safety glasses. Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Gloves.

Section 8: Exposure Controls/Personal Protection (continued)

Personal Protection in Case of a Large Spill:Splash goggles. Full suit. Dust respirator. Boots. Gloves. A self contained breathing apparatus should be used to avoidinhalation of the product. Suggested protective clothing might not be sufficient; consult a specialist BEFORE handling thisproduct.

Exposure Limits: Not available.

Section 9: Physical and Chemical Properties

Physical state and appearance: Solid. (Powdered solid.)

Odor: Not available.

Taste: Not available.

Molecular Weight: 208.22 g/mole

Color: Yellow. (Light.)

pH (1% soln/water): Not applicable.

Boiling Point: 377°C (710.6°F)

Melting Point: 284°C (543.2°F) - 286 C

Critical Temperature: Not available.

Specific Gravity: 1.44 (Water = 1)

Vapor Pressure: Not applicable.

Vapor Density: 7.16 (Air = 1)

Volatility: Not available.

Odor Threshold: Not available.

Water/Oil Dist. Coeff.:The product is more soluble in oil; log(oil/water) = 3.4

Ionicity (in Water): Not available.

Dispersion Properties: See solubility in water, acetone.

Solubility: Soluble in acetone. Insoluble in cold water, hot water.

Section 10: Stability and Reactivity Data

Stability: The product is stable.

Instability Temperature: Not available.

Conditions of Instability: Excess heat, ignition sources (flames), incompatible materials, dust generation

Incompatibility with various substances: Reactive with oxidizing agents.

Corrosivity: Not available.

Special Remarks on Reactivity: Not available.

Special Remarks on Corrosivity: Not available.

Polymerization: Will not occur.

Section 11: Toxicological Information

Routes of Entry: Inhalation. Ingestion.

Toxicity to Animals: WARNING: THE LC50 VALUES HEREUNDER ARE ESTIMATED ON THE BASIS OF A 4-HOUR EXPOSURE. Acute oraltoxicity (LD50): >5000 mg/kg [Mouse]. Acute dermal toxicity (LD50): >1000 mg/kg [Rat]. Acute toxicity of the dust (LC50):>1300 mg/m3 4 hours [Rat].

Chronic Effects on Humans: MUTAGENIC EFFECTS: Mutagenic for mammalian somatic cells. Mutagenic for bacteria and/ or yeast.

Other Toxic Effects on Humans: Slightly hazardous in case of skin contact (irritant), of ingestion, of inhalation.

Special Remarks on Toxicity to Animals:

Lowest Published Lethal Dose: LDL [Rat] - oral; Dose: 15000 mg/kg

Special Remarks on Chronic Effects on Humans: May affect genetic material.

Special Remarks on other Toxic Effects on Humans: Acute Potential Health Effects: Skin: May cause skin irritation. Eyes: Causes eye irritation. Inhalation: May cause respiratorytract irritation. Ingestion: May cause gastrointestinal tract irritation with nausea, vomiting, and diarrhea. Chronic PotentialHealth Effects: Skin: Repeated or prolonged exposure may cause skin sensitization, an allergic reaction. Ingestion: Prolonged

or repeated ingestion may affect the liver, blood, metabolism respiratory system (emphysema), and urinary system

Section 12: Ecological Information

Ecotoxicity:Not available.

BOD5 and COD: Not available.

Products of Biodegradation: Possibly hazardous short term degradation products are not likely. However, long term degradation products may arise.

Toxicity of the Products of Biodegradation: The products of degradation are less toxic than the product itself.

Special Remarks on the Products of Biodegradation:Not available.

Section 13: Disposal Considerations

Waste Disposal:

Section 14: Transport Information

14.1. Land transport (ADR-RID)

General Information		: ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID

: 9,

: III

14.2. Sea transport (IMDG) [English only]

General Information			: ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID

: 9

: III

14.3. Air transport (ICAO-IATA) [English only]

General Information		: ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID

: 9,

: III

Federal and State Regulations: New Jersey: Anthracene TSCA 8(b) inventory: Anthracene TSCA 4(a) proposed test rules: Anthracene TSCA 8(a)IUR: Anthracene TSCA 8(d) H and S data reporting: Anthracene: effective: 12/28/84; sunset: 11/09/93

Other Regulations: OSHA: Hazardous by definition of Hazard Communication Standard (29 CFR 1910.1200). EINECS: This product is on theEuropean Inventory of Existing Commercial Chemical Substances.

Other Classifications:

WHMIS (Canada): Not controlled under WHMIS (Canada).

DSCL (EEC): R20/21- Harmful by inhalation and in contact with skin. R36/37/38- Irritating to eyes, respiratory system and skin. R40-Possible risks of irreversible effects. R43- May cause sensitization by skin contact. S2- Keep out of the reach of children.S24- Avoid contact with skin. S26- In case of contact with eyes, rinse immediately with plenty of water and seek medicaladvice. S36/37/39- Wear suitable protective clothing, gloves and eye/face protection. S46- If swallowed, seek medical adviceimmediately and show this container or label.

HMIS (U.S.A.):

Health Hazard: 2

Fire Hazard: 1

Reactivity: 0

Personal Protection: E

National Fire Protection Association (U.S.A.):

Health: 0

Flammability: 1

Reactivity: 0

Specific hazard:

Protective Equipment: Gloves. Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Safety glasses.

Section 15: Other Regulatory Information(Continued)

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.