MATERIAL SAFETY DATA SHEET

ANTIMONY SULPHATE

MSDS CAS: 7446-32-4

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:Antimony Sulphate

CAS#:  7446-32-4

C.I. No.:Not available.

Synonym: Not available.

Chemical Name:Not available.

Chemical Formula:Sb2(SO4)3

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Antimony sulphate | 7446-32-4 | 98-100% |

Toxicological Data on Ingredients:Not applicable.

Section 3: Hazards Identification

HAZARDS IDENTIFICATION

Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008 [EU-GHS/CLP]

Acute toxicity, Inhalation (Category 4)

Acute toxicity, Oral (Category 4)

Chronic aquatic toxicity (Category 2)

Classification according to EU Directives 67/548/EEC or 1999/45/EC

Harmful by inhalation and if swallowed. Toxic to aquatic organisms, may cause long-term adverse effects

in the aquatic environment. Harmful by inhalation and if swallowed. Toxic to aquatic organisms, may cause

long-term adverse effects in the aquatic environment.

Label elements

Labelling according Regulation (EC) No 1272/2008 [CLP]

Pictogram

Signal word Warning

Hazard statement(s)

H302 Harmful if swallowed.

H332 Harmful if inhaled.

H411 Toxic to aquatic life with long lasting effects.

Precautionary statement(s)

P273 Avoid release to the environment.

Supplemental Hazard

Statements

none

Aldrich - 10783 Page 2 of 6

According to European Directive 67/548/EEC as amended.

Hazard symbol(s)

R-phrase(s)

R20/22 Harmful by inhalation and if swallowed.

R51/53 Toxic to aquatic organisms, may cause long-term adverse effects in the

aquatic environment.

S-phrase(s)

S61 Avoid release to the environment. Refer to special instructions/ Safety

data sheets.

Other hazards – none

Section 4: First Aid Measures

Description of first aid measures

General advice

Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

In case of skin contact

Wash off with soap and plenty of water. Consult a physician.

In case of eye contact

Flush eyes with water as a precaution.

If swallowed

Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Most important symptoms and effects, both acute and delayed

To the best of our knowledge, the chemical, physical, and toxicological properties have not been

thoroughly investigated.

Indication of any immediate medical attention and special treatment needed

no data available

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media

Use extinguishing measures that are appropriate to local circumstances and the surrounding environment.

Special hazards arising from the substance or mixture

Antimony oxide

Advice for firefighters

Wear self contained breathing apparatus for fire fighting if necessary.

Aldrich - 10783 Page 3 of 6

Further information

The product itself does not burn.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures

Use personal protective equipment. Avoid dust formation. Avoid breathing vapors, mist or gas. Ensure

adequate ventilation. Avoid breathing dust.

Environmental precautions

Prevent further leakage or spillage if safe to do so. Do not let product enter drains. Discharge into the

environment must be avoided.

Methods and materials for containment and cleaning up

Pick up and arrange disposal without creating dust. Sweep up and shovel. Keep in suitable, closed

containers for disposal.

Reference to other sections

For disposal see section 13.

Section 7: Handling and Storage

Precautions for safe handling

Avoid contact with skin and eyes. Avoid formation of dust and aerosols.

Provide appropriate exhaust ventilation at places where dust is formed.

Conditions for safe storage, including any incompatibilities

Store in cool place. Keep container tightly closed in a dry and well-ventilated place.

Specific end uses

no data available

Section 8: Exposure Controls/Personal Protection

Control parameters

Components with workplace control parameters

Exposure controls

Appropriate engineering controls

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and

at the end of workday.

Personal protective equipment

Eye/face protection

Safety glasses with side-shields conforming to EN166 Use equipment for eye protection tested

and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

Section 8: Exposure Controls/Personal Protection (continued)

Skin protection

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique

(without touching glove's outer surface) to avoid skin contact with this product. Dispose of

contaminated gloves after use in accordance with applicable laws and good laboratory practices.

Wash and dry hands.

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and

the standard EN 374 derived from it.

Body Protection

Complete suit protecting against chemicals, The type of protective equipment must be selected

according to the concentration and amount of the dangerous substance at the specific workplace.

Respiratory protection

For nuisance exposures use type P95 (US) or type P1 (EU EN 143) particle respirator.For higher

level protection use type OV/AG/P99 (US) or type ABEK-P2 (EU EN 143) respirator cartridges.

Use respirators and components tested and approved under appropriate government standards

such as NIOSH (US) or CEN (EU).

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

a) Appearance Form: solid

b) Odour                                                         : no data available

c) Odour Threshold                                        : no data available

d) pH                                                               : no data available

e) Melting point/freezing                                : no data available

point

f) Initial boiling point and                              : no data available

boiling range

g) Flash point                                                   : not applicable

h) Evaporation rate                                         : no data available

i) Flammability (solid, gas)                             : no data available

j) Upper/lower                                                 : no data available

flammability or

explosive limits

k) Vapour pressure                                         : no data available

l) Vapour density                                            : no data available

Section 9: Physical and Chemical Properties (continued)

m) Relative density                                            : no data available

n) Water solubility                                             : no data available

o) Partition coefficient: noctanol/                     : no data available

water

p) Autoignition                                                   : no data available

temperature

q) Decomposition                                               : no data available

temperature

r) Viscosity : no data available

s) Explosive properties : no data available

t) Oxidizing properties : no data available

Other safety information                       : no data available

Section 10: Stability and Reactivity Data

Reactivity

no data available

Chemical stability

no data available

Possibility of hazardous reactions

no data available

Conditions to avoid

no data available

Incompatible materials

Strong oxidizing agents

Hazardous decomposition products

Other decomposition products - no data available

Section 11: Toxicological Information

Routes of Entry: Inhalation. Ingestion.

Toxicity to Animals: LD50: Not available. LC50: Not available.

Chronic Effects on Humans: Not available.

Other Toxic Effects on Humans: Slightly hazardous in case of skin contact (irritant), of ingestion, of inhalation.

Special Remarks on Toxicity to Animals: Not available.

Special Remarks on Chronic Effects on Humans: Not available.

Special Remarks on other Toxic Effects on Humans: Acute Potential Health Effects: Skin: Eyes: May cause mechanical eye irritation. Inhalation: May cause respiratory tractirritation. Ingestion: No recognized

health hazard. Chronic Potential Health Effects: Inhalation: Prolonged or repeatedinhalation of high concentrations may lead to a depositoin of tricalcium phosphate crystals in the lung tissue The toxicological

properties of this substance have not been fully investigated

Section 12: Ecological Information

Toxicity

no data available

Persistence and degradability

no data available

Bioaccumulative potential

no data available

Mobility in soil

no data available

Results of PBT and vPvB assessment

no data available

Other adverse effects

Toxic to aquatic life with long lasting effects.

Section 13: Disposal Considerations

Waste Disposal:

Product

Offer surplus and non-recyclable solutions to a licensed disposal company.

Contaminated packaging

Dispose of as unused product.

Section 14: Transport Information

UN number

ADR/RID: 3077 IMDG: 3077 IATA: 3077

UN proper shipping name

ADR/RID: ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID, N.O.S. (Diantimony

tris(sulphate))

IMDG: ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID, N.O.S. (Diantimony

tris(sulphate))

IATA: Environmentally hazardous substance, solid, n.o.s. (Diantimonytris(sulphate))

Transport hazard class(es)

ADR/RID: 9 IMDG: 9 IATA: 9

Packaging group

ADR/RID: III IMDG: III IATA: III

Environmental hazards

ADR/RID: yes IMDG Marine pollutant: yes IATA: yes

Special precautions for user

Further information

EHS-Mark required (ADR 2.2.9.1.10, IMDG code 2.10.3) for single packagings and combination

packagings containing inner packagings with Dangerous Goods > 5L for liquids or > 5kg for solids.

Section 15: Other Regulatory Information

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

Safety, health and environmental regulations/legislation specific for the substance or mixture

no data available

Chemical Safety Assessment

no data available

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.