MATERIAL SAFETY DATA SHEET

ANTIMONY SULPHIDE BLACK

(EXTRA PURE)

MSDS CAS: 1345-04-6

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:Antimony Sulphide Black

CAS#:  1345-04-6

C.I. No.:Not available.

Synonym: Not available.

Chemical Name:Not available.

Chemical Formula: Sb2S3

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Antimony Sulphide Black | 1345-04-6 | 100 |

Toxicological Data on Ingredients:Not applicable.

Section 3: Hazards Identification

Classification of the substance or mixture

Classification EC 67/548 or EC 1999/45

Classification		: Xn; R20/22

N; R51-53

Hazard Class and Category Code(s), Regulation (EC) No 1272/2008 (CLP)

Health hazards	: Specific Target Organ Toxicity - Single exposure - Respiratory tract               irritation -Category 3 - Warning (CLP : STOT SE 3) H335

Acute toxicity, Oral - Category 4 - Warning (CLP : Acute Tox. 4) H302

Skin irritation - Category 2 - Warning (CLP : Skin Corr. 2) H315

Eye irritation - Category 2A - Warning (CLP : Eye Irrit. 2) H319

Acute toxicity, Inhalation - Category 4 - Warning (CLP : Acute Tox. 4) H332Environmental hazards : Hazardous to the aquatic environment - Acute hazard - Category 1 - Warning (CLP: Aquatic Acute 1) H400

Hazardous to the aquatic environment - Chronic hazard - Category 2 (CLP :Aquatic Chronic 2) H411

Section 4: First Aid Measures

Description of first aid measures

Inhalation :Assure fresh air breathing. Allow the victim to rest. Remove to fresh air and keep at rest in a position comfortable for breathing. Call a POISON CENTER or doctor if you feel unwell.

Skin contact :Remove affected clothing and wash all exposed skin area with mild soap and water, followed by warm water rinse. Wash contaminated clothing before reuse. Wash with plenty of soap and water. Specific treatment (see on this label). Get medical advice. If skin irritation occurs :

Eye contact :Rinse immediately with plenty of water. Obtain medical attention if pain, blinking or redness persist. Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. If eye irritation persists : Get medical advice.

Ingestion :Obtain emergency medical attention. Rinse mouth. Call a POISON CENTER or doctor if you feel unwell. Do NOT induce vomiting.

Most important symptoms and effects, both acute and delayed

Symptoms relating to use :Danger of serious damage to health by prolonged exposure through inhalation.

Swallowing a small quantity of this material will result in serious health hazard.

Causes serious eye irritation.

Causes skin irritation. May cause respiratory irritation.

Indication of any immediate medical attention and special treatment needed

General information :Never give anything by mouth to an unconscious person. If you feel unwell, seek

medical advice (show the label where possible).

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media :Foam. Dry powder. Carbon dioxide. Water spray. Sand.

Unsuitable extinguishing media :Do not use a heavy water stream.

Surrounding fires :Use water spray or fog for cooling exposed containers.

Special hazards arising from the substance or mixture

Hazardous combustion products : Under fire conditions, hazardous fumes will be present.

Advice for fire-fighters

Protection against fire :Do not enter fire area without proper protective equipment, including respiratory

protection.

Special procedures :Exercise caution when fighting any chemical fire. Avoid (reject) fire-fighting water to

enter environment.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures

For emergency responders : Equip cleanup crew with proper protection.

Ventilate area.

For non-emergency personnel : Evacuate unnecessary personnel.

Environmental precautions

Environmental precautions :Prevent entry to sewers and public waters. Notify authorities if product enterssewers or public waters.Avoid release to the environment.

Methods and material for containment and cleaning up

Clean up methods :Soak up spills with inert solids, such as clay or diatomaceous earth as soon as

possible. Collect spillage. Store away from other materials.

Reference to other sections

See section 8. Exposure controls/personal protection

Section 7: Handling and Storage

Handling :Wash thoroughly after handling. Use only outdoors or in a well-ventilated area. Wash thoroughly after handling. Avoid breathing dust, fume, gas, mist, vapours, spray. Do not eat, drink or smoke when using this product.

Technical protective measures :Provide good ventilation in process area to prevent formation of vapour.

Special precautions :Avoid breathing dust, fume, gas, mist, vapours, spray.

Conditions for safe storage, including any incompatibilities

Storage :Keep only in the original container in a cool, well ventilated place. Keep container

tightly closed.

Storage - away from :Strong bases. Strong acids. Sources of ignition. Direct sunlight.

Specific end use(s)

Specific end use(s) :None..

Section 8: Exposure Controls/Personal Protection

Exposure controls

Personal protection :Avoid all unnecessary exposure.

• Respiratory protection :In case of insufficient ventilation, wear suitable respiratory equipment.

• Hand protection :Wear protective gloves.

• Skin protection : Wear suitable protective clothing.

• Eye protection :Chemical goggles or safety glasses.

• Others :When using, do not eat, drink or smoke.

Control parameters

Occupational Exposure Limits :No data available.

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Physical state at 20 °C			: Solid.

Colour					: Black

Odour					: Odorless

Odour threshold				: No data available.

pH value					: No data available.

Melting point [°C]			: 550 °C

Section 9: Physical and Chemical Properties(Continued)

Decomposition point [°C] 		: N/A

Critical temperature [°C] 		: N/A

Auto-ignition temperature [°C]: N/A

Flammability (solid, gas) 		: N/A

Flash point [°C] 				: N/A

Boiling point [°C]			: N/A

Initial boiling point [°C]	: N/A

Final boiling point [°C]		: N/A

Evaporation rate			: N/A

Vapour pressure [20°C]		: N/A

Vapour pressure mm/Hg		: N/A

Vapour density 				: N/A

Density [g/cm3]			: 4,64

Relative density, gas (air=1)	: N/A

Relative density, liquid (water=1): N/A

Solubility in water [% weight] 	: Insoluble in water

Solubility in water		: N/A

Log Pow octanol / water at 20°C : No data available.

Solubility 					: N/A

Viscosity at 40°C [mm2/s] 		: N/A

Other information

Explosive properties 			: N/A

Explosion limits - upper [%]: N/A

Explosion limits - lower [%] 		: N/A

Oxidising properties 			: No data available.

Section 10: Stability and Reactivity Data

Reactivity

Reactivity :Highly reactive material.

Chemical stability

Chemical stability :Stable under recommended storage conditions.

Possibility of hazardous reactions

Hazardous reactions : Not established.

Conditions to avoid

Section 10: Stability and Reactivity Data (continued)

Conditions to avoid :Direct sunlight. Extremely high or low temperatures.

Incompatible materials

Materials to avoid :Strong acids. Strong bases.

Hazardous decomposition products

Hazardous decomposition products :Fumes. Carbon monoxide. Carbon dioxide.

Section 11: Toxicological Information

Information on toxicological effects

Acute toxicity

• Inhalation				: Harmful if inhaled.

• Dermal 				: Based on available data, the classification criteria are not met.

• Ingestion				: Harmful if swallowed.

Corrosion 				: Based on available data, the classification criteria are not met.

Irritation 				: Causes serious eye irritation.Causes skin irritation. May cause respiratory irritation.

Sensitization 			: Based on available data, the classification criteria are not met.

Mutagenicity			: Based on available data, the classification criteria are not met.

Carcinogenicity 			: Based on available data, the classification criteria are not met.

Toxic for reproduction		: Based on available data, the classification criteria are not met.

STOT-single exposure 		: Based on available data, the classification criteria are not met.

STOT-repeated exposure	: Based on available data, the classification criteria are not met.

Aspiration hazard 		: Based on available data, the classification criteria are not met.

Section 12: Ecological Information

Toxicity

Toxicity information :Very toxic to aquatic life.

Toxic to aquatic life with long lasting effects.

Persistence - degradability

Persistence - degradability :May cause long-term adverse effects in the environment.

Bioaccumulative potential

Bioaccumulative potential :Not established.

Mobility in soil

Mobility in soil :Not established.

Results of PBT and vPvB assessment

Results of PBT and vPvBassessment :The substance does not fulfil the criteria to be identified as PBT substance or vPvB

substance according to Annex XIII of Regulation REACH.

Other adverse effects

Environmental precautions :Avoid release to the environment.

Section 13: Disposal Considerations

Waste Disposal:

General : Avoid release to the environment. Dispose in a safe manner in accordance with local/national regulations. Dispose of this material and its container to hazardous or special waste collection point, in accordance with local, regional, national and/or international regulation.

Section 14: Transport Information

14.1. Land transport (ADR-RID)

Proper shipping name			: ANTIMONY COMPOUND, INORGANIC, SOLID, N.O.S.

UN N° 					: 1549

H.I. nr 				``	: 60

ADR - Class 				: 6.1

Labelling - Transport 			: 6.1 : Toxic substances.

ADR – Group				: III

Section 14: Transport Information (continued)

14.2. Sea transport (IMDG) [English only]

Proper shipping name			: ANTIMONY COMPOUND, INORGANIC, SOLID, N.O.S.

UN N° 					: 1549

IMO-IMDG - Class or division	: 6.1 : Toxic substances.

IMO-IMDG - Packing group		: III

14.3. Air transport (ICAO-IATA) [English only]

Proper shipping name			: ANTIMONY COMPOUND, INORGANIC, SOLID, N.O.S.

UN N°					: 1549

IATA - Class or division		: 6.1 : Toxic substances.

IATA - Packing group 			: III

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture

: Ensure all national/local regulations are observed.

REACH Restrictions - Annex XVII :The components of this product are not subject to restrictions.

REACH Authorisation - Annex XIV :The components of this product are not subject to authorization.

Chemical Safety Assessment

Chemical Safety Assessment : It has not been carried out.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.