MATERIAL SAFETY DATA SHEET

D-ARGININE

MSDS :157-06-2

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:D-ARGININE

CAS#: 157-06-2

C.I. No.:Not available.

Synonym:(R)-2-Amino-5-guanidinopentanoic acid

Chemical Name:Not available.

Chemical Formula:C6H14N4O2

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| D-ARGININE | 157-06-2 | 100 |

Section 3: Hazards Identification

Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008

Eye irritation (Category 2), H319

For the full text of the H-Statements mentioned in this Section.

Classification according to EU Directives 67/548/EEC or 1999/45/EC

Xi			 Irritant 			R36

For the full text of the R-phrases mentioned in this Section.

Label elements

Hazard statement(s)

H319 					Causes serious eye irritation.

Precautionary statement(s)

P305 + P351 + P338 			IF IN EYES: Rinse cautiously with water for several minutes. Remove

contact lenses, if present and easy to do. Continue rinsing.

Supplemental Hazard		 none

Statements

Other hazards:None

Section 4: First Aid Measures

Description of first aid measures

General advice

Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

In case of skin contact

Wash off with soap and plenty of water. Consult a physician.

In case of eye contact

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed

Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Most important symptoms and effects, both acute and delayed

The most important known symptoms and effects are described in the labelling and/or in

section 11

Indication of any immediate medical attention and special treatment needed

No data available

Section 5: Fire and Explosion Data

Extinguishing media

Extinguishing media

Suitable extinguishing media

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special hazards arising from the substance or mixture

Carbon oxides, nitrogen oxides (NOx)

Advice for firefighters

Wear self contained breathing apparatus for fire fighting if necessary.

Further information

no data available

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures

Use personal protective equipment. Avoid dust formation. Avoid breathing vapours, mist or gas. Ensure adequate ventilation. Avoid breathing dust. For personal protection see section 8.

Environmental precautions

Do not let product enter drains.

Methods and materials for contain ent and cleaning up

Pick up and arrange disposal without creating dust. Sweep up and shovel. Keep in suitable, closed containers for disposal.

Reference to other sections

For disposal see section 13.

Section 7: Handling and Storage

Precautions for safe handling

Avoid contact with skin and eyes. Avoid formation of dust and aerosols. Provide appropriate exhaust ventilation at places where dust is formed.

Conditions for safe storage, including any incompatibilities

Store in cool place. Keep container tightly closed in a dry and well-ventilated place.

Section 7: Handling and Storage

Specific end uses

Apart from the uses mentioned in section 1.3 no other specific uses are stipulated

Section 8: Exposure Controls/Personal Protection

Control parameters

Components with workplace control parameters

Exposure controls

Appropriate engineering controls

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday.

Personal protective equipment

Eye/face protection

Safety glasses with side-shields conforming to EN166 Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

Skin protection

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices. Wash and dry hands.

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

Body Protection

impervious clothing, The type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace.

Respiratory protection

For nuisance exposures use type P95 (US) or type P1 (EU EN 143) particle respirator.For higher level protection use type OV/AG/P99 (US) or type ABEK-P2 (EU EN 143) respirator cartridges. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Control of environmental exposure

Do not let product enter drains.

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

a)Appearance Form					: powder

Colour						: White

b)Odour							: No data available.

c)Odour threshold					: No data available.

d)pH								: Not applicable.

e)Melting point/Freezing point			: 226 °C - dec.

f)Initial boiling point andboiling range		: No data available.

g)Autoignition temperature				: No data available.

h)Flammability (solid, gas)				: No data available.

i)Upper/lower flammability or explosive limits	 : No data available.

j)Flash point [°C]						: No data available.

k)Evaporation rate					: No data available.

l)Vapour pressure					: No data available.

m)Vapour density					: No data available.

n)Relative density,					: No data available.

o)Solubility in water					: No data available.

p)Viscosity							: No data available.

q)Explosive properties					: No data available.

r)Oxidising properties					: No data available.

s)Decomposition temperature	: No data available.

t)Autoignition temperature				 : No data available.

u)Molecular Weight 					: 174,20 g/mol

Other safety information

no data available

Section 10: Stability and Reactivity Data

Reactivity :No data available.

Chemical stability :Stable under recommended storage conditions.

Possibility of hazardous reactions : No data available.

Conditions to avoid : No data available.

Section 10: Stability and Reactivity Data

Incompatible materials:Strong oxidizing agents.

Hazardous decomposition products

Other decomposition products - no data available

In the event of fire: see section 5

Section 11: Toxicological Information

Information on toxicological effects

Acute toxicity

no data available

Skin corrosion/irritation

no data available

Serious eye damage/eye irritation

no data available

Respiratory or skin sensitization

no data available

Germ cell mutagenicity

no data available

Carcinogenicity

IARC: No component of this product present at levels greater than or equal to 0.1% is identified asprobable, possible or confirmed human carcinogen by IARC.

Reproductive toxicity

no data available

Specific target organ toxicity - single exposure

no data available

Specific target organ toxicity - repeated exposure

no data available

Aspiration hazard

no data available

Additional Information

RTECS: Not available

To the best of our knowledge, the chemical, physical, and toxicological properties have not been

thoroughly investigated.

Section 12: Ecological Information

Toxicity: No data available.

Persistence - degradability :No data available.

Bioaccumulative potential : Not established.

Mobility in soil : Not established.

Results of PBT and vPvBassessment :

PBT/vPvB assessment not available as chemical safety assessment not required/not conducted

Other adverse effects :

No data available.

Section 13: Disposal Considerations

Waste treatment methods

Product

Offer surplus and non-recyclable solutions to a licensed disposal company.Dissolve or mix the material with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and scrubber.

Contaminated packaging

Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

General information			: Not regulated.

Sea transport (IMDG) [English only]

General information			: Not regulated.

Air transport (ICAO-IATA) [English only]

General information			: Not regulated.

Section 15: Other Regulatory Information

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

Safety, health and environmental regulations/legislation specific for the substance or mixture

no data available

Chemical Safety Assessment

For this product a chemical safety assessment was not carried out

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.