MATERIAL SAFETY DATA SHEET

ARSENIC TRIOXIDE 99% AR

(Arsenic (III) Oxide)

MSDS CAS :1327-53-3

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:ARSENIC TRIOXIDE 99% AR

CAS#:1327-53-3

C.I. No.:Not available.

Synonym :Arsenous acid / Arsenic trioxide

Chemical Name:Not available.

Chemical Formula:Not available.

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| ARSENIC TRIOXIDE AR | 1327-53-3 | 100% |

Section 3: Hazards Identification

Label elements

Labelling according to Regulation (EC) No. 1272/2008 [CLP]

Hazard statements (CLP) :H300 - Fatal if swallowed

H314 - Causes severe skin burns and eye damage

H350 - May cause cancer

H410 - Very toxic to aquatic life with long lasting effects

Precautionary statements (CLP) :

P201 - Obtain special instructions before use

P264 - Wash hands thoroughly after handling

P273 - Avoid release to the environment

P280 - Wear protective gloves, protective clothing, eye protection, face protection

P305+P351+P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing

P310 - Immediately call a doctor

Other hazards:

This substance/mixture does not meet the PBT criteria of REACH regulation, annex XIII

This substance/mixture does not meet the vPvB criteria of REACH regulation, annex XIII

Section 4: First Aid Measures

Description of first aid measures

First-aid measures after inhalation : Remove victim to fresh air and keep at rest in a position comfortable for breathing. Immediately call a POISON CENTER or doctor/physician.

First-aid measures after skin contact : Get immediate medical advice/attention. Wash with plenty of soap and water.

First-aid measures after eye contact : Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Immediately call a POISON CENTER or doctor/physician.

First-aid measures after ingestion : Rinse mouth. Immediately call a POISON CENTER or doctor/physician. Do NOT induce vomiting.

Most important symptoms and effects, both acute and delayed

Symptoms/injuries : Causes severe skin burns and eye damage.

Symptoms/injuries after inhalation : May cause cancer by inhalation.

Symptoms/injuries after ingestion : Fatal if swallowed.

Indication of any immediate medical attention and special treatment needed

Treat symptomatically.

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media : Carbon dioxide. Dry powder. Foam. Water spray.

Unsuitable extinguishing media : Do not use a heavy water stream.

Special hazards arising from the substance or mixture:No additional information available

Advice for firefighters

Protection during firefighting : Do not enter fire area without proper protective equipment, including respiratory protection.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures

For non-emergency personnel

Emergency procedures : Evacuate unnecessary personnel.

For emergency responders

Protective equipment : Use personal protective equipment as required.

Emergency procedures : Ventilate area.

Environmental precautions:Avoid release to the environment.

Methods and material for containment and cleaning up

Methods for cleaning up : Soak up spills with inert solids, such as clay or diatomaceous earth as soon as possible. On land, sweep or shovel into suitable containers.

Section 7: Handling and Storage

Precautions for safe handling

Precautions for safe handling :Do not breathe vapours. Obtain special instructions before use. Avoid contact with skin and eyes.

Hygiene measures :Do not eat, drink or smoke when using this product. Wash contaminated clothing before reuse. Wash hands and other exposed areas with mild soap and water before eating, drinking or smoking and when leaving work.

Section 7: Handling and Storage (Continued)

Conditions for safe storage, including any incompatibilities

Storage conditions : Store in a well-ventilated place. Keep container tightly closed.

Section 8: Exposure Controls/Personal Protection

Control parameters: No additional information available

Exposure controls Hand protection :Protective gloves

Eye protection :Chemical goggles or face shield

Skin and body protection :Wear suitable protective clothing Respiratory protection : Wear respiratory protection

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Physical state at 20 °C			: Solid.

Color						: White powder

Odor						: Odourless

Odor threshold				: No data available.

pH value					: No data available.

Melting point [°C]			: 312 °C

Decomposition point [°C]		:N/A

Critical temperature [°C] 		: N/A

Auto-ignition temperature [°C]	: N/A

Flammability (solid, gas)		:N/A

Flash point [°C]				: No data available.

Boiling point [°C]				:457°C

Initial boiling point [°C]		: N/A

Final boiling point [°C]			: N/A

Evaporation rate				: N/A

Vapour pressure [20°C]			: N/A

Vapour pressure mm/Hg		: N/A

Vapour density				: N/A

Section 9: Physical and Chemical Properties (Continued)

Density [g/cm3]				:3.74 g/cm³

Relative density, gas (air=1)		: N/A

Relative density, liquid (water=1)	: N/A

Solubility in water [% weight]	: Patially soluble in

Solubility in water			:N/A

Log Pow octanol / water at 20°C	: No data available.

Solubility					: Water: 3.7 g/100ml @ 20°C

Viscosity at 40°C [mm2/s]		: N/A

Molecular Weight                               : 128.09

Other information

Explosive properties			: N/A

Explosion limits - upper [%]		: N/A

Explosion limits - lower [%]		: N/A

Oxidizing properties			: No data available.

Section 10: Stability and Reactivity Data

Reactivity:Thermal decomposition generates : Corrosive vapours

Chemical stability:Stable under normal conditions.

Possibility of hazardous reactions:No additional information available

Conditions to avoid:No additional information available

Incompatible materials: No additional information available.

Hazardous decomposition products:Thermal decomposition generates : Corrosive vapours.

Section 11: Toxicological Information

Information on toxicological effects

Acute toxicity: Oral: Fatal if swallowed.

Section 11: Toxicological Information (Continued)

Skin corrosion/irritation : Causes severe skin burns and eye damage.

Serious eye damage/irritation : Serious eye damage, category 1, implicit

Respiratory or skin sensitisation : Not classified Germ cell mutagenicity : Not classified Carcinogenicity : May cause cancer.

Reproductive toxicity : Not classified

Specific target organ toxicity (single exposure) : Not classified

Specific target organ toxicity (repeated exposure): Not classified

Aspiration hazard : Not classified

Potential adverse human health effects and symptoms : Fatal if swallowed.

Section 12: Ecological Information

Toxicity

Toxicity information :Very toxic to aquatic life with long lasting effects.

Persistence – degradability

Persistence - degradability :May cause long-term adverse effects in the environment.

Bio accumulative potential

Bio accumulativepotential :No additional information available

Mobility in soil

Mobility in soil :No additional information available

Results of PBT and vPvB assessment

Results of PBT and vPvBassessment:The substance does not fulfil the criteria to be identified as PBT substance or vPvBsubstance according to Annex XIII of Regulation REACH.

Other adverse effects

Environmental precautions:No additional information available

Section 13: Disposal Considerations

Waste treatment methods

General:Avoid release to the environment.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name		: ARSENIC TRIOXIDE

UN N° 				: 1561

ADR - Class 			: 6.1

Labelling – Transport 	:  UN 1561 ARSENIC TRIOXIDE, 6.1, II, (D/E), ENVIRONMENTALLY HAZARDOUS

Sea transport (IMDG) [English only]

Proper shipping name		: ARSENIC TRIOXIDE

UN N° 				: 1561

IMO-IMDG - Class or division 	:6.1

IMO-IMDG - Packing group 	:II

Air transport (ICAO-IATA) [English only]

Proper shipping name 		: ARSENIC TRIOXIDE

UN N° 				: 1561

IATA - Class or division 		: 6.1

IATA - Packing group 		:II

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture

15.1.1. EU-Regulations No REACH Annex XVII restrictions

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.