MATERIAL SAFETY DATA SHEET

AIBN 98% (For Synthesis)

MSDS CAS: 78-67-1

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:AIBN 98% (For Synthesis)

CAS#: 78-67-1

C.I. No.:Not available.

Synonym:Not available.

Chemical Name:alpha,alpha-AZOISOBUTYRONITRILE.

Chemical Formula:C8H12N4

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| alpha,alpha-Azoisobutyronitrile | 78-67-1 | 100 |

Section 3: Hazards Identification

The substance does not fulfil the criteria to be identified as PBT substance or vPvB

substance according to Annex XIII of Regulation REACH.

Section 4: First Aid Measures

Eye Contact:

Rinse immediately with plenty of water. Obtain medical attention if pain, blinking or redness persist.

Skin Contact:

Remove affected clothing and wash all exposed skin area with mild soap and water, followed by warm water rinse.

Inhalation:

Assure fresh air breathing. Allow the victim to rest. Remove to fresh air and keep at rest in a position comfortable for breathing. Call a POISON CENTER or doctor if you feel unwell.

Serious Inhalation: Not available.

Ingestion:

Obtain emergency medical attention. Call a POISON CENTER or doctor if you feel unwell. Rinse mouth. Do NOT induce vomiting.

Serious Ingestion: Not available.

Section 5: Fire and Explosion Data

Hazardous combustion products:

Under fire conditions, hazardous fumes will be present

Extinguishing media:

Suitable extinguishing media: Foam. Dry powder. Carbon dioxide. Water spray. Sand.

Unsuitable extinguishing media: Do not use a heavy water stream.

Surrounding fires: Use water spray or fog for cooling exposed containers.

Flammable Limits: Not available.

Fire Hazards in Presence of Various Substances: Not available.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

For emergency responders           : Equip cleanup crew with proper protection. Ventilate area.

For non-emergency personnel     : Evacuate unnecessary personnel.

Environmental precautions          : Prevent entry to sewers and public waters. Notify authorities if product enters sewers or public waters.

Section 7: Handling and Storage

Precautions:

Wash thoroughly after handling. Do not eat, drink or smoke when using this product. Avoid breathing dust, fume, gas, mist, vapours, spray. Use only outdoors or in a well-ventilated area.

Storage:

Keep only in the original container in a cool, well ventilated place. Keep container closed when not in use.

Storage away from:

Strong bases. Strong acids. Sources of ignition. Direct sunlight.

Section 8: Exposure Controls/Personal Protection

Exposure controls:

Personal protection: Avoid all unnecessary exposure.

• Respiratory protection: Wear approved mask.

• Hand protection: Wear protective gloves.

• Eye protection: Chemical goggles or safety glasses.

• Others: When using, do not eat, drink or smoke.

Section 9: Physical and Chemical Properties

Physical state and appearance: Solid.

Odor: Characteristic

Taste: Not available.

Molecular Weight: Not available.

Section 9: Physical and Chemical Properties (Continued)

Color: White to off white.

pH (1% soln/water): Not applicable.

Boiling Point: Not available.

Melting Point: 102 - 104 °C - dec

Critical Temperature: Not available.

Specific Gravity: Not available.

Vapor Pressure: Not applicable.

Vapor Density: Not available.

Volatility: Not available.

Odor Threshold: Not available.

Water/Oil Dist. Coeff.:Not available.

Ionicity (in Water): Not available.

Dispersion Properties: Not available.

Solubility: Not available.

Section 10: Stability and Reactivity Data

Stability: Stable under recommended storage conditions.

Instability Temperature: Not available.

Conditions of Instability: Not available.

Incompatibility with various substances: Strong acids. Strong bases.

Corrosivity: Not available.

Special Remarks on Reactivity: Not available.

Special Remarks on Corrosivity: Not available.

Polymerization: Will not occur.

Section 11: Toxicological Information

Routes of Entry: Not available.

Toxicity to Animals: Not available.

Chronic Effects on Humans: Not available.

Other Toxic Effects on Humans:Not available.

Special Remarks on Toxicity to Animals: Not available.

Special Remarks on Chronic Effects on Humans: Not available.

Special Remarks on other Toxic Effects on Humans: Not available.

Section 12: Ecological Information (Continued)

Ecotoxicity: Not established.

BOD5 and COD: Not available.

Products of Biodegradation: Not available.

Toxicity of the Products of Biodegradation: Not available.

Special Remarks on the Products of Biodegradation: Not available.

Section 13: Disposal Considerations

Waste Disposal:Avoid release to the environment. Dispose in a safe manner in accordance with local/national regulations.Dispose of this material and its container to hazardous or special waste collection

point, in accordance with local, regional, national and/or international regulation.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name : AIBN 98% (For Synthesis)

UN N° 		: 3234

ADR - Class 	: 4.1: Flammable solids, self-reactive substances and desensitized explosives.

Sea transport (IMDG) [English only]

Proper shipping name: AIBN 98% (For Synthesis)

UN N° 		: 3234

IMO-IMDG - Class or division :4.1: Flammable solids, self-reactive substances and desensitized explosives.

IMO-IMDG - Packing group :III

Air transport (ICAO-IATA) [English only]

Proper shipping name : AIBN 98% (For Synthesis)

UN N° : 3234

IATA - Class or division : 4.1: Flammable solids, self-reactive substances and desensitized explosives.

IATA - Packing group : III

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture:

Ensure all national/local regulations are observed.

REACH Restrictions - Annex XVII    : The components of this product are not subject to restrictions.

REACH Authorisation - Annex XIV   : The components of this product are not subject to authorization.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.