MATERIAL SAFETY DATA SHEET

BARIUM OXALATE 99% Extra Pure

MSDS CAS: 516-02-9

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: BARIUM OXALATE 99% Extra Pure

CAS#: 516-02-9

Synonym:

Chemical Name: Not available.

Chemical Formula: BaC2O4

Molecular Weight: 225.34

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight | Hazardous |
| --- | --- | --- | --- |
| Barium Oxalate | 516-02-9 | 98-100% | Yes |

Section 3: Hazards Identification

Emergency Overview:Warning! Get medical attention immediately if ingested. Renal damage can result.

Potential Chronic Health Effects: Minor irritation may be the result of inhalation, skin contact, or eye contact. Redness may also occur upon skin contact. Ingestion may cause severe gastroenteritis with vomiting and diarrhea. Renal damage can result from ingestion.

Section 4: First Aid Measures

Eye Contact: Flush with large amounts of water to remove all trace of chemical from eyes (at least 15-20 minutes). Get medical attention.

Skin Contact: Wash with soap and water. Get medical attention if redness and irritation persists.

Inhalation:Move to fresh air. If breathing becomes troublesome, get medical attention.

Serious Inhalation: Not available.

Ingestion:  Get medical attention immediately.

Section 5: Fire and Explosion Data

Extinguishing Media:Water, water spray, or foam.

Special Fire Fighting Procedures:Move containers from fire if it can be done without risk.

Fire and Explosion Hazzards: Not available.

Hazardous Combustion Products:Reacts with strong acids to form oxalic acid. Runoff water may be corrosive and toxic.

Section 5: Fire and Explosion Data(Continued)

Protective Measure in Fire:Avoid contact and dike area for later disposal. Prevent runoff from entering natural waterways.

Special Remarks on Fire Hazards:Not available.

Special Remarks on Explosion Hazards:Not available.

Section 6: Accidental Release Measures

Personal Precaution in Spill: Avoid contact with material. Wear personal protective equipment as outlined in Section 8.

Environment:Dike ahead of large spill to prevent material from entering natural waterways. Restrict access to spill area.

Spill Clean Up:Sweep up carefully to remove as much uncontaminated material as possible. Contaminated material is considered a toxic waste and must be disposed of in a licensed toxic waste dump

Section 7: Handling and Storage

Handling Precautions:Store away from strong acids. Wash hands before eating or smoking. No eating or smoking in work area.

Storage Precautions:Not Available

Section 8: Exposure Controls/Personal Protection

Protective Equipment:Use adequate ventilation to meet exposure limits. Where exposure limits may be exceeded, use OSHA approved respirator.

Section 8: Exposure Controls/Personal Protection (Continued)

Process Conditions:Threshold Limit Value: TWAair (Barium compounds) 500 mg/m3

Respirators:Use OSHA approved respirator.

Protective Gloves: Wear general purpose gloves.

Eye Protection:Wear chemical safety goggles.

OtherProtection:Wear full cover protective clothing.

Hygienic Work Practices: No smoking or eating in work area.

Section 9: Physical and Chemical Properties

Physical state and appearance	: Powder.

Odor						: Odourless.

Taste						: Not available.

Specific Gravity				: 2.658

Solubility					:Soluble in water.

Solubility Value				: 0.0016

Melting Point				:Not Available.

Boiling Point                  			:Not Available.

Section 10: Stability and Reactivity Data

Stability				: The product is stable.

Hazardous Decomp Products		: Reacts with strong acids to form Oxalic acid.

Conditionsto avoid			: Contact with strong acids.

Section 11: Toxicological Information

Toxic Dose LD 50:Not Available

Toxicological Info: Severe irritation if ingested - severe gastroenteritis with vomiting and diarrhea. Renal damage can result.

Inhalation:Not available

Skin: Not Availble

Eyes: Not available.

Medical Symptoms:Not available

Section 12: Ecological Information

Toxicity: Not available.

Distribution: Not available.

Chemical Fate:Not available.

Section 13: Disposal Considerations

Waste Disposal:Contaminated residue and sweepings are considered toxic waste and must be disposed of in a licensed toxic waste dump. If quantities are large enough to be significant, contact manufacturer for proper treatment.

Section 14: Transport Information

1. Land transport (ADR-RID)

Proper shipping name 				: Barium compounds, n.o.s.

UN N° 						: UN1564

Section 14: Transport Information (Continued)

ADR - Class 					:6.1

ADR - Group 					: II

2. Sea transport (IMDG) [English only]

Proper shipping name 				: Barium compounds, n.o.s.

UN N° 						: UN1564

IMO-IMDG - Class or division 		: 6.1

IMO-IMDG - Packing group 			: II

3. Air transport (ICAO-IATA) [English only]

Proper shipping name 				: Barium compounds, n.o.s.)

UN N° 						: UN1564

IATA - Class or division 			: 6.1

IATA - Packing group 				: II

Section 15: Other Regulatory Information

Chemical Inventory Status Part 1:

Ingredient:CASNO:TSCA:EINECS:China: Japan: Austrialia:

Barium Oxalate:516-02-9:No:No:No:No:No

Chemical Inventory Status Part 2

Ingredient:CASNO:Korea:DSL:NDSL:Phil.:

Barium Oxalate:516-02-9:No:No:No:No

Federal, State, International Regulations- Part 1

Ingredient:CASNO:SARA302:SARA313:RQ:TPQ:LISTChemicalCatg:

Barium Oxalate:516-02-9:No:No:No:No

Federal, State, International Regulations- Part 2

Ingredient:CASNO: RCRA261.33: TSCA	8(d): CERCLA

Barium Oxalate:516-02-9:No:No:No

Section 15: Other Regulatory Information (Continued)

Additional Items

Chemical Weapons Convention:Acute:TSCA12(b):CDTA:SARA311/312:Reactivity:Chronic:

No:No:No:No:No:No:No

Fire Pressure:AustrialianHazChem Code: Poison Schedule: WHMIS:

No:No:No:No:No

Section 16 - Additional Information

References: Not available.

Other Special Considerations:Not available.

Disclaimer:

*****************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.