MATERIAL SAFETY DATA SHEET

BARIUM THIOSULPHATE

MSDS CAS: 35112-53-9

Section 1: Chemical Product andCompany Identification

Section 1: Chemical Product

Product Name:BARIUM THIOSULPHATE

CAS#: 35112-53-9

Synonym:Not available.

Chemical Name:Not available.

Chemical Formula:BaO3S2

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Barium Thiosulphate | 35112-53-9 | >95 |

Section 3: Hazards Identification

Risk advice to man and the environment:Harmful by inhalation and if swallowed.

Section 4: First Aid Measures

General advice:Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled:

If breathed in, move person into fresh air. If not breathing give artificial respiration Consult a physician.

In case of skin contact:Wash off with soap and plenty of water. Consult a physician.

In case of eye contact:

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed:

Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Section 5: Fire and Explosion Data

Suitable extinguishing media:

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special protective equipment for fire-fighters:

Wear self contained breathing apparatus for fire fighting if necessary.

Section 6: Accidental Release Measures

Personal precautions:Use personal protective equipment. Avoid breathing vapors, mist or gas. Ensure adequate ventilation.

Environmental precautions:Do not let product enter drains.

Methods for cleaning up:Pick up and arrange disposal without creating dust. Keep in suitable, closed containers for disposal.

Section 7: Handling and Storage

Handling:Avoid formation of dust and aerosols. Provide appropriate exhaust ventilation at places where dust is formed. Normal measures for preventive fire protection.

Storage:Store in cool place. Keep container tightly closed in a dry and well-ventilated place.

Section 8: Exposure Controls/Personal Protection

Personal protective equipment

Respiratory protection: Where risk assessment shows air-purifying respirators are appropriate use a dust mask type N95 (US) or type P1 (EN 143) respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Hand protection:The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it. Handle with gloves.

Eye protection:Safety glasses.

Skin and body protection:Choose body protection according to the amount and concentration of the dangerous substance at the work place.

Hygiene measures:Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday.

Section 9: Physical and Chemical Properties

Appearance Form	: Solid

pH				: No data available

Molecular Weight	: 246.49 g/mole

Melting point		: No data available

Boiling point		: No data available

Flash point			: No data available

Ignition temperature	: No data available

Lower explosion limit	: No data available

Upper explosion limit	: No data available

Water solubility		: No data available

Section 10: Stability and Reactivity Data

Storage stability:Stable under recommended storage conditions.

Materials to avoid:Strong oxidizing agents

Hazardous decomposition products:

Hazardous decomposition products formed under fire conditions. - Oxides of phosphorus, Barium oxide

Section 11: Toxicological Information

Acute toxicity:No data available

Irritation and corrosion:No data available

Chronic exposure

IARC: No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

Signs and Symptoms of Exposure:To the best of our knowledge, the chemical, physical, and toxicological properties have not been thoroughly investigated.

Potential Health Effects

Inhalation Harmful if inhaled. May cause respiratory tract irritation.

Skin May be harmful if absorbed through skin. Causes skin irritation.

Eyes May cause eye irritation.

Ingestion May be harmful if swallowed.

Target Organs Heart, Nerves., Kidney, Gastrointestinal tract, Bone marrow, Spleen., Liver

Section 12: Ecological Information

Elimination information (persistence and degradability):No data available

Ecotoxicity effects:No data available

Further information on ecology:No data available

Section 13: Disposal Considerations

Product:Observe all federal, state, and local environmental regulations. Contact a licensed professional waste disposal service to dispose of this material. Dissolve or mix the material with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and scrubber.

Contaminated packaging:Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name: BARIUM COMPOUND, N.O.S.(Barium thiosulphate)

UN N°: 1564

H.I. nr: -

ADR - Class: -

Labelling - Transport: 6.1

ADR - Group: III

Sea transport (IMDG) [English only]

Proper shipping name: BARIUM COMPOUND, N.O.S.

UN N°: 1564

IMO-IMDG - Class or division: 6.1

IMO-IMDG - Packing group: III

Air transport (ICAO-IATA) [English only]

Proper shipping name: Barium compound, n.o.s.

UN N°: 1564

IATA - Class or division: 6.1

IATA - Packing group: III

Section 15: Other Regulatory Information

Labelling according to EC Directives

EC Label

Hazard symbols: Xn Harmful

R-phrase(s):R20/22 Harmful by inhalation and if swallowed.

Section 15: Other Regulatory Information (Continued)

S-phrase(s):S28 After contact with skin, wash immediately with plenty of water.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

OXFORD LAB FINE CHEM LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, OXFORD LAB FINE CHEM LLP will not be responsible for damages resulting from use of or reliance upon this information.