MATERIAL SAFETY DATA SHEET

BENZOIC ANHYDRIDE 99% (For Synthesis)

MSDS CAS :-93-97-0

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:BENZOIC ANHYDRIDE 99% (For Synthesis)

CAS#: 93-97-0

C.I. No.:Not available.

Synonym:Not available.

Chemical Name:Not available.

Chemical Formula:(C6H5CO)2O

Molecular Weight: 226.23

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| BENZOIC ANHYDRIDE | 93-97-0 | 100% |

Section 3: Hazards Identification

Potential Acute Health Effects:Hazardous in case of skin contact (irritant), of eye contact (irritant), of ingestion, of inhalation.

Potential Chronic Health Effects:

CARCINOGENIC EFFECTS: Not available.

MUTAGENIC EFFECTS: Not available.

TERATOGENIC EFFECTS: Not available.

DEVELOPMENTAL TOXICITY: Not available. The substance may be toxic to upper respiratory tract, skin, eyes. Repeated orprolonged exposure to the substance can produce target organs damage.

Section 4: First Aid Measures

Eye Contact:Check for and remove any contact lenses. In case of contact, immediately flush eyes with plenty of water for at least 15 minutes. Cold water may be used. WARM water MUST be used. Get medical attention.

Skin Contact:In case of contact, immediately flush skin with plenty of water. Cover the irritated skin with an emollient. Remove contaminated clothing and shoes. Wash clothing before reuse. Thoroughly clean shoes before reuse. Get medical attention.

Serious Skin Contact: Wash with a disinfectant soap and cover the contaminated skin with an anti-bacterial cream. Seek medical attention.

Inhalation: If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medicalattention.

Serious Inhalation:Not available.

Section 4: First Aid Measures(Continued)

Ingestion:Do NOT induce vomiting unless directed to do so by medical personnel. Never give anything by mouth to an unconscious person. If large quantities of this material are swallowed, call a physician immediately. Loosen tight clothing such as a collar,tie, belt or waistband.

Serious Ingestion: Not available.

Section 5: Fire and Explosion Data

Flammability of the Product:May be combustible at high temperature.

Auto-Ignition Temperature:Not available.

Flash Points:Not available.

Flammable Limits:Not available.

Products of Combustion: Not available.

Fire Hazards in Presence of Various Substances:Slightly flammable to flammable in presence of heat.

Explosion Hazards in Presence of Various Substances:Slightly explosive in presence of open flames and sparks. Non-explosive in presence of shocks.

Fire Fighting Media and Instructions:

SMALL FIRE:Use DRY chemical powder.

LARGE FIRE:Use water spray, fog or foam. Do not use water jet.

Special Remarks on Fire Hazards:As with most organic solids, fire is possible at elevated temperatures

Special Remarks on Explosion Hazards:Fine dust dispersed in air in sufficient concentrations, and in the presence of an ignition source is a potential dust explosionhazard.

Section 6: Accidental Release Measures

Small Spill:Use appropriate tools to put the spilled solid in a convenient waste disposal container. Finish cleaning by spreading water onthe contaminated surface and dispose of according to local and regional authority requirements.

Large Spill:Use a shovel to put the material into a convenient waste disposal container. Finish cleaning by spreading water on thecontaminated surface and allow to evacuate through the sanitary system.

Section 7: Handling and Storage

Precautions:Keep away from heat. Keep away from sources of ignition. Ground all equipment containing material. Do not breathe dust.Wear suitable protective clothing. In case of insufficient ventilation, wear suitable respiratory equipment. If you feel unwell,seek medical attention and show the label when possible. Avoid contact with skin and eyes. Keep away from incompatiblessuch as oxidizing agents.

Storage:Keep container tightly closed. Keep container in a cool, well-ventilated area.

Section 8: Exposure Controls/Personal Protection

Engineering Controls:Use process enclosures, local exhaust ventilation, or other engineering controls to keep airborne levels below recommendedexposure limits. If user operations generate dust, fume or mist, use ventilation to keep exposure to airborne contaminantsbelow the exposure limit.

Personal Protection:Splash goggles. Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Gloves.

Personal Protection in Case of a Large Spill:Splash goggles. Full suit. Dust respirator. Boots. Gloves. A self contained breathing apparatus should be used to avoidinhalation of the product. Suggested protective clothing might not be sufficient; consult a specialist BEFORE handling thisproduct.

Exposure Limits:Not available

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties:

Physical state and Appearance		: Solid

Odour						: No data available.

Taste							: No data available.

Molecular Weight				: 226.23 g/mole

Color							:White. Off-white.

pH (1% soln/water)				:Not available.

Boiling Point					:360°C (680°F)

Melting Point					:42°C (107.6°F) - 43 C.

Critical Temperature				:Not available.

Specific Gravity					:1.1989 @ 15 deg. C.(Water = 1)

Vapor Pressure					:Not applicable.

Vapor Density					:7.8 (Air = 1)

Volatility						:Not available.

Odor Threshold					:Not available.

Water/Oil Dist. Coeff.				:Not available.

Ionicity (in Water)				:Not available.

Dispersion Properties				:Not available.

Solubility:Soluble in diethyl ether, acetone. Very slightly soluble in cold water. Solubility in water: 0.01g/liter. Soluble in alcohol,

chloroform, ethyl acetate, benzene, toluene, xylene, glacial acetic acid, acetic anhydride. Moderately soluble in petroleum

ether.

Section 10: Stability and Reactivity Data

Stability:The product is stable.

Instability Temperature:Not available.

Conditions of Instability:Excess heat, incompatible materials, dust generation, moisture, water.

Incompatibility with various substances:Reactive with oxidizing agents. Slightly reactive to reactive with moisture.

Section 10: Stability and Reactivity Data (Continued)

Corrosivity:Non-corrosive in presence of glass.

Special Remarks on Reactivity:Moisture sensitive. May react with water

Special Remarks on Corrosivity:Not available.

Polymerization:Will not occur.

Section 11: Toxicological Information

Routes of Entry:Inhalation. Ingestion.

Toxicity to Animals:

LD50: Not available.

LC50: Not available.

Chronic Effects on Humans:May cause damage to the following organs: upper respiratory tract, skin, eyes.

Other Toxic Effects on Humans:Hazardous in case of skin contact (irritant), of ingestion, of inhalation.

Special Remarks on Toxicity to Animals:Not available.

Special Remarks on Chronic Effects on Humans:Not available.

Special Remarks on other Toxic Effects on Humans:

Acute Potential Health Effects: Skin: Causes skin irritation. Eyes: Causes eye irritation. Inhalation: Causes respiratory tractand mucous membrane irritation. Ingestion: Causes gastrointestinal tract irritation with nausea, vomiting and diarrhea. Thetoxicological properties of this substance have not been fully investigated.

Section 12: Ecological Information

Ecotoxicity: Not available.

BOD5 and COD:Not available.

Products of Biodegradation:Possibly hazardous short term degradation products are not likely. However, long term degradation products may arise.

Toxicity of the Products of Biodegradation: Not available.

Special Remarks on the Products of Biodegradation:Not available.

Section 13: Disposal Considerations

Waste treatment methods: Waste must be disposed of in accordance with federal, state and local environmental control regulations.

Section 14: Transport Information

Land transport (ADR-RID)

General information			: Not regulated.

Sea transport (IMDG) [English only]

General information			: Not regulated.

Air transport (ICAO-IATA) [English only]

General information			: Not regulated.

Section 15: Other Regulatory Information

Federal and State Regulations:TSCA 8(b) inventory:Benzoic Anhydride

Other Regulations:Not available.

Other Classifications:

WHMIS (Canada): CLASS D-2B: Material causing other toxic effects (TOXIC).

DSCL (EEC): R36/37/38- Irritating to eyes, respiratory system and skin. S24/25- Avoid contact with skin and eyes. S28- After contact withskin, wash immediately with plenty of water. S37- Wear suitable gloves. S46- If swallowed, seek medical advice immediatelyand show this container or label.

HMIS (U.S.A.):

Health Hazard:2

Fire Hazard:1

Reactivity:1

Personal Protection:E

National Fire Protection Association (U.S.A.):

Health:2

Flammability:1

Reactivity:1

Specific hazard:

Protective Equipment:Gloves. Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Wear appropriate respiratorwhen ventilation is inadequate. Splash goggles.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.