MATERIAL SAFETY DATA SHEET

BERYLLIUM METAL POWDER AR

MSDS CAS:-7440-41-7

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:BERYLLIUM METAL POWDER AR

CAS#: 7440-41-7

Synonym:Not available.

Chemical Name:Not available.

Chemical Formula:Be

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Beryllium Metal Powder AR | 7440-41-7 | 100 |

Section 3: Hazards Identification

Risk advice to man and the environment:

May cause cancer by inhalation. Also toxic if swallowed. Also very toxic by inhalation. Irritating to eyes, respiratory system and skin. May cause sensitization by skin contact. Also toxic: danger of serious damage to health by prolonged exposure through inhalation.

Section 4: First Aid Measures

General advice:Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled:

If breathed in, move person into fresh air. If not breathing give artificial respirationConsult a physician.

In case of skin contact:Wash off with soap and plenty of water. Consult a physician.

In case of eye contact:

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed:Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Section 5: Fire and Explosion Data

Suitable extinguishing media:

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special protective equipment for fire-fighters:

Wear self contained breathing apparatus for fire fighting if necessary.

Section 6: Accidental Release Measures

Personal precautions:Wear respiratory protection. Avoid dust formation. Avoid breathing dust. Ensure adequate ventilation. Evacuate personnel to safe areas.

Environmental precautions:Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Methods for cleaning up:Pick up and arrange disposal without creating dust. Keep in suitable, closed containers for disposal.

Section 7: Handling and Storage

Handling:Avoid exposure - obtain special instructions before use. Avoid formation of dust and aerosols. Provide appropriate exhaust ventilation at places where dust is formed. Normal measures for preventive fire protection.

Storage:Store in cool place. Keep container tightly closed in a dry and well-ventilated place.

Section 8: Exposure Controls/Personal Protection

Personal protective equipment

Respiratory protection: Where risk assessment shows air-purifying respirators are appropriate use a full-face particle respirator type N99 (US) or type P2 (EN 143) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Hand protection:The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it. Handle with gloves.

Eye protection:Safety glasses.

Skin and body protection:Choose body protection according to the amount and concentration of the dangerous substance at the work place.

Hygiene measures:Avoid contact with skin, eyes and clothing. Wash hands before breaks and immediately after handling the product.

Section 9: Physical and Chemical Properties

Appearance Form		: Foil

Colour				: No data available

pH					: No data available

Molecular Weight		: 9.01g/mole

Melting point			: No data available

Boiling point			: No data available

Flash point				: No data available

Ignition temperature		: No data available

Lower explosion limit		: No data available

Upper explosion limit		: No data available

Density				:No data available

Water solubility			: No data available

Section 10: Stability and Reactivity Data

Chemical stability:Stable under recommended storage conditions.

Materials to avoid:Alkali metals

Hazardous decomposition products

Hazardous decomposition products formed under fire conditions. - Carbon oxides, nitrogen oxides (NOx).

Section 11: Toxicological Information

Acute toxicity:LD50 Intravenous - rat - 0,496 mg/kg

Remarks: Liver:Hepatitis (hepatocellular necrosis), zonal.

LD50 Intratracheal - rat - 51 mg/kg

Irritation and corrosion:No data available

Sensitization:May cause allergic skin reaction.

Chronic exposure

Carcinogenicity - rat –IntratrachealTumorigenic: Neoplastic by RTECS criteria. Lungs,

Thorax, or Respiration: Tumors. Lungs, Thorax, or Respiration: Bronchiogenic carcinoma.

Carcinogenicity - rabbit – IntravenousTumorigenic:Equivocal tumorigenic agent by RTECS criteria. Musculoskeletal:Tumors.IARC: Group 1 - The agent (mixture) is carcinogenic to humans. (BERYLLIUM

Section 11: Toxicological Information (Continued)

FOIL) Genotoxicity in vitro - Human - HeLa cell DNA damage Genotoxicity in vitro - mouse - Ascites tumor DNA damage

Potential health effects

InhalationMay be harmful if inhaled. May cause respiratory tract irritation.

SkinMay be harmful if absorbed through skin. May cause skin irritation.

Eyes May cause eye irritation.

Ingestion May be harmful if swallowed.

Section 12: Ecological Information

Elimination information (persistence and degradability):No data available

Ecotoxicity effects:No data available

Further information on ecology:No data available

Section 13: Disposal Considerations

Product:Observe all federal, state, and local environmental regulations. Contact a licensed professional waste disposal service to dispose of this material. Dissolve or mix the material with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and scrubber.

Contaminated packaging:Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name: BERYLLIUM POWDER

UN N°: 1567

H.I. nr: -

ADR - Class: 6.1 (4.1)

Labelling - Transport: -

ADR - Group: II

Section 14: Transport Information (Continued)

Sea transport (IMDG) [English only]

Proper shipping name: BERYLLIUM POWDER

UN N°: 1567

IMO-IMDG - Class or division: 6.1 (4.1)

IMO-IMDG - Packing group: II

Air transport (ICAO-IATA) [English only]

Proper shipping name: BERYLLIUM POWDER

UN N°: 1567

IATA - Class or division: 6.1 (4.1)

IATA - Packing group: II

Section 15: Other Regulatory Information

Labelling according to EC Directives

Hazard symbols: T+ Very toxic

R-phrase(s):

R49 May cause cancer by inhalation.

R25 Also toxic if swallowed.

R26 Also very toxic by inhalation.

R36/37/38 Irritating to eyes, respiratory system and skin.

R43 May cause sensitization by skin contact.

R48/23 Also toxic: danger of serious damage to health by prolonged exposure through inhalation.

S-phrase(s):

S53 Avoid exposure - obtain special instructions before use.

S45 In case of accident or if you feel unwell, seek medical advice immediately (show the label where possible). Restricted to professional users.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.