MATERIAL SAFETY DATA SHEET

BERYLLIUM OXIDE POWDER 99.9%

MSDS CAS:-1304-56-9

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:BERYLLIUM OXIDE POWDER

CAS#: 1304-56-9

Synonym:Not available.

Chemical Name:Not available.

Chemical Formula:BeO

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Beryllium Oxide Powder | 1304-56-9 | 100 |

Section 3: Hazards Identification

According to European Directive 67/548/EEC as amended. Toxic if swallowed. Toxic: danger of serious damage to health by prolonged exposure through inhalation. May cause cancer by inhalation. Very toxic by inhalation. Irritating to eyes, respiratory system and skin. May cause sensitization by skin contact.

Section 4: First Aid Measures

General advice:Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled:

If breathed in, move person into fresh air. If not breathing give artificial respirationConsult a physician.

In case of skin contact:Wash off with soap and plenty of water. Consult a physician.

In case of eye contact:

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed:Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Section 5: Fire and Explosion Data

Suitable extinguishing media:

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special protective equipment for fire-fighters:

Wear self contained breathing apparatus for fire fighting if necessary.

Section 6: Accidental Release Measures

Personal precautions:Wear respiratory protection. Avoid dust formation. Avoid breathing vapors, mist or gas. Ensure adequate ventilation. Evacuate personnel to safe areas. Avoid breathing dust.

Environmental precautions:Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Section 6: Accidental Release Measures (Continued)

Methods for cleaning up:Pick up and arrange disposal without creating dust. Keep in suitable, closed containers for disposal.

Section 7: Handling and Storage

Precautions for safe handling:

Avoid contact with skin and eyes. Avoid formation of dust and aerosols. Avoid exposure - obtain special instructions before use. Provide appropriate exhaust ventilation at places where dust is formed. Normal measures for preventive fire protection.

Conditions for safe storage

Keep container tightly closed in a dry and well-ventilated place. Store in cool place.

Section 8: Exposure Controls/Personal Protection

Personal protective equipment

Respiratory protection: Where risk assessment shows air-purifying respirators are appropriate use a full-face particle respirator type N99 (US) or type P2 (EN 143) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Hand protection:Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices. Wash and dry hands. The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

Eye protection:Face shield and safety glasses Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

Skin and body protection:Complete suit protecting against chemicals, The type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace.

Hygiene measures:Avoid contact with skin, eyes and clothing. Wash hands before breaks and immediately after handling the product.

Section 9: Physical and Chemical Properties

Appearance Form		: Powder

Colour				: White

pH					: No data available

Molecular Weight		: 9.01g/mole

Melting point			: 2,54 °C

Boiling point			: 3,90 °C

Flash point				: No data available

Ignition temperature		: No data available

Lower explosion limit		: No data available

Upper explosion limit		: No data available

Density				:3,01 g/cm3 at 25 °C

Water solubility			: No data available

Section 10: Stability and Reactivity Data

Chemical stability:Stable under recommended storage conditions.

Conditions to avoid:No data available

Materials to avoid:Alkali metals

Hazardous decomposition products

Hazardous decomposition products formed under fire conditions. - Beryllium oxides

Section 11: Toxicological Information

Acute toxicity:No data available

Skin corrosion/irritation:No data available

Serious eye damage/eye irritation:No data available

Respiratory or skin sensitization:May cause sensitization by skin contact.

Germ cell mutagenicity:No data available

Section 11: Toxicological Information (Continued)

Carcinogenicity:Possible human carcinogen

IARC: 2A- Group 2A: Probably carcinogenic to humans (Beryllium oxide)

Reproductive toxicity:No data available

Specific target organ toxicity - single exposure

May cause respiratory irritation.

Specific target organ toxicity - repeated exposure

Causes damage to organs through prolonged or repeated exposure.

Aspiration hazard:No data available

Potential health effects

Inhalation May be fatal if inhaled. Causes respiratory tract irritation.

Ingestion Toxic if swallowed.

Skin Toxic if absorbed through skin. Causes skin irritation.

Eyes Causes serious eye irritation.

Signs and Symptoms of Exposure:

To the best of our knowledge, the chemical, physical, and toxicological properties have not been thoroughly investigated.

Section 12: Ecological Information

Toxicity:No data available

Persistence and degradability:No data available

Bioaccumulative potential:No data available

Mobility in soil:No data available

PBT and vPvB assessment:No data available

Other adverse effects:No data available

Section 13: Disposal Considerations

Product:Contact a licensed professional waste disposal service to dispose of this material. Dissolve or mix the material with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and scrubber. Offer surplus and non-recyclable solutions to a licensed disposal company.

Contaminated packaging:Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name: BERYLLIUM COMPOUND, N.O.S. (Beryllium oxide)

UN N°: 1566

H.I. nr: -

ADR - Class: 6.1

Labelling - Transport: -

ADR - Group: II

Sea transport (IMDG) [English only]

Proper shipping name: BERYLLIUM COMPOUND, N.O.S. (Beryllium oxide)

UN N°: 1566

IMO-IMDG - Class or division: 6.1

IMO-IMDG - Packing group: II

Air transport (ICAO-IATA) [English only]

Proper shipping name: BERYLLIUM COMPOUND, N.O.S. (Beryllium oxide)

UN N°: 1566

IATA - Class or division: 6.1

IATA - Packing group: II

Section 15: Other Regulatory Information

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.