MATERIAL SAFETY DATA SHEET

BETA CAROTENE 20%

Extra Pure

MSDS CAS: 7235-40-7

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:BETA CAROTENE 20%

CAS#: 7235-40-7

Synonym:Not available.

Chemical Name:beta-Carotene

Chemical Formula:BeONot available.

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Carotenoids activity | - | 2.5 |

Toxicological Data on Ingredients:Carotenoids activity LD50: Not available. LC50: Not available.

Section 3: Hazards Identification

Potential Acute Health Effects:

Slightly hazardous in case of skin contact (irritant), of eye contact (irritant), of ingestion, of inhalation.

Potential Chronic Health Effects:

CARCINOGENIC EFFECTS: Not available. MUTAGENIC EFFECTS: Not available. TERATOGENIC EFFECTS: Not available. DEVELOPMENTAL TOXICITY: Not available. Repeated or prolonged exposure is not known to aggravate medical condition.

Section 4: First Aid Measures

Eye Contact:Check for and remove any contact lenses. In case of contact, immediately flush eyes with plenty of water for at least 15 minutes. Get medical attention if irritation occurs.

Skin Contact:Wash with soap and water. Cover the irritated skin with an emollient. Get medical attention if irritation develops.

Serious Skin Contact:Not available.

Inhalation:If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical attention.

Serious Inhalation:Not available.

Ingestion:Do NOT induce vomiting unless directed to do so by medical personnel. Never give anything by mouth to an unconscious person. If large quantities of this material are swallowed, call a physician immediately. Loosen tight clothing such as a collar, tie, belt or waistband.

Serious Ingestion:Not available.

Section 5: Fire and Explosion Data

Flammability of the Product:May be combustible at high temperature.

Auto-Ignition Temperature:Not available.

Flash Points:Not available.

Section 5: Fire and Explosion Data (Continued)

Flammable Limits:Not available.

Products of Combustion:Not available.

Fire Hazards in Presence of Various Substances:Slightly flammable to flammable in presence of heat. Non-flammable in presence of shocks.

Explosion Hazards in Presence of Various Substances:Slightly explosive in presence of open flames and sparks. Non-explosive in presence of shocks.

Fire Fighting Media and Instructions:SMALL FIRE: Use DRY chemical powder. LARGE FIRE: Use water spray, fog or foam. Do not use water jet.

Special Remarks on Fire Hazards:As with most organic solids, fire is possible at elevated temperatures

Special Remarks on Explosion Hazards:Fine dust dispersed in air in sufficient concentrations, and in the presences of an ignition source is a potential dust explosion hazard.

Section 6: Accidental Release Measures

Small Spill:Use appropriate tools to put the spilled solid in a convenient waste disposal container. Finish cleaning by spreading water on the contaminated surface and dispose of according to local and regional authority requirements.

Large Spill:Use a shovel to put the material into a convenient waste disposal container. Finish cleaning by spreading water on the contaminated surface and allow to evacuate through the sanitary system.

Section 7: Handling and Storage

Precautions:Keep away from heat. Keep away from sources of ignition. Ground all equipment containing material. Do not breathe dust.

Storage:Keep container tightly closed. Keep container in a cool, well-ventilated area. Do not store above 30°C (86°F). store at temperatures between 15 and 30 deg. C. Air Sensitive. Sensitive to light. Store in light-resistant containers. Moisture sensitive.

Section 8: Exposure Controls/Personal Protection

Engineering Controls:Use process enclosures, local exhaust ventilation, or other engineering controls to keep airborne levels below recommended exposure limits. If user operations generate dust, fume or mist, use ventilation to keep exposure to airborne contaminants below the exposure limit.

Personal Protection:Safety glasses. Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Gloves.

Personal Protection in Case of a Large Spill:Splash goggles. Full suit. Dust respirator. Boots. Gloves. A self contained breathing apparatus should be used to avoid inhalation of the product. Suggested protective clothing might not be sufficient; consult a specialist BEFORE handling this product.

Exposure Limits:Not available.

Section 9: Physical and Chemical Properties

Physical state and appearance:Solid. (Powdered solid.)

Odor					:Characteristic.

Taste					:Not available.

Molecular Weight		:Not applicable.

Color					:Orange.

pH (1% soln/water)		:Not applicable.

Boiling Point			:Not available.

Melting Point			:Not available.

Critical Temperature		:Not available.

Specific Gravity			:Not available.

Vapor Pressure			:Not applicable.

Vapor Density			:Not available.

Volatility				:Not available.

Odor Threshold			:Not available.

Water/Oil Dist. Coeff.		:Not available.

Ionicity (in Water)		:Not available.

Dispersion Properties		:See solubility in water, diethyl ether, acetone.

Solubility:Solubility Information for beta-Carotene: Soluble in diethyl ether, acetone. Very slightly soluble in methanol. Insoluble in coldwater, hot water. Soluble in benzene, chloroform, carbon disulfide. Moderately soluble in petroleium ether, oils. Soluble in fatsolvents

Section 10: Stability and Reactivity Data

Stability:The product is stable.

Instability Temperature:Not available.

Conditions of Instability:Excess heat, light, exposure to air and moisture/water, excess dust generation

Incompatibility with various substances:Not available.

Corrosivity:Not available.

Special Remarks on Reactivity:Beta-carotene absorbs oxygen from air giving rise to inactive, colorless oxidation products. Air and light sensitive. Moisture sensitive. Protect from light and moisture

Special Remarks on Corrosivity:Not available.

Polymerization:Will not occur.

Section 11: Toxicological Information

Routes of Entry:Inhalation. Ingestion.

Toxicity to Animals:LD50: Not available. LC50: Not available.

Chronic Effects on Humans:Not available.

Other Toxic Effects on Humans:

Slightly hazardous in case of skin contact (irritant), of ingestion, of inhalation.

Special Remarks on Toxicity to Animals:Not available.

Special Remarks on Chronic Effects on Humans:Not available.

Special Remarks on other Toxic Effects on Humans:

Acute Potential Health Effects: Skin: Not expected to be irritating to skin. Eyes: May cause eye irritation by mechanical action.Ingestion: Low hazard for usual industrial handling. When carotene-containing foods are eaten in large quantities, beta-carotene and other carotenoids can be absorbed and transferred to various

Section 11: Toxicological Information (Continued)

tissues in large enough quantities to impart anorange or yellow color to serum ("hypercarotenemia") and skin. Because beta-carotene has the potential to be convertedinto vitamin A, it is theoretically possible that

high levels of carotene intake could produce elevated levels of Vitamin A.However, high carotene intake is not associated with Vitamin A toxicity. Inhalation: Inhalation of large amounts may causemild respiratory tract irritation. Respiratory tract irritation is a low hazard for usual industrial handling. Chronic Potential Healtheffects: no information found.

Section 12: Ecological Information

Ecotoxicity:Not available.

BOD5 and COD:Not available.

Products of Biodegradation:Possibly hazardous short term degradation products are not likely. However, long term degradation products may arise.

Toxicity of the Products of Biodegradation:Not available.

Special Remarks on the Products of Biodegradation:Not available.

Section 13: Disposal Considerations

Waste Disposal:

Waste must be disposed of in accordance with federal, state and local environmental control regulations.

Section 14: Transport Information

Land transport (ADR-RID)

General information: Not regulated.

Sea transport (IMDG) [English only]

General information: Not regulated.

Section 14: Transport Information (Continued)

Air transport (ICAO-IATA) [English only]

General information: Not regulated.

Section 15: Other Regulatory Information

Federal and State Regulations:No products were found.

Other Regulations:Not available. or of its ingredients

Other Classifications:

WHMIS (Canada): Not controlled under WHMIS (Canada).

DSCL (EEC): This product is not classified according to the EU regulations. S24/25- Avoid contact with skin and eyes.

HMIS (U.S.A.):

Health Hazard: 1

Fire Hazard: 1

Reactivity: 0

Personal Protection: E

National Fire Protection Association (U.S.A.):

Health: 0

Flammability: 1

Reactivity: 0

Specific hazard:

Protective Equipment:Gloves. Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Safety glasses.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.