MATERIAL SAFETY DATA SHEET

BISMUTH BROMIDE 98%

MSDS CAS: 7787-58-8

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:BISMUTH BROMIDE

CAS#:7787-58-8

Synonym:Not Available.

Chemical Name:Bismuth Bromide

Chemical Formula:BiBr3

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Bismuth Bromide | 7787-58-8 | <= 100 % |

Section 3: Hazards Identification

Risk advice to man and the environment:Causes burns.

Section 4: First Aid Measures

General advice:Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled:

If breathed in, move person into fresh air. If not breathing give artificial respiration Consult a physician.

In case of skin contact:Take off contaminated clothing and shoes immediately. Wash off with soap and plenty of water. Consult a physician.

In case of eye contact:

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed:Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Section 5: Fire and Explosion Data

Suitable extinguishing media:

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special protective equipment for fire-fighters:

Wear self contained breathing apparatus for fire fighting if necessary.

Section 6: Accidental Release Measures

Personal precautions:Use personal protective equipment. Avoid dust formation. Avoid breathing dust. Ensure adequate ventilation. Evacuate personnel to safe areas.

Environmental precautions:Do not let product enter drains.

Methods for cleaning up:

Pick up and arrange disposal without creating dust. Keep in suitable, closed containers for disposal.

Section 7: Handling and Storage

Handling:Avoid formation of dust and aerosols. Provide appropriate exhaust ventilation at places where dust is formed. Normal measures for preventive fire protection.

Storage:

Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Moisture sensitive.

Section 8: Exposure Controls/Personal Protection

Personal protective equipment

Respiratory protection:Where risk assessment shows air-purifying respirators are appropriate use a full-face particle respirator type N100 (US) or type P3 (EN 143) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Hand protection:The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it. Handle with gloves.

Eye protection:Safety glasses

Skin and body protection:Choose body protection according to the amount and concentration of the dangerous substance at the work place.

Hygiene measures:Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday.

Section 9: Physical and Chemical Properties

Appearance Form		: Powder

pH					: No data available

Molecular Weight		: 448.69 g/mole

Melting point			: No data available

Boiling point			: No data available

Flash point				: Not applicable

Ignition temperature		: No data available

Lower explosion limit		: No data available

Section 9: Physical and Chemical Properties (Continued)

Upper explosion limit		: No data available

Density				: 5,7 g/ml at 25 °C

Water solubility			: No data available

Section 10: Stability and Reactivity Data

Storage stability:Stable under recommended storage conditions.

Materials to avoid:Strong oxidizing agents

Hazardous decomposition products:

Hazardous decomposition products formed under fire conditions. - Carbon oxides, Hydrogen bromide gas

Section 11: Toxicological Information

Acute toxicity:No data available

Irritation and corrosion:No data available

Sensitisation:No data available

Chronic exposure

IARC: No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

Signs and Symptoms of Exposure

Symptoms of chronic bismuth toxicity in humans consists of decreased appetite, weakness, rheumatic pain, diarrhea, fever, metal line on the gums, foul breathe, gingivitis, and dermatitis. Jaundice and conjunctival hemorrhage are rare, but have been reported. Bismuth nephropathy with proteinuria may occur. The kidney is the site of highest concentration with the liver being considerably lower. Bismuth does pass into the amniotic fluid and into the fetus., Material is extremely destructive to tissue of the mucous membranes and upper respiratory tract, eyes, and skin., Cough, Shortness of breath, Headache

Section 11: Toxicological Information (Continued)

Potential Health Effects

Inhalation May be harmful if inhaled. Material is extremely destructive to the tissue of the mucous membranes and upper respiratory tract.

Skin May be harmful if absorbed through skin. Causes skin burns.

Eyes Causes eye irritation.

Ingestion Harmful if swallowed.

Section 12: Ecological Information

Elimination information (persistence and degradability):No data available

Ecotoxicity effects:No data available

Further information on ecology:No data available

Section 13: Disposal Considerations

Product:Observe all federal, state, and local environmental regulations. Contact a licensed professional waste disposal service to dispose of this material. Dissolve or mix the material with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and scrubber.

Contaminated packaging:Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name: CORROSIVE SOLID, ACIDIC, INORGANIC, N.O.S. (Bismuth tribromide)

UN N°: 3260

H.I. nr: -

ADR - Class: 8

Labelling - Transport: -

ADR - Group: II

Section 14: Transport Information (Continued)

Sea transport (IMDG) [English only]

Proper shipping name: CORROSIVE SOLID, ACIDIC, INORGANIC, N.O.S. (Bismuth tribromide)

UN N°: 3260

IMO-IMDG - Class or division: 8

IMO-IMDG - Packing group: II

Air transport (ICAO-IATA) [English only]

Proper shipping name: CORROSIVE SOLID, ACIDIC, INORGANIC, N.O.S. (Bismuth tribromide)

UN N°: 3260

IATA - Class or division: 8

IATA - Packing group: II

Section 15: Other Regulatory Information

Not Regulated

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.