MATERIAL SAFETY DATA SHEET

N,O-Bis-(TRIMETHYLSILYL)TRIFLUORO ACETAMIDE

MSDS CAS: -25561-30-2

Section 1: Chemical Product andCompany Identification

Section 1: Chemical Product

Product Name:N,O-Bis-(TRIMETHYLSILYL)TRIFLUORO ACETAMIDE

CAS#: -25561-30-2

C.I. No.:Not available.

Synonym:Ethanimidic acid, 2,2,2-trifluoro-N- (trimethylsilyl)-, trimethylsilyl ester; Trimethylsilyl2,2,2- trifluoro-N-(trimethylsilyl)acetimidate; BSTFA

Chemical Name:Bis(trimethylsilyl)trifluoroacetamide)

Chemical Formula:C8-H18-F3-N-O-Si2

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification: OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| N,O-Bis-(TRIMETHYLSILYL)TRIFLUORO ACETAMIDE | 25561-30-2 | 98 |

Section 3: Hazards Identification

Potential Acute Health Effects:

Hazardous in case of skin contact (irritant), of eye contact (irritant), of ingestion, of inhalation. Slightly hazardous in case of skin contact (corrosive, permeator), of eye contact (corrosive). Liquid or spray mist may produce tissue damage particularly on mucous membranes of eyes, mouth and respiratory tract. Skin contact may produce burns. Inhalation of the spray mist may produce severe irritation of respiratory tract, characterized by coughing, choking, or shortness of breath.

Potential Chronic Health Effects:

CARCINOGENIC EFFECTS: Not available. MUTAGENIC EFFECTS: Not available. TERATOGENIC EFFECTS: Not available. DEVELOPMENTAL TOXICITY: Not available. Repeated or prolonged contact with spray mist may produce chronic eye irritation and severe skin irritation. Repeated or prolonged exposure to spray mist may produce respiratory tract irritation leading to frequent attacks of bronchial infection.

Section 4: First Aid Measures

Eye Contact:

Check for and remove any contact lenses. In case of contact, immediately flush eyes with plenty of water for at least 15 minutes. Get medical attention immediately.

Skin Contact:

In case of contact, immediately flush skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Cover the irritated skin with an emollient. Wash clothing before reuse. Thoroughly clean shoes before reuse. Get medical attention immediately.

Serious Skin Contact:

Wash with a disinfectant soap and cover the contaminated skin with an anti-bacterial cream. Seek medical attention.

Inhalation:

If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical attention.

Serious Inhalation:

Evacuate the victim to a safe area as soon as possible. Loosen tight clothing such as a collar, tie, belt or waistband. WARNING: It may be hazardous to the person providing aid to give mouth-to-mouth resuscitation when the inhaled material is toxic, infectious or corrosive. Seek immediate medical attention.

Ingestion:

Do NOT induce vomiting unless directed to do so by medical personnel. Never give anything by mouth to an unconscious person. If large quantities of this material are swallowed, call a physician immediately. Loosen tight clothing such as a collar, tie, belt or waistband.

Serious Ingestion: Not available.

Section 5: Fire and Explosion Data

Flammability of the Product: Flammable.

Auto-Ignition Temperature: >300°C (572°F)

Flash Points: CLOSED CUP: 23°C (73.4°F).

Flammable Limits: Not available.

Products of Combustion:

These products are carbon oxides (CO, CO2), nitrogen oxides (NO, NO2...), halogenated compounds. Some metallic oxides.

Fire Hazards in Presence of Various Substances:

Highly flammable in presence of open flames and sparks, of heat. Non-flammable in presence of shocks.

Explosion Hazards in Presence of Various Substances:

Risks of explosion of the product in presence of mechanical impact: Not available. Risks of explosion of the product in presence of static discharge: Not available.

Fire Fighting Media and Instructions:

Flammable liquid. SMALL FIRE: Use DRY chemical powder. LARGE FIRE: Use alcohol foam, water spray or fog. Cool containing vessels with water jet in order to prevent pressure build-up, autoignition or explosion.

Special Remarks on Fire Hazards: Not available.

Special Remarks on Explosion Hazards: Not available.

Section 6: Accidental Release Measures

Small Spill: Absorb with an inert material and put the spilled material in an appropriate waste disposal.

Large Spill:

Flammable liquid. Corrosive liquid. Keep away from heat. Keep away from sources of ignition. Stop leak if without risk. Absorb with DRY earth, sand or other non-combustible material. Do not get water inside container. Do not touch spilled material. Use water spray curtain to divert vapor drift. Prevent entry into sewers, basements or confined areas; dike if needed. Call for assistance on disposal.

Section 7: Handling and Storage

Precautions:

Keep container dry. Keep away from heat. Keep away from sources of ignition. Ground all equipment containing material. Do not breathe gas/fumes/ vapor/spray. Never add water to this product. In case of insufficient ventilation, wear suitable respiratory equipment. If you feel unwell, seek medical attention and show the label when possible. Avoid contact with skin and eyes. Keep away from incompatibles such as oxidizing agents, acids.

Section 7: Handling and Storage (Continued)

Storage:

Store in a segregated and approved area. Keep container in a cool, well-ventilated area. Keep container tightly closed and sealed until ready for use. Avoid all possible sources of ignition (spark or flame). Do not store above 5°C (41°F). Moisture sensitive.

Section 8: Exposure Controls/Personal Protection

Engineering Controls:

Provide exhaust ventilation or other engineering controls to keep the airborne concentrations of vapors below their respective threshold limit value. Ensure that eyewash stations and safety showers are proximal to the work-station location.

Personal Protection:

Face shield. Full suit. Vapor respirator. Be sure to use an approved/certified respirator or equivalent. Gloves. Boots.

Personal Protection in Case of a Large Spill:

Splash goggles. Full suit. Vapor respirator. Boots. Gloves. A self contained breathing apparatus should be used to avoid inhalation of the product. Suggested protective clothing might not be sufficient; consult a specialist BEFORE handling this product.

Exposure Limits:Not available.

Section 9: Physical and Chemical Properties

Physical state and appearance		: Liquid.

Odor							: Amine like. (Slight.)

Taste							: Not available.

Molecular Weight				: 257.4 g/mole

Color							: Clear Colorless.

pH (1% soln/water)				: Not available.

Boiling Point					: 47.5°C (117.5°F)

Melting Point					: -10°C (14°F)

Critical Temperature				: Not available.

Specific Gravity					: 0.969 (Water = 1)

Vapor Pressure					: Not available.

Vapor Density					: Not available.

Volatility						: Not available.

Section 9: Physical and Chemical Properties (Continued)

Odor Threshold					: Not available.

Water/Oil Dist. Coeff.				: Not available.

Ionicity (in Water)				: Not available.

Dispersion Properties				: Not available.

Solubility						: Not available.

Section 10: Stability and Reactivity Data

Stability: The product is stable.

Instability Temperature: Not available.

Conditions of Instability: Heat, ignition sources (sparks, flames), moisture, incompatible materials

Incompatibility with various substances:

Reactive with oxidizing agents, acids. Slightly reactive to reactive with moisture.

Corrosivity: Not available.

Special Remarks on Reactivity: Moisture sensitive

Special Remarks on Corrosivity: Not available.

Polymerization: Will not occur.

Section 11: Toxicological Information

Routes of Entry: Absorbed through skin. Eye contact. Inhalation.

Toxicity to Animals:

LD50: Not available. LC50: Not available.

Chronic Effects on Humans: Not available.

Other Toxic Effects on Humans:

Hazardous in case of skin contact (irritant), of ingestion, of inhalation. Slightly hazardous in case of skin contact (corrosive, permeator), of eye contact (corrosive).

Special Remarks on Toxicity to Animals: Not available.

Special Remarks on Chronic Effects on Humans: Not available.

Section 11: Toxicological Information (Continued)

Special Remarks on other Toxic Effects on Humans:

Acute Potential Health Effects: Skin: Causes skin irritation with possible burns. Eyes: Causes eye irritation with possible burns. Inhalation: Causes respiratory tract and mucous membrane irritation with possible burns. Inhalation may result in spasm, inflammatin and edema of the larynx and bronchi, chemical pneumonitis, and pulmonary edema. Symptoms may includeburning sensation, coughing, wheezing, laryngitis, shortness of breath, headache, nausea and vomiting. Ingestion: Maycause gastrointestinal tract irritation and possible burns. The toxicological properties of this substance have not been fully

investigated.

Section 12: Ecological Information

Ecotoxicity: Not available.

BOD5 and COD: Not available.

Products of Biodegradation:

Possibly hazardous short term degradation products are not likely. However, long term degradation products may arise.

Toxicity of the Products of Biodegradation: The products of degradation are more toxic than the product itself.

Special Remarks on the Products of Biodegradation: Not available.

Section 13: Disposal Considerations

Waste Disposal:

Waste must be disposed of in accordance with federal, state and local environmental control regulations.

Section 14: Transport Information

14.1. Land transport (ADR-RID)

Proper shipping name 			: FLAMMABLE LIQUID, CORROSIVE, N.O.S.

UN N°					: 2924

H.I. nr					: 38

ADR - Class 				: 3

Labelling - Transport 			: 3 : Flammable liquid. 8 : Corrosive substance.

Section 14: Transport Information (Continued)

ADR - Group 				: III

14.2. Sea transport (IMDG) [English only]

Proper shipping name			: FLAMMABLE LIQUID, CORROSIVE, N.O.S.

UN N°					: 2924

IMO-IMDG - Class or division	: 3 : Flammable liquid. ( 8 : Corrosive substance. )

IMO-IMDG - Packing group 		: III

14.3. Air transport (ICAO-IATA) [English only]

Proper shipping name 			: FLAMMABLE LIQUID, CORROSIVE, N.O.S.

UN N°					: 2924

IATA - Class or division 		: 3 : Flammable liquid. ( 8 : Corrosive substance. )

IATA - Packing group		: III

Section 15: Other Regulatory Information

Federal and State Regulations: No products were found.

Other Regulations:

OSHA: Hazardous by definition of Hazard Communication Standard (29 CFR 1910.1200). EINECS: This product is on the European Inventory of Existing Commercial Chemical Substances.

Other Classifications:

WHMIS (Canada): Not controlled under WHMIS (Canada).

DSCL (EEC):

R10- Flammable. R36/37/38- Irritating to eyes, respiratory system and skin. S2- Keep out of the reach of children. S16- Keep away from sources of ignition - No smoking. S24/25- Avoid contact with skin and eyes. S26- In case of contact with eyes, rinse immediately with plenty of water and seek medical advice. S39- Wear eye/face protection. S46- If swallowed, seek medical advice immediately and show this container or label.

HMIS (U.S.A.):

Health Hazard: 2

Fire Hazard: 3

Reactivity: 1

Personal Protection:

National Fire Protection Association (U.S.A.):

Health: 2

Section 15: Other Regulatory Information (Continued)

Flammability:3

Reactivity:1

Specific hazard:

Protective Equipment:

Gloves. Full suit. Vapor respirator. Be sure to use an approved/certified respirator or equivalent. Face shield.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.