MATERIAL SAFETY DATA SHEET

BOUIN`S FLUID (Histopathology Fixative)

MSDS CAS:

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:BOUIN`S FLUID

CAS#:

C.I. No.:Not available.

Synonym:Not available.

Chemical Name:Not available.

Chemical Formula:Not available.

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| 2,4,6-trinitrophenol; picric acid | 88-89-1 | > 0.1 < 1 % |
| Formaldehyde | 50-00-0 | > 15 < 25 % |
| Acetic acid | 64-19-7 | > 1 < 5 % |
| Water | 7732-18-5 | > 50 < 75 % |

Section 3: Hazards Identification

Classification of the substance or mixture:

Classification EC 67/548 or EC 1999/45:

Classification: Carc. Cat. 3; R40

Xn; R20/21/22

C; R34

Xi; R37

R43

Hazard Class and Category Code(s), Regulation (EC) No 1272/2008 (CLP):

Health hazards: Carcinogenicity - Category 2 - Warning  (CLP : Carc. 2) H351

Specific Target Organ Toxicity - Single exposure - Respiratory tract irritation -

Category 3 - Warning  (CLP : STOT SE 3) H335

Acute toxicity, Oral - Category 4 - Warning  (CLP : Acute Tox. 4) H302

Skin sensitisation - Category 1 - Warning  (CLP : Skin Sens. 1) H317

Acute toxicity, Inhalation - Category 4 - Warning  (CLP : Acute Tox. 4) H332

Skin corrosion - Category 1B - Danger  (CLP : Skin Corr. 1B) H314

Acute toxicity, dermal - Category 4 - Warning  (CLP : Acute Tox. 4) H312

Other hazards:  The substance does not fulfil the criteria to be identified as PBT substance or vPvB

substance according to Annex XIII of Regulation REACH.

Section 4: First Aid Measures

Eye Contact:Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Immediately call a POISON CENTER or doctor.

Skin Contact:Get medical advice. If skin irritation or rash occurs :Specific treatment (see on this label). Wash contaminated clothing before reuse. Immediately call a POISON CENTER or doctor. Wash with plenty of soap and water. Remove/Take off immediately all contaminated clothing.

Serious Skin Contact:Not available.

Inhalation:Immediately call a POISON CENTER or doctor. Remove to fresh air and keep at rest in a position comfortable for breathing. Call a POISON CENTER or doctor ifyou feel unwell.Serious Inhalation:Not available.

Ingestion:Do NOT induce vomiting. Rinse mouth. Immediately call a POISON CENTER or doctor. Call a POISON CENTER or doctor if you feel unwell.

Serious Ingestion:Not available.

Most important symptoms and effects, both acute and delayed:

Symptoms relating to use: Suspected of causing cancer. Causes severe skin burns and eye damage.

Section 4: First Aid Measures (Continued)

Repeated exposure to this material can result in absorption through skin causingsignificant health hazard.

May cause an allergic skin reaction. May cause respiratory irritation. Danger of serious damage to health by prolonged exposure through inhalation. Swallowing a small quantity of this material will result in serious health hazard.

General information: Never give anything by mouth to an unconscious person. If you feel unwell, seek

medical advice (show the label where possible).

Section 5: Fire and Explosion Data

Extinguishing media:

Suitable extinguishing media: Foam. Dry powder. Carbon dioxide. Water spray. Sand.

Unsuitable extinguishing media: Do not use a heavy water stream.

Surrounding fires: Use water spray or fog for cooling exposed containers.

Special hazards arising from the substance or mixture

Hazardous combustion products:  Under fire conditions, hazardous fumes will be present.

Thermal decomposition generates :Corrosive vapours.

Advice for fire-fighters:

Protection against fire: Do not enter fire area without proper protective equipment, including respiratory protection.

Special procedures: Exercise caution when fighting any chemical fire. Avoid (reject) fire-fighting water to enter environment.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

For emergency responders: Equip cleanup crew with proper protection.

Ventilate area.

For non-emergency personnel: Evacuate unnecessary personnel.

Environmental precautions:

Environmental precautions: Prevent entry to sewers and public waters. Notify authorities if product enterssewers or public waters.

Methods and material for containment and cleaning up:

Clean up methods: Soak up spills with inert solids, such as clay or diatomaceous earth as soon as

possible. Collect spillage. Store away from other materials.

Section 7: Handling and Storage

Precautions:Contaminated work clothing should not be allowed out of the workplace. Use only outdoors or in a well-ventilated area. Do not breathe dust, fume, gas, mist, vapours, spray. Wash thoroughly after handling. Wash contaminated clothing before reuse. Obtain special instructions before use. Do not eat, drink or smoke when using this product. Avoid breathing dust, fume, gas, mist, vapours, spray.

Technical protective measures: Provide good ventilation in process area to prevent formation of vapour.

Special precautions: Avoid breathing dust, fume, gas, mist, vapours, spray.

Storage:Keep only in the original container in a cool, well ventilated place. Keep container closed when not in use.

Storage regulation :Comply with applicable regulations.

Storage - away from:Strong bases. Strong acids. Sources of ignition. Direct sunlight.

Section 8: Exposure Controls/Personal Protection

Personal protection:  Avoid all unnecessary exposure.

• Respiratory protection:  In case of insufficient ventilation, wear suitable respiratory equipment.

• Hand protection:  Wear protective gloves.

• Eye protection:  Chemical goggles or safety glasses.

• Others:  When using, do not eat, drink or smoke.

Occupational Exposure Limits: No data available

Section 9: Physical and Chemical Properties

Physical state and appearance: Liquid.

Odor					:Characteristic.

Taste					: Not available.

Molecular Weight		: Not applicable.

Color					: Clear Yellow.

pH (1% soln/water)		: Not applicable.

Boiling Point			: Not available.

Melting Point			: Not available.

Critical Temperature		: Not available.

Specific Gravity			: Not available.

Vapor Pressure			: Not applicable.

Section 9: Physical and Chemical Properties (Continued)

Vapor Density			: Not available.

Volatility				: Not available.

Odor Threshold			: Not available.

Water/Oil Dist. Coeff.		: Not available.

Ionicity (in Water)		: Not available.

Dispersion Properties		: Not available.

Solubility: Not available.

Section 10: Stability and Reactivity Data

Stability:The product is stable.

Instability Temperature:Not available.

Conditions to avoid:  Direct sunlight. Extremely high or low temperatures.

Materials to avoid:  Strong acids. Strong bases.

Hazardous decomposition products:  Fumes. Carbon monoxide. Carbon dioxide.

Thermal decomposition generates : Corrosive vapours.

Section 11: Toxicological Information

Information on toxicological effects:

Toxicity information : The product has been not fully tested. The calculated risk has been done under the

requirements of the EU regulations.

Acute toxicity:

• Inhalation: Harmful if inhaled.

• Dermal: Harmful in contact with skin.

• Ingestion: Harmful if swallowed.

Corrosion: Causes severe skin burns and eye damage.

Irritation: May cause respiratory irritation.

Sensitization: May cause an allergic skin reaction.

Mutagenicity: Based on available data, the classification criteria are not met.

Carcinogenicity: Suspected of causing cancer.

Toxic for reproduction: Based on available data, the classification criteria are not met.

STOT-single exposure: Based on available data, the classification criteria are not met.

STOT-repeated exposure: Based on available data, the classification criteria are not met.

Aspiration hazard: Based on available data, the classification criteria are not met.

Section 12: Ecological Information

Ecotoxicity: The product has been not fully tested. The calculated risk has been done under the requirements of the EU regulations.

BOD5 and COD: Not available.

Persistence – degradability : Biodegradable.

Bioaccumulativepotential : Not established.

Mobility in soil :Not established.

Results of PBT and vPvBassessment : The substance does not fulfil the criteria to be identified as PBT substance or vPvB substance according to Annex XIII of Regulation REACH.

Environmental precautions :Avoid release to the environment.

Special Remarks on the Products of Biodegradation: Not available.

Section 13: Disposal Considerations

Waste Disposal:

Avoid release to the environment. Dispose in a safe manner in accordance with

local/national regulations.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name: CORROSIVE LIQUID, ACIDIC, ORGANIC, N.O.S

UN N°: 3265

ADR – Class: 8

Labelling – Transport: 8: CORROSIVE LIQUID.

ADR – Group: III

Sea transport (IMDG) [English only]

Proper shipping name: CORROSIVE LIQUID, ACIDIC, ORGANIC, N.O.S

UN N°: 3265

IMO-IMDG - Class or division:8: CORROSIVE LIQUID.

IMO-IMDG - Packing group: III

Air transport (ICAO-IATA) [English only]

Proper shipping name: CORROSIVE LIQUID, ACIDIC, ORGANIC, N.O.S

UN N°: 3265

IATA - Class or division: 8: CORROSIVE LIQUID.

IATA - Packing group: III

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture:

Safety, health and environmental               : Ensure all national/local regulations are observed.

regulations/legislation specific for the

substance or mixture

REACH Restrictions - Annex XVII: The components of this product are not subject to restrictions.

REACH Authorisation - Annex XIV: The components of this product are not subject to authorization.

Chemical Safety Assessment                     :Chemical Safety Assessment: It has not been carried out.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.