MATERIAL SAFETY DATA SHEET

BRADY'S REAGENT

MSDS CAS:

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:BRADY'S REAGENT

CAS#:

C.I. No.:Not available.

Synonym:Not available.

Chemical Name:Not available.

Chemical Formula:Not available.

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| METHANOL EXTRA PURE | 67-56-1 | 70 - 75 |
| 2,4-DINITROPHENYL HYDRAZINE 98% | 119-26-6 | > 15 < 25 % |
| SULPHURIC ACID 98% EXTRA PURE | 7664-93-9 | 5 -8 |

Section 3: Hazards Identification

Classification of the substance or mixture:

Classification according to Regulation (EC)No. 1272/2008 [CLP]:

Classification:

Flammable liquids,  H225

Category 2

Acute toxicity(oral), H301

Category 3

Acute Toxicity(dermal), H311

Category 3

H311

Acute Toxicity(inhalation:gas)       H331

Category 3

Skin corrosion/irritation, H315

Category 2

Serious eye damage/eye irritation, H319

Category 2

Specific target organ toxicity — singleexposure,  H370

Category 1

Section 4: First Aid Measures

Eye Contact:Rinse cautiously with water for several minutes. Remove contact lenses, if   present and easy to do. Continue rinsing. Get medical advice/attention.

Skin Contact:Get medical advice. Wash contaminated clothing before reuse. Immediately call a POISON CENTER or doctor. Wash with plenty of soap and water. Remove/Take off immediately all contaminated clothing.

Serious Skin Contact:Not available.

Inhalation:Immediately call a POISON CENTER or doctor. Remove to fresh air and keep at rest in a position comfortable for breathing. Call a POISON CENTER or doctor ifyou feel unwell.Serious Inhalation:Not available.

Ingestion:Do NOT induce vomiting. Rinse mouth. Immediately call a POISON CENTER or doctor. Call a POISON CENTER or doctor if you feel unwell.

Serious Ingestion:Not available.

Section 5: Fire and Explosion Data

Extinguishing media:

Suitable extinguishing media: Foam. Dry powder. Carbon dioxide. Water spray. Sand.

Unsuitable extinguishing media: Do not use a heavy water stream.

Special hazards arising from the substance or mixture

Fire hazard:Highly flammable liquid and vapour.

Explosion hazard:May form flammable/explosive vapour-air mixture.

Advice for fire-fighters:

No data available.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

For emergency responders: Equip cleanup crew with proper protection.

Ventilate area.

For non-emergency personnel: Evacuate unnecessary personnel.

Environmental precautions:

Environmental precautions: Prevent entry to sewers and public waters. Notify authorities if product enterssewers or public waters.

Methods and material for containment and cleaning up:

Clean up methods: On land, sweep or shovel into suitable containers.

Section 7: Handling and Storage

Precautions:No naked lights. No smoking. Use onlynon-sparkingtools. Do not breathevapours.

Technical protective measures: Provide good ventilation in process area to prevent formation of vapour.

Special precautions: Avoid breathing dust, fume, gas, mist, vapours, spray.

Storage:Keep in fireproo place. Keep container tightly closed.

Incompatiblematerials:Heat sources.

Section 8: Exposure Controls/Personal Protection

Personal protection:  Avoid all unnecessary exposure.

• Respiratory protection:  In case of insufficient ventilation, wear suitable respiratory equipment.

• Hand protection:  Wear protective gloves.

• Eye protection:  Chemical goggles or safety glasses.

• Others:  When using, do not eat, drink or smoke.

Occupational Exposure Limits: No data available

Section 9: Physical and Chemical Properties

Physical state and appearance: Liquid.

Odor					:Characteristic.

Taste					: Not available.

Molecular Weight		: Not applicable.

Color					: Orange to red.

pH (1% soln/water)		: Not applicable.

Boiling Point			: 64.7 °C.

Melting Point			: -98 °C.

Critical Temperature		: Not available.

Specific Gravity			: Not available.

Vapor Pressure			: 96.5 at 20°C.

Vapor Density			: Not available.

Volatility				: Not available.

Odor Threshold			: Not available.

Water/Oil Dist. Coeff.		: Not available.

Ionicity (in Water)		: Not available.

Dispersion Properties		: Not available.

Solubility: Not available.

Section 10: Stability and Reactivity Data

Chemical Stability:Highly flammable liquid and vapour. May form flammable/explosive vapour-air mixture.

Instability Temperature:Not available.

Conditions to avoid:  Heat, open flame.

Materials to avoid:  Not available.

Hazardous decomposition products:  May release flammable gases.

Section 11: Toxicological Information

Acute toxicity:Oral: Toxic if   swallowed. Dermal: Toxic in contact with skin. Inhalation:gas: Toxic if inhaled.

Skin corrosion/irritation: Causes Skin irritation.

Serious eye damage/irritation: Causes serious eye irritation.

Respiratory or skin sensitisation: Not Classified.

Germ cell mutagenicity: Not Classified.

Carcinogenicity: Not classified.

Reproductive toxicity: Not classified.

Specific target organ toxicity (single exposure): Causes damage to organs.

Section 12: Ecological Information

Ecotoxicity: Not available.

BOD5 and COD: Not available.

Persistence – degradability :Not available.

Bioaccumulativepotential : Not established.

Mobility in soil :Not established.

Results of PBT and vPvBassessment :Not available.

Environmental precautions :Not available.

Special Remarks on the Products of Biodegradation: Not available.

Section 13: Disposal Considerations

Waste Disposal:

Avoid release to the environment. Dispose in a safe manner in accordance with

local/national regulations.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name :METHANOL

UN N° 		: 1230

ADR - Class 	: 3 (6.1)

Section 14: Transport Information (Continued)

Sea transport (IMDG) [English only]

Proper shipping name: METHANOL

UN N° 		: 1230

IMO-IMDG - Class or division :3 (6.1)

IMO-IMDG - Packing group :II

Air transport (ICAO-IATA) [English only]

Proper shipping name : METHANOL

UN N° : 1230

IATA - Class or division : 3 (6.1)

IATA - Packing group : II

Section 15: Other Regulatory Information

EU-Regulations:

Contains nosubstanceswithAnnexXVII restrictions

Contains  No REACH Candidate substance

Contains No REACH Annex XIV substances.

Germany:

VwVwS Annex reference: Water hazard class (WGK) 3, strongly Hazardous to water (Classification according to VwVwS, Annex 4).

2th Ordinance Implementing the Federal Immission Control Act - 12.BImSchV : Is not subject of the 12.         BlmSchV (Hazardous Incident Ordinance.

Denmark:

Class for fire hazard: Class I-1

Store unit- 1 liter

Classification remarks: F <Flam. Liq. 2>; Emergencymanagement guidelinesfor thestorage of flammable liquids mustbe followed.Recommendations Danish Regulation: Young  people  below  the age of 18 Years are not allowed to use  the  product Pregnant/breastfeeding  women  working  with  the product must not be in direct contact with the product.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.