MATERIAL SAFETY DATA SHEET

BRILLIANT CRESYL BLUE SOLUTION (ALCOHOLIC) MSDS CAS:

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:BRILLIANT CRESYL BLUE SOLUTION (ALCOHOLIC)

CAS#:

C.I.NO.:

Synonym:Not available.

Chemical Name:Not available.

Chemical Formula:Not available.

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| BRILLIANT CRESYL BLUE (C.I. No.-51010) | 81029-05-2 | > 1 < 5 % |
| Ethanol | 64-17-5 | > 99 % |

Section 3: Hazards Identification

Classification of the substance or mixture:

Classification EC 67/548 or EC 1999/45

Classification: C; R35

Hazard Class and Category Code(s), Regulation (EC) No 1272/2008 (CLP)

Health hazards: Skin corrosion - Category 1A - Danger (CLP : Skin Corr. 1A) H314

Other hazards : The substance does not fulfil the criteria to be identified as PBT substance or vPvB

substance according to Annex XIII of Regulation REACH.

Section 4: First Aid Measures

Description of first aid measures:

Inhalation: Remove to fresh air and keep at rest in a position comfortable for breathing.

Immediately call a POISON CENTER or doctor.

Skin contact: Immediately call a POISON CENTER or doctor. Wash with plenty of soap and water. Remove/Take off immediately all contaminated clothing.

Eye contact: Immediately call a POISON CENTER or doctor. Remove contact lenses, if present and easy to do. Continue rinsing. Rinse cautiously with water for several minutes.

Ingestion: Do NOT induce vomiting. Rinse mouth. Immediately call a POISON CENTER or doctor.

Most important symptoms and effects, both acute and delayed:

Symptoms relating to use: Causes severe skin burns and eye damage.

Indication of any immediate medical attention and special treatment needed:

General information: Never give anything by mouth to an unconscious person. If you feel unwell, seek medical advice (show the label where possible).

Section 5: Fire and Explosion Data

Extinguishing media:

Suitable extinguishing media: Foam. Dry powder. Carbon dioxide. Water spray. Sand.

Unsuitable extinguishing media: Do not use a heavy water stream.

Surrounding fires: Use water spray or fog for cooling exposed containers.

Section 5: Fire and Explosion Data (Continued)

Special hazards arising from the substance or mixture:

Flammable class: Highly flammable liquid and vapour.

Hazardous combustion products: Under fire conditions, hazardous fumes will be present.

Thermal decomposition generates : Corrosive vapours.

Advice for fire-fighters:

Protection against fire: Do not enter fire area without proper protective equipment, including respiratory protection.

Special procedures: Exercise caution when fighting any chemical fire. Avoid (reject) fire-fighting water to enter environment.

Section 6: Accidental Release Measures

Personal precautions:Use personal protective equipment. Avoid dust formation.Avoid breathing dust. Ensure adequate ventilation.

Environmental precautions:Do not let product enter drains.

Methods and materials for containment and cleaning up:

Pick up and arrange disposal without creating dust. Keep in suitable, closed containers for disposal.

Section 7: Handling and Storage

Handling: Handle empty containers with care because residual vapours are flammable. Wash thoroughly after handling. Avoid contact during pregnancy or while nursing. Do not breathe dust, fume, gas, mist, vapours, spray.

Technical protective measures: Provide good ventilation in process area to prevent formation of vapour.

Proper grounding procedures to avoid static electricity should be followed. Use

explosion-proof electrical, ventilating, lighting, equipment.

Special precautions : No naked lights. No smoking.

Storage: Keep only in the original container in a cool, well ventilated place. Keep in fireproof

place. Ground/bond container and receiving equipment. Keep container tightly

closed.

Storage regulation: Comply with applicable regulations.

Storage - away from: Strong bases. Strong acids. Sources of ignition. Direct sunlight. Heat sources.

Section 8: Exposure Controls/Personal Protection

Personal protection:

Personal protection: Avoid all unnecessary exposure.

• Respiratory protection: Wear approved mask.

• Hand protection: Wear protective gloves.

• Skin protection: Wear suitable protective clothing.

• Eye protection: Chemical goggles or face shield.

• Others: When using, do not eat, drink or smoke.

Control parameters:

Occupational Exposure Limits: No data available

Section 9: Physical and Chemical Properties

Appearance Form	: Liquid

Color                                      : Colorless

Odour                                     : Characteristic

pH				: No data available

Molecular Weight	: No data available

Melting point		: No data available

Boiling point		: 78 °C

Flash point			: No data available

Ignition temperature	: No data available

Lower explosion limit	: No data available

Upper explosion limit	: No data available

Water solubility		: No data available

Section 10: Stability and Reactivity Data

Chemical stability:Stable under recommended storage conditions.

Materials to avoid:Strong acids. Strong bases.

Conditions to avoid :  Direct sunlight. Extremely high or low temperatures. Open flame.

Hazardous decomposition products:Fumes. Carbon monoxide. Carbon dioxide.May release flammable gases.Thermal decomposition generates : Corrosive vapours.

Section 11: Toxicological Information

Information on toxicological effects:

Toxicity information: The product has been not fully tested. The calculated risk has been done under the

requirements of the EU regulations.

Acute toxicity:

• Inhalation: Based on available data, the classification criteria are not met.

• Dermal: Based on available data, the classification criteria are not met.

• Ingestion: Based on available data, the classification criteria are not met.

Corrosion: Causes severe skin burns and eye damage.

Irritation: Based on available data, the classification criteria are not met.

Sensitization: Based on available data, the classification criteria are not met.

Mutagenicity: Based on available data, the classification criteria are not met.

Carcinogenicity: Based on available data, the classification criteria are not met.

Toxic for reproduction: Based on available data, the classification criteria are not met.

STOT-single exposure: Based on available data, the classification criteria are not met.

STOT-repeated exposure: Based on available data, the classification criteria are not met.

Aspiration hazard: Based on available data, the classification criteria are not met.

Section 12: Ecological Information

Toxicity:The product has been not fully tested. The calculated risk has been done under the requirements of the EU regulations.

Persistence and degradability:Biodegradable.

Bioaccumulative potential:No data available.

Mobility in soil:No data available

PBT and vPvB assessment:The substance does not fulfil the criteria to be identified as PBT substance or vPvBsubstance according to Annex XIII of Regulation REACH.

Other adverse effects:

Environmental precautions:  Avoid release to the environment.

Section 13: Disposal Considerations

General: Avoid release to the environment. Dispose in a safe manner in accordance with

local/national regulations. Dispose of this material and its container to hazardous or special waste collection

point, in accordance with local, regional, national and/or international regulation.

Special precautions: Handle empty containers with care because residual vapours are flammable.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name :ETHANOL (ETHYL ALCOHOL)

UN N° 		: 1170

ADR - Class 	: 3 II

Sea transport (IMDG) [English only]

Proper shipping name: ETHANOL (ETHYL ALCOHOL)

UN N° 		: 1170

IMO-IMDG - Class or division :3

IMO-IMDG - Packing group :II

Air transport (ICAO-IATA) [English only]

Proper shipping name : ETHANOL (ETHYL ALCOHOL)

UN N° : 1170

IATA - Class or division : 3

IATA - Packing group : II

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture:

Safety, health and environmental:  Ensure all national/local regulations are observed.

regulations/legislation specific

for the substance or mixture

REACH Restrictions - Annex XVII:  The components of this product are not subject to restrictions.

REACH Authorisation - Annex XIV:The components of this product are not subject to authorization.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.