MATERIAL SAFETY DATA SHEET

CADMIUM FLUORIDE 98%

MSDS CAS: 7790-79-6

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: CADMIUM FLUORIDE

CAS#: 7790-79-6

Synonym: Not available.
Chemical Name: Not available.

Chemical Formula: CdF2

Brand : OXFORD

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Cadmium Fluoride | 7790-79-6 | 98-100% |

Section 3: Hazards Identification

Risk advice to man and the environment :

May cause cancer. May cause heritable genetic damage. May impair fertility. May cause harm to the unborn child. Also toxic if swallowed. Also very toxic by inhalation. Also toxic: danger of serious damage to health by prolonged exposure through inhalation and if swallowed. Very toxic to aquatic organisms, may cause long-term  adverse effects in the aquatic environment.

Section 4: First Aid Measures

Eye Contact :

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

Skin Contact :

Wash off with soap and plenty of water. Consult a physician.

Inhalation :

If breathed in, move person into fresh air. If not breathing give artificial respiration Consult a physician.

General advice:

Consult a physician. Show this safety data sheet to the doctor in attendance.

Swallowed :

Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Section 5: Fire and Explosion Data

Suitable extinguishing media  :

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special protective equipment for fire-fighters :

Wear self contained breathing apparatus for fire fighting if necessary.

Section 6: Accidental Release Measures

Personal precautions :

Wear respiratory protection. Avoid dust formation. Avoid breathing dust. Ensure adequate ventilation. Evacuate personnel to safe areas.

Environmental precautions :

Prevent further leakage or spillage if safe to do so. Do not let product enter drains. Discharge into the

environment must be avoided.

Methods for cleaning up :

Pick up and arrange disposal without creating dust. Keep in suitable, closed containers for disposal.

Section 7: Handling and Storage

Handling :

Avoid exposure - obtain special instructions before use.Avoid formation of dust and aerosols.

Provide appropriate exhaust ventilation at places where dust is formed. Normal measures for preventive fire

protection.

Storage :

Store in cool place. Keep container tightly closed in a dry and well-ventilated place.

Section 8: Exposure Controls/Personal Protection

Personal protective equipment

Respiratory protection :

Where risk assessment shows air-purifying respirators are appropriate use a dust mask type N95 (US) or

type P1 (EN 143) respirator. Where risk assessment shows air-purifying respirators are appropriate use a

full-face particle respirator type N99 (US) or type P2 (EN 143) respirator cartridges as a backup to

engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator.

Use respirators and components tested and approved under appropriate government standards such as

NIOSH (US) or CEN (EU).

Hand protection :

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard

EN 374 derived from it. Handle with gloves.

Section 8: Exposure Controls/Personal Protection (Continued)

Eye protection :

Face shield and safety glasses.

Skin and body protection :

Choose body protection according to the amount and concentration of the dangerous substance at the work

place.

Hygiene measures :

Avoid contact with skin, eyes and clothing. Wash hands before breaks and immediately after handling the

product.

Section 9: Physical and Chemical Properties

Appearance :

Form 		 				: powder.

Colour  	           				: white.

Molecular Weight 				: 150.41g/mole.

pH							: no data available.

Melting point					: no data available.

Boiling point					: no data available.

Flash point						: not applicable.

Ignition temperature				: no data available.

Lower explosion limit				: no data available.

Upper explosion limit				: no data available.

Density						: 6,33 g/mL at 25 °C.

Water solubility					: no data available.
Relative density					: 6,33 g/cm3 at 25 °C

Section 10: Stability and Reactivity Data

Storage stability :

Stable under recommended storage conditions.

Materials to avoid :

Oxidizing agents.

Hazardous decomposition products :

Hazardous decomposition products formed under fire conditions. - Hydrogen fluoride, Cadmium/cadmium

oxides.

Section 11: Toxicological Information

Acute toxicity		 :     LD50 Oral - guinea pig - 150 mg/kg.

Irritation and corrosion :   no data available.

Sensitisation 		 :        no data available.

Chronic exposure:

Not classifiable as to its carcinogenicity to humans (Cadmium fluoride)
Probably carcinogenic to humans (Cadmium fluoride)

Signs and Symptoms of Exposure :

Salivation, Nausea, Vomiting, Fever, Kidney injury may occur., Damage to the lungs.

Potential Health Effects :

Inhalation           : May be fatal if inhaled. May cause respiratory tract irritation.

Skin                      : May be harmful if absorbed through skin. May cause skin irritation.

Eyes                     : May cause eye irritation.

Ingestion             : Toxic if swallowed.

Target Organs : Kidney injury may occur., Damage to the lungs., Kidney, Lungs.

Section 12: Ecological Information

Elimination information (persistence and degradability) : no data available

Ecotoxicity effects  :  no data available.

Further information on ecology  : Very toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

Section 13: Disposal Considerations

Product :

Observe all federal, state, and local environmental regulations. Contact a licensed professional waste disposal

service to dispose of this material. Dissolve or mix the material with a combustible solvent and burn in a

chemical incinerator equipped with an afterburner and scrubber.

Contaminated packaging : Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name 			: CADMIUM COMPOUND (Cadmium fluoride)

UN N°					:  2570

H.I. nr					:  Not Available

ADR - Class				:  6.1

Labelling - Transport    			:  Not Available

Sea transport (IMDG) [English only]

Proper shipping name 			: CADMIUM COMPOUND (Cadmium fluoride)

UN N°		         			: 2570

IMO-IMDG - Class or division 	: 6.1

IMO-IMDG - Packing group 	 	: II

Section 14: Transport Information (Continued)

Air transport (ICAO-IATA) [English only]

Proper shipping name 			: CADMIUM COMPOUND (Cadmium fluoride)

UN N° 					: 2570

IATA - Class or division : 9 		: 6.1

IATA - Packing group 	      		: II

Section 15: Other Regulatory Information

Labelling :

May cause cancer.

May cause heritable genetic damage.

May impair fertility.

May cause harm to the unborn child.

Also toxic if swallowed.

Also very toxic by inhalation.

Toxic :

Danger of serious damage to health by prolonged exposure through inhalation and if swallowed.

Very toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

Avoid exposure :

Obtain special instructions before use.

In case of accident or if you feel unwell, seek medical advice immediately (show the label where possible).

This material and its container must be disposed of as hazardous waste.

Avoid release to the environment. Refer to special instructions/ Safety data sheets.

Restricted to professional users.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Chem makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Chem will not be responsible for damages resulting from use of or reliance upon this information.