MATERIAL SAFETY DATA SHEET

CADMIUM PERCHLORATE

MSDS CAS: 79490-00-9

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: Cadmium Perchlorate

CAS#: 79490-00-9

Synonym:

Chemical Name: Not available.

Chemical Formula: : CdCl2O8· xH2O

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Cadmium Perchlorate | 79490-00-9 | 100% |

Section 3: Hazards Identification

Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008 :

Oxidizing solids (Category 2), H272

Acute toxicity, Inhalation (Category 4), H332

Acute toxicity, Dermal (Category 4), H312

Acute toxicity, Oral (Category 4), H302

Acute aquatic toxicity (Category 1), H400

Chronic aquatic toxicity (Category 1), H410

Classification according to EU Directives 67/548/EEC or 1999/45/EC

O  :  Oxidising

Xn:  Harmful

N  :  Dangerous for the environment

Label elements

Hazard statement(s):

May intensify fire; oxidiser.

Harmful if swallowed.

Harmful in contact with skin.

Harmful if inhaled.

Very toxic to aquatic life with long lasting effects.

Precautionary statement(s):

Keep/Store away from clothing/ combustible materials.

Avoid release to the environment.

Wear protective gloves/ protective clothing.

Dispose of contents/ container to an approved waste disposal plant.

Supplemental Hazard: Not available.

Other hazards:  Not available.

Section 4: First Aid Measures

Description of first aid measures

General advice:

Consult a physician. Show this safety data sheet to the doctor in attendance.

Section 4: First Aid Measures (Continued)

If inhaled:

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

Skin Contact:

Wash off with soap and plenty of water. Consult a physician.

Eye Contact:

Flush eyes with water as a precaution.

If swallowed:

Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media:

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special hazards arising from the substance or mixture:

Hydrogen chloride gas, Cadmium/cadmium oxides

Advice for firefighters:

Wear self contained breathing apparatus for fire fighting if necessary.

Further information:

Use water spray to cool unopened containers.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Use personal protective equipment. Avoid dust formation. Avoid breathing vapours, mist or gas. Ensure

adequate ventilation. Evacuate personnel to safe areas. Avoid breathing dust.

Environmental precautions:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains. Discharge into the

environment must be avoided.

Methods and materials for containment and cleaning up:

Sweep up and shovel. Contain spillage, and then collect with an electrically protected vacuum cleaner or

by wet-brushing and place in container for disposal according to local regulations (see section 13). Keep

in suitable, closed containers for disposal.

Section 7: Handling and Storage

Precautions for safe handling:

Avoid contact with skin and eyes. Avoid formation of dust and aerosols.

Provide appropriate exhaust ventilation at places where dust is formed.Keep away from sources of ignition

No smoking. Keep away from heat and sources of ignition.Normal measures for preventive fire protection.

Conditions for safe storage, including any incompatibilities:

Store in cool place. Keep container tightly closed in a dry and well-ventilated place.

Section 8: Exposure Controls/Personal Protection

Exposure controls

Appropriate engineering controls:

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and

at the end of workday.

Personal protective equipment

Eye/face protection:

Safety glasses with side-shields conforming to EN166 Use equipment for eye protection tested

and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

Skin protection:

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique

(without touching glove's outer surface) to avoid skin contact with this product. Dispose of

contaminated gloves after use in accordance with applicable laws and good laboratory practices.

Wash and dry hands.

Body Protection:

Complete suit protecting against chemicals, The type of protective equipment must be selected

according to the concentration and amount of the dangerous substance at the specific workplace.

Respiratory protection:

Where risk assessment shows air-purifying respirators are appropriate use a full-face particle

respirator type N100 (US) or type P3 (EN 143) respirator cartridges as a backup to engineering

controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use

respirators and components tested and approved under appropriate government standards.

Section 8: Exposure Controls/Personal Protection (Continued)

Control of environmental exposure:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains. Discharge into

the environment must be avoided.

Section 9: Physical and Chemical Properties

Appearance Form				: Crystals with lumps

Odour                				: no data available

Odour		  				: Threshold no data available

Molecular Weight  				: 311,31 g/mol
pH no     		  				: data available

Melting point	  				: no data available

Boiling point 	  				: no data available
 Flash point		  				: no data available

Evapouration rate  				: no data available

Flammability (solid, gas)			: no data available

flammability or  explosive limits		: no data available

Flash point		 				: no data available

Evapouration rate				: no data available

Flammability (solid, gas)			: no data available

Flammability or explosive limits		: no data available

Section 10: Stability and Reactivity Data

Reactivity			:    no data available

Chemical stability	: Stable under recommended storage conditions.

Possibility of hazardous reactions: no data available

Conditions to avoid	: no data available

Incompatible materials: Organic materials

Hazardous decomposition products: Other decomposition products - no data available

Section 11: Toxicological Information

Information on toxicological effects

Acute toxicity			: no data available

Skin corrosion/irritation	: no data available

Serious eye damage/eye irritation: no data available

Respiratory or skin sensitisation : no data available

Germ cell mutagenicity	: no data available

Reproductive toxicity	: no data available

Specific target organ toxicity - single exposure : no data available

Specific target organ toxicity - repeated exposure: no data available

Aspiration hazard		:no data available.

Additional Information

RTECS: Not available.

Acute inhalation exposure to cadmium fumes may cause "metal fume fever" with flu-like symptoms of

weakness, fever, headache, chills, nausea, vomiting, dizziness, sweating, muscular pain, cough and

difficulty breathing. Acute pulmonary edema may develop within 24 hours and reaches a maximum by

three days.The first chronic effect of exposure to cadmium is generally kidney damage, manifested by

excretion of excessive protein in the urine, followed by anemia, teeth discoloration and loss of smell.

Cadmium also is believed to cause pulmonary emphysema and bone disease., To the best of our

knowledge, the chemical, physical, and toxicological properties have not been thoroughly investigated.

Section 12: Ecological Information

Toxicity  : no data available

Persistence and degradability : no data available

Bioaccumulative  potential	: no data available

Mobility in soil   : no data available

Results of PBT and vPvB assessment  : PBT/vPvB assessment not available as chemical safety. assessment not required/not conducted.

Other adverse effects   :Very toxic to aquatic life with long lasting effects

Section 13: Disposal Considerations

Waste treatment methods

Product:

Contact a licensed professional waste disposal service to dispose of this material. Burn in a chemical

incinerator equipped with an afterburner and scrubber but exert extra care in igniting as this material is

highly flammable. Offer surplus and non-recyclable solutions to a licensed disposal company.

Contaminated packaging:

Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name 		: PERCHLORATES, INORGANIC, N.O.S.

UN N°				:  1481

H.I. nr				:  No Data Available

ADR - Class			:  5.1

Labelling - Transport    		:

Sea transport (IMDG) [English only]

Proper shipping name 		: PERCHLORATES, INORGANIC, N.O.S.

UN N°		         		: 1481

IMO-IMDG - Class or division : 5.1

IMO-IMDG - Packing group 	 : II

Air transport (ICAO-IATA) [English only]

Proper shipping name 		: PERCHLORATES, INORGANIC, N.O.S.

UN N° 				: 1481

IATA - Class or division : 9 	: 5.1

IATA - Packing group 	      	: II

`

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture:

no data available.

Chemical Safety Assessment: For this product a chemical safety assessment was not carried out.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.