MATERIAL SAFETY DATA SHEET

CADMIUM TELLURIDE

MSDS CAS: 1036-25-8

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: CADMIUM TELLURIDE

CAS#: 1036-25-8

Synonym:

Chemical Name: Not available.

Chemical Formula: : : CdTe

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Cadmium telluride | 1036-25-8 | >95% |

Section 3: Hazards Identification

Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008 [EU-GHS/CLP]:

Acute toxicity, Inhalation (Category 4)

Acute toxicity, Dermal (Category 4)

Acute toxicity, Oral (Category 4)

Acute aquatic toxicity (Category 1)

Chronic aquatic toxicity (Category 1)

Classification according to EU Directives 67/548/EEC or 1999/45/EC:

Very toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment. Harmful

by inhalation, in contact with skin and if swallowed. Harmful by inhalation, in contact with skin and if

swallowed. Very toxic to aquatic organisms, may cause long-term adverse effects in the aquatic

environment.

Label elements

Hazard statement(s):

Harmful if swallowed.

Harmful in contact with skin.

Harmful if inhaled.

Very toxic to aquatic life with long lasting effects.

Precautionary statement(s):

Avoid release to the environment.

Wear protective gloves/ protective clothing.

Dispose of contents/ container to an approved waste disposal plant.

Supplemental Hazard: Not available.

R-phrase(s):

Harmful by inhalation, in contact with skin and if swallowed. Very toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

S-phrase(s):

This material and its container must be disposed of as hazardous waste.  Avoid release to the environment. Refer to special instructions/ Safety  data sheets.

Section 4: First Aid Measures

Description of first aid measures

General advice:

Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled:

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.
Skin Contact:

Wash off with soap and plenty of water. Consult a physician.

Eye Contact:

Flush eyes with water as a precaution.

If swallowed:

Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Most important symptoms and effects, both acute and delayed:

Acute inhalation exposure to cadmium fumes may cause "metal fume fever" with flu-like symptoms of

weakness, fever, headache, chills, nausea, vomiting, dizziness, sweating, muscular pain, cough and

difficulty breathing. Acute pulmonary edema may develop within 24 hours and reaches a maximum by

three days.The first chronic effect of exposure to cadmium is generally kidney damage, manifested by

excretion of excessive protein in the urine, followed by anemia, teeth discoloration and loss of smell.

Cadmium also is believed to cause pulmonary emphysema and bone disease.

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media:

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special hazards arising from the substance or mixture:

no data available

Advice for firefighters:

Wear self contained breathing apparatus for fire fighting if necessary.
Further information:

no data available

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Use personal protective equipment. Avoid dust formation. Avoid breathing vapors, mist or gas. Ensure

adequate ventilation. Avoid breathing dust.

Environmental precautions:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains. Discharge into the

environment must be avoided.

Methods and materials for containment and cleaning up:

Pick up and arrange disposal without creating dust. Sweep up and shovel. Keep in suitable, closed

containers for disposal.

Section 7: Handling and Storage

Precautions for safe handling:

Avoid contact with skin and eyes. Avoid formation of dust and aerosols.

Provide appropriate exhaust ventilation at places where dust is formed.

Conditions for safe storage, including any incompatibilities:

Store in cool place. Keep container tightly closed in a dry and well-ventilated place.

Section 8: Exposure Controls/Personal Protection

Control parameters:

Components with workplace control parameters.

Exposure controls

Appropriate engineering controls:

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and

at the end of workday.

Personal protective equipment

Eye/face protection:

Safety glasses with side-shields conforming to EN166 Use equipment for eye protection tested

and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

Section 8: Exposure Controls/Personal Protection (Continued)

Skin protection:

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique

(without touching glove's outer surface) to avoid skin contact with this product. Dispose of

contaminated gloves after use in accordance with applicable laws and good laboratory practices.

Wash and dry hands.

Body Protection:

Complete suit protecting against chemicals, The type of protective equipment must be selected

according to the concentration and amount of the dangerous substance at the specific workplace.

Respiratory protection:

For nuisance exposures use type P95 (US) or type P1 (EU EN 143) particle respirator.For higher

level protection use type OV/AG/P99 (US) or type ABEK-P2 (EU EN 143) respirator cartridges.

Use respirators and components tested and approved under appropriate government standards

such as NIOSH (US) or CEN (EU).

Section 9: Physical and Chemical Properties

Appearance Form : crystalline

Colour		  : Black
Odour		  : O

Molecular Weight  : 240,01 g/mol

Odour Threshold   :  no data available

pH no     		  : data available

Melting point	  : 11041 °C / 1905.8 °F

Boiling point 	  : no data available 
Flash point		  : no data available

Evapouration rate  : no data available

Flammability (solid, gas) : no data available

flammability or  explosive limits: no data available

Vapour pressure	  : no data available

Vapour density 	  : no data available

Relative density 	  : 6,2 g/cm3 at 25 °C

Section 9: Physical and Chemical Properties (Continued)

Water solubility 	  :no data available
 Water solubility 	  :no data available
 Autoignition temperature    :no data available

Decomposition temperature :no data available
 Viscosity : no data available

Explosive properties  : no data available

Oxidizing properties  : no data available

Section 10: Stability and Reactivity Data

Reactivity			:    no data available

Chemical stability	: no data available
Possibility of hazardous reactions: no data available

Conditions to avoid	: no data available

Incompatible materials : Strong oxidizing agents, Strong acids.

Hazardous decomposition products: Other decomposition products - no data available

Section 11: Toxicological Information

Information on toxicological effects

Acute toxicity	: LD50 Intraperitoneal - rat - 2.820 mg/kg Remarks: Behavioral: Tremor. Behavioral: Change in motor activity (specific assay). Nutritional and Gross  Metabolic: Weight loss or decreased weight gain.

Skin corrosion/irritation : no data available

Serious eye damage/eye irritation: no data available

Respiratory or skin sensitisation : no data available

Germ cell mutagenicity: no data available

Reproductive toxicity: no data available

Specific target organ toxicity - single exposure : no data available

Specific target organ toxicity - repeated exposure: no data available

Aspiration hazard: no data available.

Section 11: Toxicological Information (Continued)

Potential health effects

Inhalation : Harmful if inhaled. May cause respiratory tract irritation.

Ingestion   : Harmful if swallowed.

Skin 	          : Harmful if absorbed through skin. May cause skin irritation.

Eyes 	         : May cause eye irritation.

Signs and Symptoms of Exposure:

Acute inhalation exposure to cadmium fumes may cause "metal fume fever" with flu-like symptoms of

weakness, fever, headache, chills, nausea, vomiting, dizziness, sweating, muscular pain, cough and

difficulty breathing. Acute pulmonary edema may develop within 24 hours and reaches a maximum by

three days.The first chronic effect of exposure to cadmium is generally kidney damage, manifested by excretion of excessive protein in the urine, followed by anemia, teeth discoloration and loss of smell.

Cadmium also is believed to cause pulmonary emphysema and bone disease.

Section 12: Ecological Information

Toxicity  : no data available

Persistence and degradability : no data available

Bioaccumulative  potential	: no data available

Mobility in soil   : no data available

Results of PBT and vPvB assessment  : PBT/vPvB assessment not available as chemical safety. assessment not required/not conducted.

Other adverse effects   :Very toxic to aquatic life with long lasting effects

Section 13: Disposal Considerations

Waste treatment methods

Product:

Offer surplus and non-recyclable solutions to a licensed disposal company. Dissolve or mix the material

with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and scrubber.

Contaminated packaging:

Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name 	: CADMIUM COMPOUND (Cadmium telluride)

UN N°			: 2570

ADR -Transport hazard class	:  6.1

ADR - Group 		: III

Environmental hazards ADR : no

Sea transport (IMDG) [English only]

Proper shipping name      : CADMIUM COMPOUND (Cadmium telluride)

UN N°		               : 2570

IMDG -Transport hazard class : 6.1

IMDG-Packaging group  : III

Environmental hazards IMDG : no

Air transport (ICAO-IATA) [English only]

Proper shipping name : Cadmium compound (Cadmium telluride)

UN N° : 2570

IATA -Transport hazard class(es): 6.1

IATA - Packing group : III

Environmental hazards IATA : no

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture:

no data available.

Chemical Safety Assessment: no data available.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.