MATERIAL SAFETY DATA SHEET

CALCIUM BORATE

MSDS CAS: 12007-56-6

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: CALCIUM BORATE

CAS#: 12007-56-6

Synonym: Calcium tetraborate 
Chemical Name: Not available.

Chemical Formula: CaB4O7.

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Calcium borate | 12007-56-6 | 95 – 100% |

Section 3: Hazards Identification

Wash with plenty of soap and water.

IF INHALED:

Remove victim to fresh air and keep at rest in a position comfortable for breathing.

If in eyes:

Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

If skin irritation occurs:

Get medical advice/ attention.

Section 4: First Aid Measures

Eye Contact: Flush eyes with large amounts of water for fifteen minutes. Separate eyelids with fingers. If irritation persists, seek medical attention.

Skin Contact: Wash skin with soap and water. If irritation persists, seek medical attention.

Inhalation: Move to a fresh air environment. Contact a physician if breathing becomes difficult.

Ingestion:  Do not induce vomiting. Seek medical attention.

Section 5: Fire and Explosion Data

Flash Points: Not available.

Explosion Limits:

Lower: Not Available

Upper: Not Available.

Auto Ignition Temperature: Not Available.

Extinguishing Media: Carbon dioxide, dry chemical powder, alcohol resistant foam or water spray.

Protective Equipment: Wear self-contained respirator and fully protective impervious suit.

Specific Hazards: May emit hazardous fumes under fire conditions.

Section 6: Accidental Release Measures

Personal Protection:

Wear a self-contained breathing apparatus, rubber boots and gloves, and disposable coveralls. Dispose of coveralls after use. Keep unprotected persons away.

Environmental Protection:

Keep spills out of sewers and bodies of water. Dike  and contain the spill with inert material. Absorb on sand, vermiculite or diatomite. Transfer material to a container for disposal or recovery. Ventilate area and wash spill site after material pickup is complete.

Section 7: Handling and Storage

Handling:

Avoid breathing dust, vapor, mist or gas. Avoid contact with skin and eyes. Avoid prolonged or repeated exposure. Use only in a chemical fume hood. Open and handle container with care. Keep ignition sources away.

Storage:

Store in a tightly closed container in a dry,  well ventilated place.

Sensitivities: Not Available

Storage Temperature: 15 – 30 ºC

Section 8: Exposure Controls/Personal Protection

Eyes:

Wear appropriate protective eyeglass or chemical  safety goggles. Make sure that there is an eyewash facility in your vicinity.

Skin:

Wear impervious gloves and protective clothing.

Respiratory:

Use a NIOSH approved respirator when exposure limits are exceeded or if irritation or other symptoms are experienced.

Section 8: Exposure Controls/Personal Protection (Continued)

Exposure Limits: 		Country 	Source 	Type 		      Value

USA 	 ACGIH 	 TWA 		 Not Available

USA 	 OSHA 	 STEL 	 Not Available

USA 	 OSHA 	 PEL 		 Not Available

Section 9: Physical and Chemical Properties

Physical state and appearance		: Powder

Odor							: Not Available

Molecular Weight				: Not Available

Melting Point					: Not Available

Boiling Point					: Not Available

pH Value						: Not Available

Density						: Not Available

Refractive Index, n20D				: Not Available

Viscosity						: Not Available

Vapor Pressure					: Not Available

Vapor Density(Air=1)				: Not Available

Solubility in Water				: Not Available

Section 10: Stability and Reactivity Data

Stability: Stable under normal temperatures and pressures.

Incompatibility:  Strong reducing agents.

Conditions to Avoid: Heat, Flame, Sparks, other ignition sources.

Hazardous Decomposition Products: Calcium oxides, Boron oxides, Hydrogen oxide gas.

Section 11: Toxicological Information

RTECS Reference		: Not Available

Target Organs			: Not Available

Toxicity Data			: Not Available

Skin corrosion/irritation	: Not Available

Serious eye damage/irritation : Not Available

Carcinogenicity			: Not Available

Section 12: Ecological Information

Toxicity				: Not Available.

Persistence and degradability : Not Available.

Bioaccumulative  potential	: Not Available.

Mobility in soil			: Not Available.

PBT and vPvB assessment	: Not Available.

Other adverse effects		: Not Available.

Section 13: Disposal Considerations

Waste Disposal:

Contact a licensed professional waste disposal service. Dispose in a manner consistent with federal, state and local environmental regulations.

Section 14: Transport Information

Land transport (ADR-RID)

General information    			: Not regulated.

Sea transport (IMDG) [English only]

General information    			: Not regulated.

Air transport (ICAO-IATA) [English only]

General information   			: Not regulated.

Section 15: Other Regulatory Information

United States:

Toxic Substance Control Act (TSCA): Listed

Superfund Amendments and Reauthorization Act (SARA 302): Not listed

Superfund Amendments and Reauthorization Act (SARA 311/312): Not listed

Superfund Amendments and Reauthorization Act (SARA 313): Not listed

European Union:

European Inventory of Existing Chemical Substances (EINECS): No. 234-511-7

Hazard Codes	: Xi

Risk Statements	: 36/37/38

Safety Statements	: 24/25

Canada :

Canadian Domestic Substances List (DSL)    : Not listed

Canadian Domestic Substances List (NDSL) : Listed

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.