MATERIAL SAFETY DATA SHEET

CALCIUM BROMATE

MSDS CAS: 10102-75-7

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: CALCIUM BROMATE

CAS#: 10102-75-7

Synonym: 
Chemical Name: Not available.

Chemical Formula: Br2CaO6

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| CALCIUM BROMATE | 10102-75-7 | 90 - 100% |

Section 3: Hazards Identification

Emergency overview: Contact with combustible material may cause fire Acute Health

Principle routes of exposure: Skin

Inhalation: May cause irritation of respiratory tract.

Ingestion: May be harmful if swallowed.

Skin contact: May cause allergic skin reaction.

Eye contact: Avoid contact with eyes.

Statements of hazard : May cause fires.

Statement of Spill or Leak - ANSI Label : Eliminate all ignition sources. Absorb and/or contain spill with inert materials (e.g., sand, vermiculite). Then place in appropriate container. For large spills, use water spray to disperse vapors, flush spill area. Prevent runoff from entering waterways or sewers.

Precautions ANSI Label : Do not breathe vapors or spray mist Do not get in eyes, on skin, or on clothing. Wash thoroughly after handling.

Section 4: First Aid Measures

General advice: In the case of accident or if you feel unwell, seek medical advice immediately (show the label where possible).

Inhalation: Move to fresh air. Call a physician immediately.

Skin contact: Rinse immediately with plenty of water and seek medical advice

Ingestion: Do not induce vomiting without medical advice.

Eye contact: In the case of contact with eyes, rinse immediately with plenty of water and seek medical advice.

Protection of first-aiders: No information available.

Medical conditions aggravated by exposure: None known.

Section 5: Fire and Explosion Data

Suitable extinguishing media: Use dry chemical, CO2, water spray or `alcohol` foam.

Specific hazards: Burning produces irritant fumes.

Unusual hazards: None known.

Special protective equipment for firefighters: Wear self contained breathing apparatus for fire fighting if necessary. In the event of fire and/or explosion do not breathe fumes.

Specific methods: Water mist may be used to cool closed containers.

Flash point: Not determined

Autoignition temperature: Not determined

NFPA rating:

NFPA Health: 2

NFPA Flammability: 2

NFPA Reactivity: 1

NFPA Special Note: ox

Section 6: Accidental Release Measures

Personal precautions		: Use personal protective equipment.

Environmental precautions	: Prevent product from entering drains.

Methods for cleaning up	: Sweep up and shovel into suitable containers for disposal.

Section 7: Handling and Storage

Storage:

ROOM TEMPERATURE

Handling: Use only in area provided with appropriate exhaust ventilation.

Section 7: Handling and Storage (Continued)

Safe handling advice: Wear personal protective equipment. Remove and wash contaminated clothing before reuse.

Technical measures/storage conditions: Keep container tightly closed in a dry and well-ventilated place. Keep away from open flames, hot surfaces and sources of ignition For safety reasons in case of fire, cans should be stored separately in closed containments. Do not expose material to high temperatures, changes in pressure or sudden shock. Keep away from contact with clothing and other combustible materials.

Incompatible products: Oxidising and spontaneously flammable products

Section 8: Exposure Controls/Personal Protection

Engineering measures: Ensure adequate ventilation, especially in confined areas.

PERSONAL PROTECTIVE EQUIPMENT

Respiratory protection: In case of mist, spray or aerosol exposure wear suitable personal respiratory protection and protective suit.

Hand protection: Pvc or other plastic material gloves.

Skin and body protection: Impervious clothing.

Eye protection: If splashes are likely to occur, wear: Safety glasses with side-shields.

Hygiene measures: Handle in accordance with good industrial hygiene and safety practice.

Section 9: Physical and Chemical Properties

Physical state					: Solid

Molecular weight					: 295.884 g/mole

Melting point/range				: No data available at this time.

Boiling point/range				: No Data available at this time.

Density						: No data available

Vapor pressure					: No data available

Evaporation rate					: No data available

Section 9: Physical and Chemical Properties (Continued)

Vapor density					: No data available

Solubility (in water)				: No data available

Flash point						: Not determined

Autoignition temperature			: Not determined

Section 10: Stability and Reactivity Data

Stability: Stable under recommended storage conditions.

Polymerization: None under normal processing.

Hazardous decomposition products: Bromine.

Materials to avoid: A strong oxidizer. Incompatible with combustible materials, reducing agents, strong acids, aluminum, ammonium salts, arsenic, carbon, copper, metal powder and sulfides, organic matter, phosphorus.

Conditions to avoid: Exposure to air or moisture over prolonged periods.

Section 11: Toxicological Information

Product Information

Acute toxicity

Components 			RTECS Number: 	Selected LD50s and LC50s

CALCIUM BROMATE 		Not Available 		Not Determined

Chronic toxicity: Chronic exposure may cause nausea and vomiting, higher exposure causes

unconsciousness.

Local effects: Symptoms of overexposure may be headache, dizziness, tiredness, nausea and

vomiting.

Section 11: Toxicological Information (Continued)

Specific effects:  May include moderate to severe erythema (redness) and moderate edema (raised

skin), nausea, vomiting, headache.

Primary irritation: No data is available on the product itself.

Carcinogenic effects:  No data is available on the product itself.

Mutagenic effects:  No data is available on the product itself.

Reproductive toxicity: No data is available on the product itself

Section 12: Ecological Information

Mobility 		 : No data available

Bioaccumulation	 : No data available

Ecotoxicity effects : No data available

Aquatic toxicity 	 : May cause long-term adverse effects in the aquatic environment.

Components : 	         U.S. DOT - Appendix B    U.S. DOT - Appendix B        United Kingdom - The Red

Marine Pollutant	     Severe Marine Pollutants      List:

CALCIUM BROMATE     Not Listed		      Not Listed		        	       Not Listed

Components		Germany VCI (WGK)     World Health Organization	Ecotoxicity - Fish Species

(WHO) - Drinking Water	Data

CALCIUM BROMATE	 Not Listed 		        Not Listed 			Not Listed

Components		Ecotoxicity-Freshwater   Ecotoxicity- Microtox		Ecotoxicity - Water Flea				Algae Data		       Data				Data

CALCIUM BROMATE 	Not Listed 		       Not Listed 			Not Listed

Components		 EPA - ATSDR Priority 	EPA - HPV Challenge      California- Priority Toxic

List				Program Chemical List	Pollutants

CALCIUM BROMATE 	Not Listed 			Not Listed 			Not Listed

Components 		California - Priority 	Toxic Pollutants 	California - Priority Toxic Pollutants

CALCIUM BROMATE 	Not Listed 					Not Listed

Section 13: Disposal Considerations

Waste from residues / unused products:

Waste disposal must be in accordance with appropriate Federal, State, and local regulations. This product, if unaltered by use, may be disposed of by treatment at a permitted facility or as advised by your local hazardous waste regulatory authority. Residue from fires extinguished with this material may be hazardous.

Contaminated packaging:  Do not re-use empty containers.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name    			: Bromates, inorganic, n.o.s.

UN N°			 			: 1450

ADR - Class 		 			: 5.1 Oxidizers

ADR - Group       		 			: II

Sea transport (IMDG) [English only]

Proper shipping name    	 		: Bromates, inorganic, n.o.s.

UN N° 			 	 		: 1450

IMO-IMDG - Class or division		:  5.1 Oxidizers

IMO-IMDG - Packing group	 		: II

Air transport (ICAO-IATA) [English only]

Proper shipping name 	  			: Bromates, inorganic, n.o.s)

UN N°		         	  			:  1450

IATA - Class or division  			: 5.1 Oxidizers

IATA - Packing group     			: II

Section 15: Other Regulatory Information

International Inventories

Components

CALCIUM BROMATE

Inventory - United States TSCA - Sect. 8(b) : Not Listed

Canada DSL Inventory List : Not Listed

EU EINECS List :233-278-9; BrHO3.1/2Ca

Korean KECL: KE-04484

U.S. regulations:

Components  : CALCIUM BROMATE

California Proposition 65  :  Not Listed

Massachusetts Right to Know List: Not Listed

New Jersey Right to Know List : Not Listed

Pennsylvania Right to Know List : Not Listed

Components : CALCIUM BROMATE

Florida substance List: Not Listed

Rhode Island Right to Know List: Not Listed

Illinois - Toxic Air Contaminants: Not Listed

Connecticut - Hazardous Air Pollutants: Not Listed

Components: CALCIUM BROMATE

SARA 313 Emission reporting/Toxic Release of Chemicals: Not Listed

CERCLA/SARA - Section 302 Extremely Haz: Not Listed

NTP: None

IARC: None

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.