MATERIAL SAFETY DATA SHEET

CALCIUM CARBONATE Precipitated 99% AR

MSDS CAS: 471-34-1

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: Calcium carbonate

CAS#: 471-34-1

Synonym: Not available.
Chemical Name: Calcium Carbonate.

Chemical Formula: CaCO3

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Calcium carbonate | 471-34-1 | 99% |

Toxicological Data on Ingredients: Calcium carbonate: ORAL (LD50): Acute: 6450 mg/kg [Rat].

Section 3: Hazards Identification

Potential Acute Health Effects:  Hazardous in case of eye contact (irritant). Slightly hazardous in case of skin contact (irritant), of ingestion, of inhalation.

Potential Chronic Health Effects:

CARCINOGENIC EFFECTS: Not available.

MUTAGENIC EFFECTS: Not available. TERATOGENIC EFFECTS: Not available.

DEVELOPMENTAL TOXICITY: Not available. The substance may be toxic to kidneys.

Repeated or prolonged exposure to the substance can produce target organs damage.

Section 4: First Aid Measures

Eye Contact: Check for and remove any contact lenses. In case of contact, immediately flush eyes with plenty of water for at least 15 minutes. Cold water may be used. WARM water MUST be used. Get medical attention.

Skin Contact: Wash with soap and water. Cover the irritated skin with an emollient. Get medical attention if irritation develops.

Serious Skin Contact: Not available.

Inhalation: If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical attention.

Serious Inhalation: Not available.

Ingestion: Do NOT induce vomiting unless directed to do so by medical personnel. Never give anything by mouth to an unconscious person. Loosen tight clothing such as a collar, tie, belt or waistband. Get medical attention if symptoms appear.

Serious Ingestion: Not available.

Section 5: Fire and Explosion Data

Flammability of the Product   : Non-flammable.

Auto-Ignition Temperature	: Not available

Flash Points			: Not available.

Section 5: Fire and Explosion Data (Continued)

Flammable Limits : Not available.

Products of Combustion : Not available.

Fire Hazards in Presence of Various Substances	 : Not available

Explosion Hazards in Presence of Various Substances :

Risks of explosion of the product in presence of mechanical impact: Not available.

Risks of explosion of the product in presence of static discharge: Not available.

Fire Fighting Media and Instructions : Not available.

Special Remarks on Fire Hazards : Will ignite and burn fiercely in contact with fluorine

Special Remarks on Explosion Hazards : When a mixture of calcium carbonate and magnesium is heated in a current of hydrogen, a violent explosion occurs.

Section 6: Accidental Release Measures

Small Spill:

Use appropriate tools to put the spilled solid in a convenient waste disposal container. Finish cleaning by spreading water on the contaminated surface and dispose of according to local and regional authority requirements.

Large Spill:

Use a shovel to put the material into a convenient waste disposal container. Finish cleaning by spreading water on the contaminated surface and allow to evacuate through the sanitary system. Be careful that the product is not present at a concentration level above TLV. Check TLV on the MSDS and with local authorities.

Section 7: Handling and Storage

Precautions:

Do not ingest. Do not breathe dust. Avoid contact with eyes. Wear suitable protective clothing. If ingested, seek medical advice immediately and show the container or the label. Keep away from incompatibles such as oxidizing agents, acids.

Storage:

Keep container tightly closed. Keep container in a cool, well-ventilated area. Hygroscopic.

Section 8: Exposure Controls/Personal Protection

Engineering Controls:

Use process enclosures, local exhaust ventilation, or other engineering controls to keep airborne levels below recommended exposure limits. If user operations generate dust, fume or mist, use ventilation to keep exposure to airborne contaminants below the exposure limit.

Personal Protection:

Splash goggles. Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Gloves.

Personal Protection in Case of a Large Spill:

Splash goggles. Full suit. Dust respirator. Boots. Gloves. A self contained breathing apparatus should be used to avoid inhalation of the product. Suggested protective clothing might not be sufficient; consult a specialist BEFORE handling this product.

Exposure Limits:

TWA: 10 (mg/m3) from ACGIH (TLV) [United States] Inhalation Total. TWA: 10 STEL: 20 (mg/m3) [Canada] Inhalation Total. TWA: 5 (mg/m3) from OSHA (PEL) [United States] Inhalation Respirable. TWA: 15 from OSHA (PEL) [United States] Inhalation Total. Consult local authorities for acceptable exposure limits.

Section 9: Physical and Chemical Properties

Physical state and appearance : Solid. (Powdered solid.)

Odor  			: Odorless.
Taste  			: Chalky

Molecular Weight 	: 100.09 g/mole

Color  			: White powder.

pH (1% soln/water) 	: 8

Boiling Point  		: Not available.

Melting Point 		: 825°C (1517°F)

Critical Temperature     : Not available.

Specific Gravity		: 2.8 (Water = 1)
Vapor Pressure		: Not applicable.

Vapor Density		: Not applicable.

Section 9: Physical and Chemical Properties (Continued)

Volatility			: Not available.

Odor Threshold		: Not available.

Water/Oil Dist. Coeff.	: Not available.

Ionicity (in Water)	: Not available.

Dispersion Properties	: Not available.
Solubility			: Water: Insoluble in water

Section 10: Stability and Reactivity Data

Stability			: The product is stable.

Instability Temperature: Not available.

Conditions of Instability: Incompatible Materials.

Incompatibility with various substances : Reactive with oxidizing agents, acids.

Corrosivity			: Non-corrosive in presence of glass.

Special Remarks on Reactivity  : Hygroscopic. Will ignite and burn fiercely in contact with fluorine. Incompatible with acids, alum, ammonium salts, mercury + hydrogen, aluminum and magnesium.
Special Remarks on Corrosivity : Not available.

Polymerization		: Will not occur.

Section 11: Toxicological Information

Routes of Entry : Inhalation. Ingestion.

Toxicity to Animals : Acute oral toxicity (LD50): 6450 mg/kg [Rat].

Chronic Effects on Humans : May cause damage to the following organs: kidneys.

Other Toxic Effects on Humans : Slightly hazardous in case of skin contact (irritant), of ingestion, of inhalation.

Special Remarks on Toxicity to Animals : Not available.

Special Remarks on Chronic Effects on Humans : Not available.

Section 11: Toxicological Information (Continued)

Special Remarks on other Toxic Effects on Humans : Acute Potential Health Effects: Skin: Causes skin irritation. Eyes: Dust causes eye irritation. Inhalation: Excessive inhalation causes respiratory tract and mucous membrane irritation. Low hazard for usual industrial handling. Ingestion: Ingestion of large amounts may cause gastrointestional tract disturbances with nausea and possibly constipation. Expected to be a low hazard for usual industrial handling. Chronic Potential Health Effects: Chronic ingestion may affect kidneys, and may cause hypercalcemia with alkalosis.

Section 12: Ecological Information

Ecotoxicity		: Not available.

BOD5 and COD	: Not available.

Products of Biodegradation : Possibly hazardous short term degradation products are not likely. However, long term degradation products may arise.

Toxicity of the Products of Biodegradation  : The product itself and its products of degradation are not toxic.

Special Remarks on the Products of Biodegradation : Not available.

Section 13: Disposal Considerations

Waste Disposal :

Waste must be disposed of in accordance with federal, state and local environmental control regulations.

Section 14: Transport Information

Land transport (ADR-RID)

General information    : Not regulated.

Sea transport (IMDG) [English only]

General information    : Not regulated.

Air transport (ICAO-IATA) [English only]

General information   : Not regulated.

Section 15: Other Regulatory Information

Federal and State Regulations: TSCA 8(b) inventory: Calcium carbonate.

Other Regulations:

EINECS:

This product is on the European Inventory of Existing Commercial Chemical Substances.

Other Classifications:

WHMIS (Canada):

Not controlled under WHMIS (Canada).
DSCL (EEC):

R36- Irritating to eyes. S2- Keep out of the reach of children. S46- If swallowed, seek medical advice immediately and show this container or label.

HMIS (U.S.A.):

Health Hazard: 2

Fire Hazard: 0

Reactivity: 0

Personal Protection: E

Section 15: Other Regulatory Information (Continued)

National Fire Protection Association (U.S.A.):

Health: 1

Flammability: 0

Reactivity: 0

Specific hazard:

Protective Equipment:

Gloves. Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Wear appropriate respirator when ventilation is inadequate. Splash goggles.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.