MATERIAL SAFETY DATA SHEET

CALCIUM HYPOPHOSPHITE 98%

MSDS CAS: 7789-79-9

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: Calcium hypophosphite

CAS#: 7789-79-9

Synonym: Calcium phosphinate.
Chemical Name: Not available.

Chemical Formula: H4CaO4P2

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Calcium hypophosphite | 7789-79-9 | 98% |

Section 3: Hazards Identification

Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008 [EU-GHS/CLP]

Flammable solids (Category 1)

Classification according to EU Directives 67/548/EEC or 1999/45/EC

Highly flammable.

Label elements

Labelling according Regulation (EC) No 1272/2008 [CLP]
Hazard statement(s) : H228 Flammable solid.

Precautionary statement(s) : P210 Keep away from heat/sparks/open flames/hot surfaces. - No smoking.

Supplemental Hazard Statements : none.

According to European Directive 67/548/EEC as amended.

R-phrase(s) : R11 Highly flammable.

S-phrase(s) :  none.

Other hazards : none.

Section 4: First Aid Measures

Description of first aid measures:

General advice 		: Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled 			: If breathed in, move person into fresh air. If not breathing, give artificial 					   respiration. Consult a physician.

In case of skin contact 	: Wash off with soap and plenty of water. Consult a physician.

In case of eye contact 	: Flush eyes with water as a precaution.

If swallowed 		: Do NOT induce vomiting. Never give anything by mouth to an unconscious 					   person. Rinse mouth with water. Consult a physician.

Most important symptoms and effects, both acute and delayed.

Indication of any immediate medical attention and special treatment needed : no data available.

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media:

Use extinguishing measures that are appropriate to local circumstances and the surrounding environment.

Special hazards arising from the substance or mixture:

Oxides of phosphorus, Calcium oxide

Advice for firefighters:

Wear self contained breathing apparatus for fire fighting if necessary.

Further information:

Use water spray to cool unopened containers. The product itself does not burn.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures :

Avoid dust formation. Avoid breathing vapors, mist or gas. Ensure adequate ventilation. Remove all

sources of ignition. Evacuate personnel to safe areas.

Environmental precautions:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Methods and materials for containment and cleaning up:

Sweep up and shovel. Contain spillage, and then collect with an electrically protected vacuum cleaner or

by wet-brushing and place in container for disposal according to local regulations (see section 13). Keep

in suitable, closed containers for disposal. Contain spillage, pick up with an electrically protected vacuum

cleaner or by wet-brushing and transfer to a container for disposal according to local regulations (see

section 13).

Section 7: Handling and Storage

Precautions for safe handling

Avoid formation of dust and aerosols.

Provide appropriate exhaust ventilation at places where dust is formed. Keep away from sources of ignition

- No smoking. Take measures to prevent the build up of electrostatic charge.

Section 7: Handling and Storage (Continued)

Conditions for safe storage, including any incompatibilities:

Store in cool place. Keep container tightly closed in a dry and well-ventilated place.

Specific end uses : no data available.

Section 8: Exposure Controls/Personal Protection

Control parameters:

Components with workplace control parameters.

Exposure controls:

Appropriate engineering controls

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and

at the end of workday.

Personal protective equipment

Eye/face protection:

Safety glasses with side-shields conforming to EN166 Use equipment for eye protection tested

and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

Skin protection:

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique

(without touching glove's outer surface) to avoid skin contact with this product. Dispose of

contaminated gloves after use in accordance with applicable laws and good laboratory practices.

Wash and dry hands:

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and

the standard EN 374 derived from it.

Body Protection:

Flame retardant antistatic protective clothing, The type of protective equipment must be selected

according to the concentration and amount of the dangerous substance at the specific workplace.

Section 8: Exposure Controls/Personal Protection (Continued)

Respiratory protection:

Where risk assessment shows air-purifying respirators are appropriate use a full-face particle

respirator type N100 (US) or type P3 (EN 143) respirator cartridges as a backup to engineering

controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use

respirators and components tested and approved under appropriate government standards such

as NIOSH (US) or CEN (EU).

Section 9: Physical and Chemical Properties

Appearance Form 				: powder

Colour 						: white

Odour 						: no data available

Odour Threshold 				: no data available

Molecular Weight 				: 170,05 g/mol
pH 							: no data available

Melting point/freezing  point 			: no data available

Initial boiling point 				: no data available

Flash point 						: not applicable

Evaporation rate 					: no data available

Flammability (solid, gas) 			: The substance or mixture is a flammable solid with the

category 1.

Upper/lower  flammability

or explosive limits 				: no data available

Vapour pressure 					: no data available

Vapour density 					: no data available

Relative density 					: no data available

Water solubility 					: no data available

Partition coefficient :octanol/water		: no data available

Autoignition  temperature 			: no data available

Decomposition  temperature 			: no data available

Viscosity 						: no data available

Explosive properties 				: no data available

Oxidizing properties 				: no data available

Section 10: Stability and Reactivity Data

Reactivity : no data available

Chemical stability : no data available

Possibility of hazardous reactions : no data available

Conditions to avoid : Heat, flames and sparks. Extremes of temperature and direct sunlight.

Incompatible materials : Oxidizing agents

Hazardous decomposition products

Other decomposition products : no data available.

Section 11: Toxicological Information

Information on toxicological effects

Acute toxicity : no data available.

Skin corrosion/irritation : no data available.

Serious eye damage/eye irritation : no data available.

Respiratory or skin sensitization : no data available.

Germ cell mutagenicity : no data available.

Carcinogenicity

IARC: No component of this product present at levels greater than or equal to 0.1% is identified as

probable, possible or confirmed human carcinogen by IARC.

Reproductive toxicity : no data available.

Specific target organ toxicity - single exposure : no data available.

Specific target organ toxicity - repeated exposure : no data available

Aspiration hazard : no data available.

Section 11: Toxicological Information (Continued)

Potential health effects

Inhalation 	: May be harmful if inhaled. May cause respiratory tract irritation.

Ingestion 	: May be harmful if swallowed.

Skin 		: May be harmful if absorbed through skin. May cause skin irritation.

Eyes 		:  May cause eye irritation.

Additional Information : RTECS: Not available.

Section 12: Ecological Information

Toxicity : no data available.

Persistence and degradability : no data available.

Bioaccumulative potential : no data available.

Mobility in soil : no data available.

Results of PBT and vPvB assessment : no data available.

Other adverse effects : no data available.

Section 13: Disposal Considerations

Waste treatment methods

Product

Burn in a chemical incinerator equipped with an afterburner and scrubber but exert extra care in igniting

as this material is highly flammable. Offer surplus and non-recyclable solutions to a licensed disposal

company. Contact a licensed professional waste disposal service to dispose of this material.

Contaminated packaging

Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name ADR/RID		: FLAMMABLE SOLID, INORGANIC, N.O.S. (Calcium

phosphinate)

UN number ADR/RID				: 3178

Transport hazard class(es) ADR/RID 	: 4.1

Packaging group ADR/RID 			: II

Environmental hazards ADR/RID		: No

Sea transport (IMDG) [English only]

Proper shipping name IMDG 			: FLAMMABLE SOLID, INORGANIC, N.O.S. (Calcium

phosphinate)

UN number IMDG 				: 3178

Transport hazard class(es) IMDG 		: 4.1

Packaging group IMDG 			: II

Air transport (ICAO-IATA) [English only]

Proper shipping name IATA 			: FLAMMABLE SOLID, INORGANIC, N.O.S. (Calcium

phosphinate)

UN number IATA 				: 3178

Transport hazard class(es) IATA 		: 4.1

Packaging group IATA 				: II

Environmental hazards IATA 		: No

Section 15: Other Regulatory Information

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

Safety, health and environmental regulations/legislation specific for the substance or mixture

: No Data Available.

Chemical Safety Assessment

: No Data Available.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.