MATERIAL SAFETY DATA SHEET

CALCIUM SULPHIDE 99%

MSDS CAS: 20548-54-3

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: Calcium Sulfide

CAS#: 20548-54-3

Synonym:

Chemical Name: Calcium Sulfide
 Chemical Formula: CaS

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Calcium Sulfide | 20548-54-3 | 100 |

Toxicological Data on Ingredients: Calcium Sulfide LD50: Not available. LC50: Not available.

Section 3: Hazards Identification

Potential Acute Health Effects:

Hazardous in case of skin contact (irritant), of eye contact (irritant), of ingestion, of inhalation (lung irritant).

Potential Chronic Health Effects:

CARCINOGENIC EFFECTS: Not available.

MUTAGENIC EFFECTS: Not available.

TERATOGENIC EFFECTS: Not available.

DEVELOPMENTAL TOXICITY: Not available.

Repeated or prolonged exposure is not known to aggravate medical condition.

Section 4: First Aid Measures

Eye Contact:

Check for and remove any contact lenses. In case of contact, immediately flush eyes with plenty of water for at least 15 minutes. Cold water may be used. Get medical attention.

Skin Contact:

In case of contact, immediately flush skin with plenty of water. Cover the irritated skin with an emollient. Remove contaminated clothing and shoes. Cold water may be used. Wash clothing before reuse. Thoroughly clean shoes before reuse. Get medical attention.

Serious Skin Contact:

Wash with a disinfectant soap and cover the contaminated skin with an anti-bacterial cream. Seek medical attention.

Inhalation:

If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical attention.

Serious Inhalation:  Not available.

Ingestion:

Do NOT induce vomiting unless directed to do so by medical personnel. Never give anything by mouth to an unconscious person. If large quantities of this material are swallowed, call a physician immediately. Loosen tight clothing such as a collar, tie, belt or waistband.

Serious Ingestion:  Not available.

Section 5: Fire and Explosion Data

Flammability of the Product: May be combustible at high temperature.

Auto-Ignition Temperature: Not applicable.

Flash Points: Not applicable.

Flammable Limits: Not applicable.

Products of Combustion: Some metallic oxides.

Fire Hazards in Presence of Various Substances:

Slightly flammable to flammable in presence of open flames and sparks, of heat. Non-flammable in presence of shocks.

Explosion Hazards in Presence of Various Substances:

Risks of explosion of the product in presence of mechanical impact: Not available.

Risks of explosion of the product in presence of static discharge: Not available.

Fire Fighting Media and Instructions:

SMALL FIRE: Use DRY chemical powder.

LARGE FIRE: Use water spray, fog or foam. Do not use water jet.

Special Remarks on Fire Hazards :

Flammable gas is emitted upon heating. When heated to decomposition it emits highly toxic fumes of Hydrogen Sulfide.

Special Remarks on Explosion Hazards:  Not available.

Section 6: Accidental Release Measures

Small Spill:

Use appropriate tools to put the spilled solid in a convenient waste disposal container. Finish  cleaning by spreading water on the contaminated surface and dispose of according to local and regional authority requirements.

Section 6: Accidental Release Measures (Continued)

Large Spill:

Use a shovel to put the material into a convenient waste disposal container. Finish cleaning by spreading water on the contaminated surface and allow to evacuate through the sanitary system. Be careful that the product is not present at a concentration level above TLV. Check TLV on the MSDS and with local authorities.

Section 7: Handling and Storage

Precautions:

Keep away from heat. Keep away from sources of ignition. Empty containers pose a fire risk, evaporate the residue under a fume hood. Ground all equipment containing material. Do not breathe dust. Wear suitable protective clothing. In case of insufficient ventilation, wear suitable respiratory equipment. If you feel unwell, seek medical attention and show the label when possible. Avoid contact with skin and eyes. Keep away from incompatibles such as oxidizing agents, acids.

Storage: 
Keep container tightly closed. Keep container in a cool, well-ventilated area.

Section 8: Exposure Controls/Personal Protection

Engineering Controls:

Use process enclosures, local exhaust ventilation, or other engineering controls to keep airborne levels below recommended exposure limits. If user operations generate dust, fume or mist, use ventilation to keep exposure to airborne contaminants below the exposure limit.

Personal Protection:

Splash goggles. Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Gloves.

Personal Protection in Case of a Large Spill:

Splash goggles. Full suit. Dust respirator. Boots. Gloves. A self contained breathing apparatus should be used to avoid inhalation of the product. Suggested protective clothing might not be sufficient; consult a specialist BEFORE handling this product.

Section 8: Exposure Controls/Personal Protection (Continued)

Exposure Limits:

STEL (as Hydrogen Sulfide): 50; CEIL (as Hydrogen Sulfide): 20 (ppm) from OSHA (PEL) [United States] TWA (as Hydrogen Sulfide); 10 STEL (as Hydrogen Sulfide): 15 (ppm) from ACGIH (TLV )[United States] Consult local authorities for acceptable exposure limits.

Section 9: Physical and Chemical Properties

Physical state and appearance: Solid. (Crystals solid.)

Odor					: Not available.

Taste					: Not available.

Molecular Weight		: 72.14 g/mole.

Color					: White.

pH (1% soln/water)		: Not available.

Boiling Point			: Not available.

Melting Point			: Decomposes.
Critical Temperature		: Not available.

Specific Gravity			: Not available.

Vapor Pressure			: Not applicable.

Vapor Density			: Not available.

Volatility				: Not available.

Odor Threshold			: Not available.

Water/Oil Dist. Coeff.		: Not available.

Ionicity (in Water)		: Not available.

Dispersion Properties		: See solubility in water.

Solubility				: Partially soluble in cold water.

Section 10: Stability and Reactivity Data

Stability: The product is stable.

Instability Temperature: Not available.

Section 10: Stability and Reactivity Data (Continued)

Conditions of Instability:

Excess heat, moisture/water,  incompatible materials.

Incompatibility with various substances :

Reactive with oxidizing  agents, acids.  Slightly  reactive to reactive with moisture.

Corrosivity	: Not available.

Special Remarks on Reactivity:

Moisture sensitive  Air sensitive. Slow decomposition especially when wet. It gives off vapor of Hydrogen sulfide. It decomposes in water to form Hydrogen Sulfide. It is incompatible with Lead Oxide, Potassium Chlorate, and Potassium Nitrate.

Special Remarks on  Corrosivity : Not available.

Polymerization:  Will not occur.

Section 11: Toxicological Information

Routes of Entry:  Inhalation. Ingestion.

Toxicity to Animals: LD50 LD50: Not available. LC50: Not available.

Chronic Effects on Humans: Not available.

Other Toxic Effects on Humans: Hazardous in case of skin contact (irritant), of ingestion, of inhalation (lung irritant).

Special Remarks on Toxicity to Animals: Not available.

Special Remarks on Chronic Effects on Humans: Not available.

Section 11: Toxicological Information (Continued)

Special Remarks on other Toxic Effects on Humans:

Acute Potential Health Effects: Skin: It causes skin irritation. Eyes: It can cause eye irritation, conjunctivitis, lacrimation, corneal opacity, photophobia, blurred vision. Inhalation: It causes respiratory tract and mucous membrane irritation with coughing, rhinitis, bronchitis due to emission of Hydrogen vapors. Other symptoms of inhalation of Hydrogen sulfide vapors headache, nausea, vomiting, diarrhea, central nervous system

depression (vertigo, ataxia, drowsiness, hyperactivity, incoordinationamnesia, dizziness, muscle cramps, weakness), palpitations, tachycardia, hypotension, cyanosis. Ingestion: Causes gastrointestinal tract irritation with vomiting, diarrhea. Calcium Sulfide decomposes in the body to form Hydrogen Sulfide. Hydrogen sulfide can cause the rapid onset of coma, convulsions, and death. It can also cause nerve and heart damage with sublethal exposure. Hydrogen Sulfide depresses the respiratory control center of the brain. Symptoms are

similar to acute inhalation. Chronic Potential Health Effects: Repeated or chronic exposures to Hydrogen Sulfide can cause respiratory tract and eye irritation, corneal opacity, bronchitis, pulmonary edema, nausea, headache, and signs of central nervous system depression (weakness, vertigo, amnesia, sleepiness, dizziness, loss of coordination). Repeated or chronic contact with skin can cause defattiang and dermatitis.

Section 12: Ecological Information

Ecotoxicity: Not available.

BOD5 and COD: Not available.

Products of Biodegradation:

Possibly hazardous short term degradation products are not likely. However, long term degradation products may arise.

Toxicity of the Products of Biodegradation: The product itself and its products of degradation are not toxic.

Special Remarks on the Products of Biodegradation: Not available.

Section 13: Disposal Considerations

Waste Disposal:

Waste must be disposed of in accordance with federal, state and local environmental control regulations.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name : FLAMMABLE SOLID, TOXIC, INORGANIC, N.O.S. (Calcium sulphide)

UN N° : 3179

H.I. nr :

ADR - Class : 4.1 (6.1)

Labelling - Transport : 4.1 (6.1) : Corrosive substance.

Sea transport (IMDG) [English only]

Proper shipping name : FLAMMABLE SOLID, TOXIC, INORGANIC, N.O.S. (Calcium sulphide)

UN N° : 3179

IMO-IMDG - Class or division : 4.1 (6.1) : Corrosive substance.

IMO-IMDG - Packing group : II

Air transport (ICAO-IATA) [English only]

Proper shipping name : FLAMMABLE SOLID, TOXIC, INORGANIC, N.O.S. (Calcium sulphide)

UN N° : 3179

IATA - Class or division : 4.1 (6.1) : Corrosive substance.

IATA - Packing group : II

Section 15: Other Regulatory Information

Federal and State Regulations :

TSCA 8(b) inventory: Calcium Sulfide.

Other Regulations:

EINECS: This product is on the European Inventory of Existing Commercial Chemical Substances.

Other Classifications:

WHMIS (Canada):

Not controlled under WHMIS (Canada).

DSCL (EEC):

R31- Contact with acids liberates toxic gas. R36/37/38- Irritating to eyes, respiratory system and skin. S28- After contact with skin, wash immediately with plenty of [***]

HMIS (U.S.A.):

Health Hazard: 2

Fire Hazard: 1

Reactivity: 1

Personal Protection: E

Protective Equipment:

Gloves.  Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Splash goggles.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.