MATERIAL SAFETY DATA SHEET

CHLORO ACETONE

(For Synthesis)

MSDS CAS: 78-95-5

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: Chloroacetone
CAS#:   78-95-5

Synonym: Not Applicable.

Chemical Name: 	Not Applicable.

Chemical Formula: C3H5ClO

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Chloroacetone | 78-95-5 | 100 |

Section 3: Hazards Identification

Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008:

Flammable liquids (Category 3), H226

Acute toxicity, Oral (Category 3), H301

Acute toxicity, Inhalation (Category 2), H330

Acute toxicity, Dermal (Category 2), H310

Skin irritation (Category 2), H315

Eye irritation (Category 2), H319

Specific target organ toxicity - single exposure (Category 3), H335

Acute aquatic toxicity (Category 1), H400

Classification according to EU Directives 67/548/EEC or 1999/45/EC:

T+, N Very toxic, Dangerous for the Environment

Label elements:

Labelling according Regulation (EC) No 1272/2008

Signal word Danger

Hazard statement(s):

H226    :   Flammable liquid and vapour.

H301    :   Toxic if swallowed.

H310 + H330 :  Fatal in contact with skin or if inhaled

H315    :   Causes skin irritation.

H319    :   Causes serious eye irritation.

H335    :   May cause respiratory irritation.

H400    :   Very toxic to aquatic life.

Precautionary statement(s):

P260    : Do not breathe dust/ fume/ gas/ mist/ vapours/ spray.

P273    : Avoid release to the environment.

P280    : Wear protective gloves/ protective clothing.

P284    : Wear respiratory protection.

P301 + P310 IF SWALLOWED : Immediately call a POISON CENTER or doctor/physician.

P302 + P350 IF ON SKIN : Gently wash with plenty of soap and water.
Supplemental Hazard Statements  :   none

Section 4: First Aid Measures

Description of first aid measures

General advice:

Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled:

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

In case of skin contact:

Wash off with soap and plenty of water. Take victim immediately to hospital. Consult a physician.

In case of eye contact:

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed:

Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Most important symptoms and effects, both acute and delayed:

The most important known symptoms and effects are described in the labelling (see section 2.2) and/or in section 11

Indication of any immediate medical attention and special treatment needed: No data available.

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media: Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special hazards arising from the substance or mixture: Carbon oxides, Hydrogen chloride gas.

Advice for firefighters: Wear self contained breathing apparatus for fire fighting if necessary.

Further information: Use water spray to cool unopened containers.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures

Wear respiratory protection. Avoid breathing vapours, mist or gas. Ensure adequate ventilation. Remove

all sources of ignition. Evacuate personnel to safe areas. Beware of vapours accumulating to form explosive concentrations. Vapours can accumulate in low areas.

Section 6: Accidental Release Measures (Continued)

Environmental precautions:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains. Discharge into the environment must be avoided.

Methods and materials for containment and cleaning up:

Contain spillage, and then collect with an electrically protected vacuum cleaner or by wet-brushing and place in container for disposal according to local regulations.

Section 7: Handling and Storage

Precautions for safe handling:

Avoid contact with skin and eyes. Avoid inhalation of vapour or mist.

Keep away from sources of ignition - No smoking. Take measures to prevent the build up of electrostatic

charge.

Conditions for safe storage, including any incompatibilities:

Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Containers which are

opened must be carefully resealed and kept upright to prevent leakage.

Recommended storage temperature: 2 - 8 °C

Light sensitive.

Specific end use(s):

Apart from the uses mentioned in section 1.2 no other specific uses are stipulated.

Section 8: Exposure Controls/Personal Protection

Control parameters

Components with workplace control parameters

Exposure controls

Appropriate engineering controls:

Avoid contact with skin, eyes and clothing. Wash hands before breaks and immediately after handling

the product.

Section 8: Exposure Controls/Personal Protection (Continued)

Personal protective equipment

Eye/face protection:

Face shield and safety glasses Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

Skin protection:

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique

(without touching glove's outer surface) to avoid skin contact with this product. Dispose of

contaminated gloves after use in accordance with applicable laws and good laboratory practices.

Wash and dry hands.

Body Protection:

Complete suit protecting against chemicals, Flame retardant antistatic protective clothing, The type

of protective equipment must be selected according to the concentration and amount of the

dangerous substance at the specific workplace.

Respiratory protection:

Where risk assessment shows air-purifying respirators are appropriate use a full-face respirator

with multi-purpose combination (US) or type ABEK (EN 14387) respirator cartridges as a backup

to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air

respirator. Use respirators and components tested and approved under appropriate government

standards such as NIOSH (US) or CEN (EU).

Control of environmental exposure:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains. Discharge into

the environment must be avoided.

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Appearance Form  : liquid

Odour Threshold   : unpleasant

Molecular Weight  : 92,52 g/mol
pH                            : no data available

Melting point          : -44,5 °C

Section 9: Physical and Chemical Properties (Continued)

Initial boiling point		: 120 °C

Flash point 				: 35 °C - closed cup.

Evapouration rate 		: no data available.

Flammability (solid, gas) 	: no data available.

Explosive limits 			: no data available.

Vapour pressure 			: no data available.

Vapour density 			: no data available.

Relative density 			: 1,162 g/mL at 25 °C

Water solubility 			: no data available.

Partition coefficient 		: no data available.

Auto-ignition temperature 	: no data available.

Decomposition temperature 	: no data available.

Viscosity 				: no data available.

Explosive properties 		: no data available.

Oxidizing properties 		: no data available.

Section 10: Stability and Reactivity Data

Reactivity: no data available.

Chemical stability:

Stable under recommended storage conditions.

Contains the following stabiliser(s):

Calcium carbonate (0,5 %)

Possibility of hazardous reactions: no data available.

Conditions to avoid: Heat, flames and sparks.

Incompatible materials: Strong bases.

Hazardous decomposition products:

Other decomposition products - no data available.

Section 11: Toxicological Information

Information on toxicological effects

Acute toxicity:

LD50 Oral - rat - 100 mg/kg

Remarks: Behavioral:Somnolence (general depressed activity). Behavioral:Ataxia. Skin and Appendages:

Other: Hair.

LD50 Dermal - rabbit - 141 mg/kg

Remarks: Behavioral:Somnolence (general depressed activity). Behavioral:Ataxia. Skin and Appendages:

Other: Hair.

Skin corrosion/irritation:  no data available.

Serious eye damage/eye irritation:  no data available.

Respiratory or skin sensitisation: no data available.

Germ cell mutagenicity: no data available.

Carcinogenicity:

IARC: No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

Reproductive toxicity: no data available.

Specific target organ toxicity - single exposure

Inhalation - May cause respiratory irritation.

Specific target organ toxicity - repeated exposure:  no data available.

Aspiration hazard: no data available.

Section 12: Ecological Information

Toxicity: no data available.

Persistence and degradability : no data available.

Section 12: Ecological Information (Continued)

Bioaccumulative potential : no data available.

Mobility in soil : no data available.

Results of PBT and vPvB assessment:

PBT/vPvB assessment not available as chemical safety assessment not required/not conducted.

Other adverse effects :  Very toxic to aquatic life.

Section 13: Disposal Considerations

Waste treatment methods

Product:

Burn in a chemical incinerator equipped with an afterburner and scrubber but exert extra care in igniting as this material is highly flammable. Offer surplus and non-recyclable solutions to a licensed disposal company.

Contaminated packaging: Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name    : CHLOROACETONE, STABILIZED

UN N°			 : 1695

ADR - Class 		 : 6.1 (3, 8)

Labelling - Transport 	 : 6.1 : Not permitted for transport

ADR- Packaging group   : I

Sea transport (IMDG) [English only]

Proper shipping name    : CHLOROACETONE, STABILIZED

UN N° 			 : 1695

IMO-IMDG - Class or division  : 6.1 (3, 8): Not permitted for transport
IMO-IMDG - Packing group : I

Section 14: Transport Information (Continued)

Air transport (ICAO-IATA) [English only]

Proper shipping name 	  : Chloroacetone, stabilized

UN N°		         	  :  1695

IATA - Class or division  : 6.1 (3, 8)

IATA - Packing group     : I

Section 15: Other Regulatory Information

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

Safety, health and environmental regulations/legislation specific for the substance or mixture: no data available.

Chemical Safety Assessment:

For this product a chemical safety assessment was not carried out

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.