MATERIAL SAFETY DATA SHEET

3-CHLORO BENZALDEHYDE 98%

(For Synthesis) (Meta-Chloro Benzaldehyde)

MSDS CAS: 587-04-2

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: 3-Chlorobenzaldehyde

CAS#:   587-04-2

Synonym: Meta-Chlorobenzaldehyde

Chemical Name: 	Not available.

Chemical Formula:  C7H5ClO

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| 3-Chlorobenzaldehyde | 587-04-2 | 100 |

Section 3: Hazards Identification

Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008 [EU-GHS/CLP]

Skin irritation (Category 2)

Eye irritation (Category 2)

Specific target organ toxicity - single exposure (Category 3)

Classification according to EU Directives 67/548/EEC or 1999/45/EC

Irritating to eyes, respiratory system and skin.

Label elements

Labelling according Regulation (EC) No 1272/2008 [CLP]

Signal word : Warning

Hazard statement(s)

H315 : Causes skin irritation.

H319 : Causes serious eye irritation.

H335 : May cause respiratory irritation.

Precautionary statement(s)

P261 : Avoid breathing dust/ fume/ gas/ mist/ vapours/ spray.

P305 + P351 + P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

Supplemental Hazard Statements :None

According to European Directive 67/548/EEC as amended.

R-phrase(s)

R36/37/38 : Irritating to eyes, respiratory system and skin.

S-phrase(s)

S26 : In case of contact with eyes, rinse immediately with plenty of water and seek medical advice.

S36 : Wear suitable protective clothing.

Section 4: First Aid Measures

Description of first aid measures

General advice:

Consult a physician. Show this safety data sheet to the doctor in attendance.

Section 4: First Aid Measures (Continued)

If inhaled:

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

In case of skin contact: Wash off with soap and plenty of water. Consult a physician.

In case of eye contact:

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed:

Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth with water.

Consult a physician.

Most important symptoms and effects, both acute and delayed:

To the best of our knowledge, the chemical, physical, and toxicological properties have not been thoroughly

investigated.

Indication of immediate medical attention and special treatment needed:  No data available.

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media:

For small (incipient) fires, use media such as "alcohol" foam, dry chemical, or carbon dioxide. For large fires,

apply water from as far as possible. Use very large quantities (flooding) of water applied as a mist or spray;

solid streams of water may be ineffective. Cool all affected containers with flooding quantities of water.

Special hazards arising from the substance or mixture:

Carbon oxides, Hydrogen chloride gas.

Precautions for fire-fighters:

Wear self contained breathing apparatus for fire fighting if necessary.

Further information:

Use water spray to cool unopened containers.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures

Use personal protective equipment. Avoid breathing vapors, mist or gas. Ensure adequate ventilation.

Remove all sources of ignition. Evacuate personnel to safe areas. Beware of vapours accumulating to form

explosive concentrations. Vapours can accumulate in low areas.

Environmental precautions:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Methods and materials for containment and cleaning up:

Contain spillage, and then collect with an electrically protected vacuum cleaner or by wet-brushing and place in container for disposal according to local regulations (see section 13). Keep in suitable, closed containers for disposal.

Section 7: Handling and Storage

Precautions for safe handling:

Avoid contact with skin and eyes. Avoid inhalation of vapour or mist.

Keep away from sources of ignition - No smoking.Take measures to prevent the build up of electrostatic charge.

Conditions for safe storage, including any incompatibilities:

Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Containers which are opened must be carefully resealed and kept upright to prevent leakage.

Section 8: Exposure Controls/Personal Protection

Control parameters

Components with workplace control parameters

Exposure controls

Appropriate engineering controls

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at

the end of workday.

Personal protective equipment

Eye/face protection:

Safety glasses with side-shields conforming to EN166 Use equipment for eye protection tested and

approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

Section 8: Exposure Controls/Personal Protection (Continued)

Skin protection:

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique

(without touching glove's outer surface) to avoid skin contact with this product. Dispose of

contaminated gloves after use in accordance with applicable laws and good laboratory practices.

Wash and dry hands.

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the

standard EN 374 derived from it.

Body Protection:

impervious clothing, The type of protective equipment must be selected according to the

concentration and amount of the dangerous substance at the specific workplace.

Respiratory protection:

Where risk assessment shows air-purifying respirators are appropriate use a full-face respirator with multi-purpose combination (US) or type ABEK (EN 14387) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Section 9: Physical and Chemical Properties

Appearance Form 		: Liquid

Colour 				: Light yellow

Odour 				: No data available

Odour 				: Threshold no data available

Ph 					: No data available

Melting point 			: 9 - 12 °C - lit.

Boiling range 			: 213 - 214 °C - lit.

Flash point 				: 88 °C - closed cup

Evaporation rate 			: No data available

Flammability (solid, gas)	: No data available

Explosive limits 			: no data available

Vapour pressure 			: no data available

Vapour density 			: no data available

Section 9: Physical and Chemical Properties (Continued)

Relative density 			: 1,241 g/cm3 at 25 °C

Water solubility 			: no data available

Partition coefficient 		: noctanol/water :no data available

Autoignition temperature 	: no data available

Decomposition temperature 	: no data available

Viscosity 				: no data available

Explosive properties		: no data available

Oxidizing properties 		: no data available.

Section 10: Stability and Reactivity Data

Reactivity:  No data available.

Chemical stability: No data available.

Possibility of hazardous reactions: No data available.

Conditions to avoid: Heat, flames and sparks.

Incompatible materials: Strong bases, Strong oxidizing agents, Strong reducing agents.

Hazardous decomposition products: Other decomposition products - no data available

Section 11: Toxicological Information

Information on toxicological effects

Acute toxicity: no data available

Skin corrosion/irritation: no data available

Serious eye damage/eye irritation: no data available

Respiratory or skin sensitization: no data available

Germ cell mutagenicity: no data available

Section 11: Toxicological Information (Continued)

Carcinogenicity

IARC: No component of this product present at levels greater than or equal to 0.1% is identified as

probable, possible or confirmed human carcinogen by IARC.

Reproductive toxicity: no data available

Specific target organ toxicity - single exposure

Inhalation - May cause respiratory irritation.

Specific target organ toxicity - repeated exposure: no data available

Aspiration hazard: no data available

Potential health effects

Inhalation : May be harmful if inhaled. Causes respiratory tract irritation.

Ingestion :May be harmful if swallowed.

Skin : May be harmful if absorbed through skin. Causes skin irritation.

Eyes : Causes serious eye irritation.

Signs and Symptoms of Exposure:

To the best of our knowledge, the chemical, physical, and toxicological properties have not been thoroughly

investigated.

Section 12: Ecological Information

Toxicity: No data available

Persistence and degradability: No data available

Bioaccumulative potential: No data available

Mobility in soil: No data available

Results of PBT and vPvB assessment: No data available

Section 13: Disposal Considerations

Waste treatment methods

Product:

This combustible material may be burned in a chemical incinerator equipped with an afterburner and scrubber. Offer surplus and non-recyclable solutions to a licensed disposal company. Contact a licensed professional waste disposal service to dispose of this material.

Section 14: Transport Information

Land transport (ADR-RID)

General information : Not regulated.

Sea transport (IMDG) [English only]

General information : Not regulated.

Air transport (ICAO-IATA) [English only]

General information : Not regulated.

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture

No data available.

Chemical Safety Assessment: No data available.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.