MATERIAL SAFETY DATA SHEET

1-CHLORO ETHYL CHLOROFORMATE 98%

MSDS CAS: 50893-53-3

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: 1-CHLORO ETHYL CHLOROFORMATE

CAS#:   50893-53-3

Synonym: Not available.
Chemical Name: Not available.

Chemical Formula:  C3H4Cl2O2

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| 1-Chloro ethyl chloroformate | 50893-53-3 | 100 |

Section 3: Hazards Identification

Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008

Flammable liquids (Category 3), H226

Acute toxicity, Oral (Category 4), H302

Acute toxicity, Inhalation (Category 3), H331

Skin corrosion (Category 1B), H314

Label elements

Labelling according Regulation (EC) No 1272/2008

Hazard statement(s):

H226 Flammable liquid and vapour.

H302 Harmful if swallowed.

H314 Causes severe skin burns and eye damage.

H331 Toxic if inhaled.

Precautionary statement(s):

P261 Avoid breathing vapours.

P280 Wear protective gloves/ protective clothing/ eye protection/ face protection.

P305 + P351 + P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

P310 Immediately call a POISON CENTER or doctor/ physician.

Section 4: First Aid Measures

Description of first aid measures

General advice: Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled:

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

In case of skin contact:

Take off contaminated clothing and shoes immediately. Wash off with soap and plenty of water. Take victim immediately to hospital. Consult a physician.

In case of eye contact: Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed: Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Section 4: First Aid Measures (Continued)

Most important symptoms and effects, both acute and delayed:

The most important known symptoms and effects are described in the labelling (see section 2.2).

Indication of any immediate medical attention and special treatment needed:

No data available.

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media

For small (incipient) fires, use media such as "alcohol" foam, dry chemical, or carbon dioxide. For large fires, apply water from as far as possible. Use very large quantities (flooding) of water applied as a mist or spray; solid streams of water may be ineffective. Cool all affected containers with flooding quantities of water.

Special hazards arising from the substance or mixture:

Hydrogen chloride gas.

Advice for firefighters:

Wear self contained breathing apparatus for fire fighting if necessary.

Further information:

Use water spray to cool unopened containers.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures

Wear respiratory protection. Avoid breathing vapours, mist or gas. Ensure adequate ventilation. Remove all sources of ignition. Evacuate personnel to safe areas. Beware of vapours accumulating to form explosive concentrations. Vapours can accumulate in low areas.

Environmental precautions:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Methods and materials for containment and cleaning up:

Contain spillage, and then collect with an electrically protected vacuum cleaner or by wet-brushing and place in container for disposal according to local regulations (see section 13).

Section 7: Handling and Storage

Precautions for safe handling:

Avoid contact with skin and eyes. Avoid inhalation of vapour or mist. Keep away from sources of ignition - No smoking.Take measures to prevent the build up of electrostatic charge.

Conditions for safe storage, including any incompatibilities:

Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Containers which are opened must be carefully resealed and kept upright to prevent leakage.

Recommended storage temperature: 2 - 8 °C Store under inert gas. Moisture sensitive. Vent periodically. Handle and open container with care. Exposure to moisture.

Specific end use(s):

Apart from the uses mentioned in section 1.2 no other specific uses are stipulated.

Section 8: Exposure Controls/Personal Protection

Control parameters

Components with workplace control parameters

Exposure controls

Appropriate engineering controls:

Avoid contact with skin, eyes and clothing. Wash hands before breaks and immediately after handling the product.

Personal protective equipment

Eye/face protection

Tightly fitting safety goggles. Faceshield (8-inch minimum). Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

Skin protection

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices. Wash and dry hands. The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

Body Protection:

Complete suit protecting against chemicals, Flame retardant antistatic protective clothing, The type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace.

Section 8: Exposure Controls/Personal Protection (Continued)

Respiratory protection:

Where risk assessment shows air-purifying respirators are appropriate use a full-face respirator with multi-purpose combination (US) or type ABEK (EN 14387) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU)

Control of environmental exposure:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Section 9: Physical and Chemical Properties

Appearance Form			: Liquid.

Colour					: Colourless.

Odour 					: No data available.

Odour Threshold 			: No data available.

pH 						: No data available.

Melting point 				: No data available.

Boiling point 				: 118 - 119 °C - lit.

Flash point 					: 40,5 °C - closed cup.

Evapouration rate 			: No data available.

Flammability (solid, gas) 		: No data available.
Explosive limits 				: No data available.

Vapour pressure 				: 359,2 hPa at 55 °C,

224,0 hPa at 20 °C

Vapour density 				: No data available.

Relative density 				: 1,325 g/cm3 at 25 °C

Water solubility 				: No data available.
Partition coefficient: noctanol/water: No data available.

Auto-ignition temperature 		: No data available.
Decomposition temperature 		: No data available.

Viscosity 					: No data available.

Explosive properties 			: No data available.

Oxidizing properties 			: No data available.

Section 10: Stability and Reactivity Data

Reactivity:  No data available.

Chemical stability: Stable under recommended storage conditions.

Possibility of hazardous reactions: No data available.

Conditions to avoid: Heat, flames and sparks.

Incompatible materials: Alcohols, Amines.

Hazardous decomposition products:

Other decomposition products : No data available.

Section 11: Toxicological Information

Information on toxicological effects

Acute toxicity: LD50 Oral - rat - 470 mg/kg

Skin corrosion/irritation: No data available.

Serious eye damage/eye irritation: No data available.

Respiratory or skin sensitisation: No data available.

Germ cell mutagenicity: No data available.

Carcinogenicity

IARC: No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

Reproductive toxicity: No data available.

Specific target organ toxicity - single exposure: No data available.

Specific target organ toxicity - repeated exposure: No data available.

Aspiration hazard: No data available.

Section 11: Toxicological Information (Continued)

Additional Information

RTECS: Not available.

Material is extremely destructive to tissue of the mucous membranes and upper respiratory tract, eyes, and skin., spasm, inflammation and edema of the larynx, spasm, inflammation and edema of the bronchi, pneumonitis, pulmonary edema, burning sensation, Cough, wheezing, laryngitis, Shortness of breath, Headache, Nausea.

Section 12: Ecological Information

Toxicity: No data available.

Persistence and degradability: No data available.

Bioaccumulative potential: No data available.

Mobility in soil: No data available.

Results of PBT and vPvB assessment:

PBT/vPvB assessment not available as chemical safety assessment not required/not conducted.

Other adverse effects: No data available.

Section 13: Disposal Considerations

Waste Disposal :

Product

Burn in a chemical incinerator equipped with an afterburner and scrubber but exert extra care in igniting

as this material is highly flammable. Offer surplus and non-recyclable solutions to a licensed disposal  company. Contact a licensed professional waste disposal service to dispose of this material.

Contaminated packaging

Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name : CHLOROFORMATES, TOXIC, CORROSIVE, FLAMMABLE, N.O.S.(1-Chloroethyl chloroformate)

UN N° : 2742

ADR - Class : 6.1(3, 8)

ADR - Group : II

Sea transport (IMDG) [English only]

Proper shipping name : CHLOROFORMATES, TOXIC, CORROSIVE, FLAMMABLE, N.O.S.(1-Chloroethyl chloroformate)

UN N° : 2742

IMO-IMDG - Class or division : 6.1(3, 8)

IMO-IMDG - Packing group : II

Air transport (ICAO-IATA) [English only]

Proper shipping name : Chloroformates, toxic, corrosive, flammable, n.o.s.(1-Chloroethyl chloroformate)

UN N° : 2742

IATA - Class or division : 6.1(3, 8)

IATA - Packing group : II

Section 15: Other Regulatory Information

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

Safety, health and environmental regulations/legislation specific for the substance or mixture

No data available.

Chemical Safety Assessment

For this product a chemical safety assessment was not carried out.

`

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.