MATERIAL SAFETY DATA SHEET

CHROMIUM BROMIDE 99%

(Hexahydrate)

(Chromium (III) bromide)

MSDS CAS: 13478-06-3

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: CHROMIUM BROMIDE

CAS#:   13478-06-3

Synonym: chromium (III) bromide

Chemical Name: Chromium triacetate

Chemical Formula:  Br3Cr·6H2O

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight | Hazardous |
| --- | --- | --- | --- |
| Chromium Bromide | 13478-06-3 | 98 | Yes |

Section 3: Hazards Identification

Risk advice to man and the environment:

Harmful in contact with skin and if swallowed. Very toxic by inhalation. Causes burns. May cause sensitization by skin contact. Possible risk of harm to the unborn child.

Section 4: First Aid Measures

General advice:

Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled:

If breathed in, move person into fresh air. If not breathing give artificial respiration Consult a physician.

In case of skin contact:

Take off contaminated clothing and shoes immediately.

Wash off with soap and plenty of water. Consult a physician.

In case of eye contact:

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed:

Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Section 5: Fire and Explosion Data

Suitable extinguishing media:

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special protective equipment for fire-fighters:

Wear self contained breathing apparatus for fire fighting if necessary.

Section 6: Accidental Release Measures

Personal precautions:

Wear respiratory protection. Avoid dust formation.  Avoid breathing dust. Ensure adequate ventilation. Evacuate personnel to safe areas.

Environmental precautions:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Methods for cleaning up:

Pick up and arrange disposal without creating dust. Keep in suitable, closed containers for disposal.

Section 7: Handling and Storage

Handling:

Avoid contact with skin and eyes. Avoid formation of dust and aerosols. Provide appropriate exhaust ventilation at places where dust is formed. Normal measures for preventive fire protection.

Storage:

Store in cool place. Keep container tightly closed in a dry and well-ventilated place.

Section 8: Exposure Controls/Personal Protection

Personal protective equipment

Respiratory protection:

Where risk assessment shows air -purifying respirators are appropriate use a full-face particle respirator type N100 (US) or type P3 (EN 143) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator.  Where risk assessment shows air-purifying respirators are appropriate use a full-face particle respirator type N99 (US) or type P2 (EN 143) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Hand protection:

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it. Handle with gloves.

Eye protection:

Face shield and safety glasses.

Section 8: Exposure Controls/Personal Protection (Continued)

Skin and body protection:

Choose body protection according to the amount and concentration of the dangerous substance at the work place.

Hygiene measures:

Avoid contact with skin, eyes and clothing.  Wash hands before breaks and immediately after handling the product.

Section 9: Physical and Chemical Properties

Appearance Form		:  Powder.

Colour				:  Dark green.

Molecular Weight		:  399.8 g/mole

pH					:  No data available.

Melting point			:  79 °C - lit.

Boiling point			:  No data available.

Flash point				:  Not applicable.

Ignition temperature  		:  No data available.

Lower explosion limit 		:  No data available.

Upper explosion limit 		:  No data available.

Density				: 5,4 g/mL at 25 °C

Water solubility  			: No data available.

Section 10: Stability and Reactivity Data

Storage stability

Stable under recommended storage conditions.

Materials to avoid:

Strong oxidizing agents.

Hazardous decomposition products:

Hazardous decomposition products formed under fire conditions. - Hydrogen bromide gas, Chromium oxides.

Section 11: Toxicological Information

Acute toxicity: No data available.

Irritation and corrosion: No data available.

Sensitisation: May cause allergic skin reaction.

Chronic exposure

IARC: No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

Signs and Symptoms of Exposure:

To the best of our knowledge, the chemical, physical, and toxicological properties have not been thoroughly investigated., Material is extremely destructive to tissue of the mucous membranes and upper respiratory tract, eyes, and skin., burning sensation, Cough, wheezing, laryngitis, Shortness of breath, Headache, Nausea,

Vomiting, spasm, inflammation and edema of the larynx, spasm, inflammation and edema of the bronchi.

Potential Health Effects:

Inhalation May be fatal if inhaled. Material is extremely destructive to the tissue of the mucous membranes and upper respiratory tract.

Skin Harmful if absorbed through skin. Causes skin burns.

Eyes Causes eye burns.

Ingestion Harmful if swallowed. Causes burns.

Target Organs Liver, Kidney.

Section 12: Ecological Information

Elimination information (persistence and degradability):  No data available.

Ecotoxicity effects:  No data available.

Further information on ecology:  No data available.

Section 13: Disposal Considerations

Waste Disposal

Product:

Observe all federal, state, and local environmental regulations. Contact a licensed professional waste disposal service to dispose of this material. Dissolve or mix the material with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and scrubber.

Contaminated packaging:

Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name: CORROSIVE SOLID, ACIDIC, INORGANIC, N.O.S. (Chromium (III)    Bromide hexahydrate)

UN N°:  3260

ADR - Class:  8

ADR - Group: III

Sea transport (IMDG) [English only]

Proper shipping name: CORROSIVE SOLID, ACIDIC, INORGANIC, N.O.S. (Chromium (III) bromide hexahydrate)

UN N°: 3260

IMO-IMDG - Class or division :  8

IMO-IMDG - Packing group: III

Air transport (ICAO-IATA) [English only]

Proper shipping name : Corrosive solid, acidic, inorganic n.o.s. (Chromium(III) bromide hexahydrate)

UN N° : 3260

IATA - Class or division : 8

IATA - Packing group : III

Section 15: Other Regulatory Information

Labelling according to EC Directives

Hazard symbols

T+ Very toxic

R-phrase(s):

R21/22 Harmful in contact with skin and if swallowed.

R26 Very toxic by inhalation.

R34 Causes burns.

R43 May cause sensitization by skin contact.

R63 Possible risk of harm to the unborn child.

S-phrase(s):

S22 Do not breathe dust.

S26 In case of contact with eyes, rinse immediately with plenty of water and seek medical advice.

S36/37/39 Wear suitable protective clothing, gloves and eye/face protection.

S45 In case of accident or if you feel unwell, seek medical advice immediately (show the

label where possible).

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.