MATERIAL SAFETY DATA SHEET

COBALT (II) BROMIDE

(Cobaltous Bromide)

MSDS CAS: 7789-43-7

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: COBALT (II) BROMIDE

CAS#:   7789-43-7

Synonym: Cobaltous Bromide

Chemical Name: Cobalt Bromide

Chemical Formula:  Br2Co

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight | Hazardous |
| --- | --- | --- | --- |
| Cobalt Bromide | 7789-43-7 | 100 | Yes |

Section 3: Hazards Identification

Risk advice to man and the environment:

May cause cancer. Also harmful by inhalation, in contact with skin and if swallowed. Irritating to eyes, respiratory system and skin. May cause sensitization by inhalation and skin contact.

Section 4: First Aid Measures

General advice:

Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled:

If breathed in, move person into fresh air. If not breathing give artificial respiration Consult a physician.

In case of skin contact:

Wash off with soap and plenty of water. Consult a physician.

In case of eye contact:

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed:

Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Section 5: Fire and Explosion Data

Suitable extinguishing media:

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special protective equipment for fire-fighters:

Wear self-contained breathing apparatus for firefighting if necessary.

Section 6: Accidental Release Measures

Personal precautions:

Use personal protective equipment.

Avoid dust formation.

Avoid breathing dust. Ensure adequate ventilation.

Evacuate personnel to safe areas.

Environmental precautions:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Methods for cleaning up:

Pick up and arrange disposal without creating dust. Keep in suitable, closed containers for disposal.

Section 7: Handling and Storage

Handling:

Avoid exposure - obtain special instructions before use. Avoid formation of dust and aerosols. Provide appropriate exhaust ventilation at places where dust is formed. Normal measures for preventive fire protection.

Storage:

Store in cool place. Keep container tightly closed in a dry and well-ventilated place.

Section 8: Exposure Controls/Personal Protection

Personal protective equipment

Respiratory protection:

Where risk assessment shows air-purifying respirators are appropriate use a full-face particle respirator type N99 (US) or type P2 (EN 143) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Hand protection:

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it. Handle with gloves.

Eye protection:

Face shield and safety glasses.

Section 8: Exposure Controls/Personal Protection (Continued)

Skin and body protection:

Choose body protection according to the amount and concentration of the dangerous substance at the work place.

Hygiene measures:

Avoid contact with skin, eyes and clothing.  Wash hands before breaks and immediately after handling the product.

Section 9: Physical and Chemical Properties

Appearance Form		: powder.

pH					: No data available.

Molecular Weight		: 218.74 g/mole.

Melting point			: No data available.

Boiling point			: No data available.

Flash point				: Not applicable.

Ignition temperature		: No data available.

Lower explosion limit		: No data available.

Upper explosion limit		: No data available.

Density				: 4,909 g/mL at 25 °C.

Water solubility			: No data available.

Section 10: Stability and Reactivity Data

Storage stability

Stable under recommended storage conditions.

Materials to avoid

Oxidizing agents, Alkali metals.

Hazardous decomposition products

Hazardous decomposition products formed under fire conditions. - Hydrogen bromide gas, Cobalt/cobalt oxides.

Section 11: Toxicological Information

Acute toxicity

LD50 Oral - rat - 406 mg/kg

Remarks: Behavioral: Somnolence (general depressed activity). Diarrhoea Nutritional and Gross

Metabolic: Weight loss or decreased weight gain.

Irritation and corrosion

Sensitisation

Chronic exposure

This product is or contains a component that has been reported to be probably carcinogenic based on its

IARC, OSHA, ACGIH, NTP, or EPA classification.

IARC: No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

Potential Health Effects

Inhalation Harmful if inhaled. Causes respiratory tract irritation.

Skin Harmful if absorbed through skin. Causes skin irritation.

Eyes Causes eye irritation.

Ingestion Harmful if swallowed.

Target Organs: Lungs, Blood

Section 12: Ecological Information

Elimination information (persistence and degradability)

Ecotoxicity effects

Further information on ecology

Section 13: Disposal Considerations

Product:

Observe all federal, state, and local environmental regulations. Contact a licensed professional waste disposal service to dispose of this material. Dissolve or mix the material with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and scrubber.

Contaminated packaging: Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name: ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID, N.O.S. (Cobalt dibromide).

UN N°: 3077

H.I. nr:

ADR - Class: 9

Labelling - Transport: 9: Flammable solids, self-reactive substances and desensitized explosives.

Sea transport (IMDG) [English only]

Proper shipping name: ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID, N.O.S. (Cobalt dibromide)

UN N°: 3077

IMO-IMDG - Class or division: 9: Marine pollutant

IMO-IMDG - Packing group: III

Air transport (ICAO-IATA) [English only]

Proper shipping name: ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID, N.O.S. (Cobalt dibromide)

UN N°: 3077

IATA - Class or division: 9 : Flammable solids, self-reactive substances and desensitized explosives.

IATA - Packing group: III

Section 15: Other Regulatory Information

Labelling according to EC Directives

Hazard symbols

T Toxic

R-phrase(s):

R45 May cause cancer.

R20/21/22 Also harmful by inhalation, in contact with skin and if swallowed.

R36/37/38 Irritating to eyes, respiratory system and skin.

R42/43 May cause sensitization by inhalation and skin contact.

S-phrase(s):

S45 In case of accident or if you feel unwell, seek medical advice immediately (show the label where possible).

S26 In case of contact with eyes, rinse immediately with plenty of water and seek medical advice.

S36/37/39 Wear suitable protective clothing, gloves and eye/face protection.

S22 Do not breathe dust.

Restricted to professional users.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.