MATERIAL SAFETY DATA SHEET

COBALT NAPHTHENATE

(6% Solution)

MSDS CAS: 61789-51-3

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: COBALT NAPHTHENATE

CAS#:   61789-51-3

Synonym: Not Available.

Chemical Name: Cobalt Naphthenate

Chemical Formula:  Not Available.

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Naphthenic acids, cobalt salts | 61789-51-3 | 50-100 |
| Stoddard solvent (0.1% Benzene) | 8052-41-3 | 30-50 |

Section 3: Hazards Identification

Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008 [EU-GHS/CLP]

Flammable liquids (Category 3)

Respiratory sensitization (Category 1)

Skin sensitization (Category 1)

Aspiration hazard (Category 1)

Classification according to EU Directives 67/548/EEC or 1999/45/EC

Flammable. Harmful: may cause lung damage if swallowed. May cause sensitization by inhalation and skin contact.

Label elements

Labelling according Regulation (EC) No 1272/2008 [CLP]

Hazard statement(s):

H226Flammable liquid and vapor.

H304 May be fatal if swallowed and enters airways.

H317May cause an allergic skin reaction.

H334 May cause allergy or asthma symptoms or breathing difficulties if inhaled.

Precautionary statement(s):

P261 Avoid breathing vapors.

P280 Wear protective gloves.

P301 + P310 IF SWALLOWED: Immediately call a POISON CENTER or doctor/ physician.

P331 Do NOT induce vomiting.

P342 + P311 If experiencing respiratory symptoms: Call a POISON CENTER or doctor/ physician.

According to European Directive 67/548/EEC as amended.

R-phrase(s):

R10Flammable.

R42/43May cause sensitization by inhalation and skin contact.

R65Harmful: may cause lung damage if swallowed.

S-phrase(s):

S23Do not breathe vapor.

S36/37 Wear suitable protective clothing and gloves.

S45 In case of accident or if you feel unwell, seek medical advice immediately (show the label where possible).

S62 If swallowed, do not induce vomiting: seek medical advice immediately and show this container or label.

Section 4: First Aid Measures

Description of first aid measures

General advice:

Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled:

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

In case of skin contact:

Wash off with soap and plenty of water. Consult a physician.

In case of eye contact:

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed:

Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Most important symptoms and effects, both acute and delayed

Indication of any immediate medical attention and special treatment needed: No data available.

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special hazards arising from the substance or mixture:

Nature of decomposition products not known.

Advice for firefighters:

Wear self-contained breathing apparatus for firefighting if necessary.

Further information:

Use water spray to cool unopened containers.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Use personal protective equipment. Avoid breathing vapors, mist or gas. Ensure adequate ventilation.

Remove all sources of ignition. Evacuate personnel to safe areas. Beware of vapors accumulating to

Form explosive concentrations. Vapors can accumulate in low areas.

Environmental precautions:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Methods and materials for containment and cleaning up:

Contain spillage, and then collect with an electrically protected vacuum cleaner or by wet-brushing and place in container for disposal according to local regulations (see section 13).

Section 7: Handling and Storage

Precautions for safe handling:

Avoid contact with skin and eyes. Avoid inhalation of vapor or mist.

Keep away from sources of ignition -No smoking. Take measures to prevent the buildup of electrostatic charge.

Conditions for safe storage, including any incompatibilities:

Store in cool place.

Keep container tightly closed in a dry and well-ventilated place.

Containers which are opened must be carefully resealed and kept upright to prevent leakage.

Section 8: Exposure Controls/Personal Protection

Control parameters

Components with workplace control parameters

Exposure controls

Appropriate engineering controls:

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday. Personal protective equipment

Section 8: Exposure Controls/Personal Protection (Continued)

Eye/face protection:

Face shield and safety glasses Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

Skin protection:

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices.

Wash and dry hands. The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

Body Protection:

Complete suit protecting against chemicals, Flame retardant antistatic protective clothing, and the type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace.

Respiratory protection:

Where risk assessment shows air-purifying respirators are appropriate use a full-face respirator with multi-purpose combination (US) or type ABEK (EN 14387) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Section 9: Physical and Chemical Properties

Appearance Form		: Liquid.

Colour				: Dark, Purple.

Odour				: No data available.

Odour Threshold			: No data available.

pH					: No data available.

Melting point			: No data available.

Boiling range			: > 300 °C-Solvent.

Flash point				: 48,9 °C-Tag closed cup

Evaporation rate			: No data available.

Flammability (solid, gas)	: No data available.

Explosive limits			: Upper explosion limit: 6 %(V), Lower explosion limit: 1 %(V)

Section 9: Physical and Chemical Properties (Continued)

Vapour pressure				: No data available.

Vapour density				: No data available.

Relative density				: 0,921 g/mL at 25 °C

Water solubility				: Negligible

Partition coefficient: n-octanol/water: No data available.

Auto-ignition temperature		: No data available.

Decomposition temperature		: No data available.

Viscosity					: No data available.

Explosive properties			: No data available.

Oxidizing properties			: No data available.

Section 10: Stability and Reactivity Data

Reactivity: no data available.

Chemical stability: no data available.

Possibility of hazardous reactions: no data available.

Conditions to avoid: Heat, flames and sparks.

Incompatible materials: Strong oxidizing agents.

Hazardous decomposition products:

Other decomposition products-no data available.

Section 11: Toxicological Information

Information on toxicological effects

Acute toxicity

LD50 Oral-rat-3.900 mg/kg

No data available.

Skin corrosion/irritation: No data available.

Section 11: Toxicological Information (Continued)

Serious eye damage/eye irritation: No data available.

Respiratory or skin sensitization: No data available.

Germ cell mutagenicity: No data available.

Carcinogenicity

IARC: 2B-Group 2B: Possibly carcinogenic to humans (Naphthenic acids, cobalt salts)

Reproductive toxicity: No data available.

Specific target organ toxicity -single exposure: No data available.

Specific target organ toxicity -repeated exposure: No data available.

Aspiration hazard: No data available

Potential health effects:

Inhalation May be harmful if inhaled. May cause respiratory tract irritation.

Ingestion May be harmful if swallowed. Aspiration hazard if swallowed -can enter lungs and cause damage.

Skin May be harmful if absorbed through skin. May cause skin irritation.

Eyes Causes eye irritation

Section 12: Ecological Information

Toxicity: No data available.

Persistence and degradability: No data available.

Bioaccumulative potential: No data available.

Mobility in soil: No data available.

Results of PBT and vPvB assessment: No data available.

Other adverse effects: No data available.

Section 13: Disposal Considerations

Waste treatment methods

Product:

Burn in a chemical incinerator equipped with an afterburner and scrubber but exert extra care in igniting as this material is highly flammable. Offer surplus and non-recyclable solutions to a licensed disposal company.

Contaminated packaging:

Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name: FLAMMABLE LIQUID, N.O.S.(Aliphatic hydrocarbons (Stoddard type))

UN N°: 1993

ADR - Class: 3

ADR - Packing group: III

Sea transport (IMDG) [English only]

Proper shipping name: FLAMMABLE LIQUID, N.O.S.(Aliphatic hydrocarbons (Stoddard type))

UN N°: 1993

IMO-IMDG - Class or division: 3

IMO-IMDG - Packing group: III

Air transport (ICAO-IATA) [English only]

Proper shipping name: Flammable liquid, n.o.s.(Aliphatic hydrocarbons (Stoddard type))

UN N°: 1993

IATA - Class or division: 3

IATA - Packing group: III

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture:

No data available.

Chemical Safety Assessment: No data available.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.