MATERIAL SAFETY DATA SHEET

COCOA BUTTER

Extra Pure

MSDS CAS: 8002-31-1

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: Cocoa Butter

CAS#:   8002-31-1

Synonym: Not Available.

Chemical Name: Not Available.

Chemical Formula:  Not Available.

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Cocoa Butter | 8002-31-1 | 100 |

Toxicological Data on Ingredients:  Not Applicable.

Section 3: Hazards Identification

Potential Acute Health Effects:

No specific information is available in our database regarding the acute toxic effects of this material for humans.

Potential Chronic Health Effects:

CARCINOGENIC EFFECTS: Not available.

MUTAGENIC EFFECTS: Not available.

TERATOGENIC EFFECTS: Not available.

DEVELOPMENTAL TOXICITY: Not available.

Repeated or prolonged exposure is not known to aggravate medical condition.

Section 4: First Aid Measures

Eye Contact:  No known effect on eye contact, rinse with water for a few minutes.

Skin Contact: No known effect on skin contact, rinse with water for a few minutes.

Serious Skin Contact:  Not Available.

Inhalation:  Allow the victim to rest in a well ventilated area. Seek immediate medical attention.

Serious Inhalation:  Not Available.

Ingestion:

Do not induce vomiting. Loosen tight clothing such as a collar, tie, belt or waistband. If the victim is not breathing, perform mouth-to-mouth resuscitation. Seek immediate medical attention.

Serious Ingestion: Not available.

Section 5: Fire and Explosion Data

Flammability of the Product: May be combustible at high temperature.

Auto-Ignition Temperature: Not applicable.

Flash Points: Not available.

Section 5: Fire and Explosion Data (Continued)

Flammable Limits: Not available.

Products of Combustion:  Not available.

Fire Hazards in Presence of Various Substances: Not applicable.

Explosion Hazards in Presence of Various Substances:

Risks of explosion of the product in presence of mechanical impact: Not available.

Risks of explosion of the product in presence of static discharge: Not available.

Fire Fighting Media and Instructions:

SMALL FIRE: Use DRY chemical powder.

LARGE FIRE: Use water spray, fog or foam. Do not use water jet.

Special Remarks on Fire Hazards:  Not available.

Special Remarks on Explosion Hazards:  Not available.

Section 6: Accidental Release Measures

Small Spill:

Use appropriate tools to put the spilled solid in a convenient waste disposal container. Finish cleaning by spreading water on the contaminated surface and dispose of according to local and regional authority requirements.

Large Spill:

If the product is in its solid form: Use a shovel to put the material into a convenient waste disposal container. If the product is in its liquid form: Absorb with an inert material and put the spilled material in an appropriate waste disposal. Finish cleaning by spreading water on the contaminated surface and allow to evacuate through the sanitary system.

Section 7: Handling and Storage

Precautions:

Keep away from heat. Keep away from sources of ignition. Empty containers pose a fire risk, evaporate the residue under a fume hood. Ground all equipment containing material. Do not breathe dust. Keep away from incompatibles such as oxidizing agents.

Section 7: Handling and Storage (Continued)

Storage:

Keep container dry. Keep in a cool place. Ground all equipment containing material. Keep container tightly closed. Keep in a cool, well-ventilated place. Combustible materials should be stored away from extreme heat and away from strong oxidizing agents.

Section 8: Exposure Controls/Personal Protection

Engineering Controls:

Use process enclosures, local exhaust ventilation, or other engineering controls to keep airborne levels below recommended exposure limits. If user operations generate dust, fume or mist, use ventilation to keep exposure to airborne contaminants below the exposure limit.

Personal Protection:  Safety glasses. Lab coat.

Personal Protection in Case of a Large Spill:

Splash goggles. Full suit. Boots. Gloves. Suggested protective clothing might not be sufficient; consult a specialist BEFORE handling this product.

Exposure Limits:  Not available.

Section 9: Physical and Chemical Properties

Physical state and appearance: Solid. (Greasy solid.)

Odor				: Not available.

Taste				: Not available.

Molecular Weight	: Not available.

Color				: Yellow. Yellowish White.

pH (1% soln/water)	: Not applicable.

Boiling Point	: Not available.

Melting Point	: 31°C (87.8°F)

Critical Temperature	: Not applicable.

Specific Gravity		: Not available.

Vapor Pressure		: Not applicable.

Vapor Density		: Not applicable.

Volatility			: Not available.

Section 9: Physical and Chemical Properties (Continued)

Odor Threshold		: Not applicable.

Water/Oil Dist. Coeff.	: Not applicable.

Ionicity (in Water)	: Not available.

Dispersion Properties	: Not available.

Solubility			: Insoluble in cold water.

Section 10: Stability and Reactivity Data

Stability: The product is stable.

Instability Temperature: Not available.

Conditions of Instability: Not available.

Incompatibility with various substances: Reactive with oxidizing agents.

Corrosivity:  Not considered to be corrosive for metals and glass.

Special Remarks on Reactivity:  Not available.

Special Remarks on Corrosivity: Not available.

Polymerization:  No.

Section 11: Toxicological Information

Routes of Entry:  Not available.

Toxicity to Animals:  LD50: Not available. LC50: Not available.

Chronic Effects on Humans:  Not available.

Other Toxic Effects on Humans:

No specific information is available in our database regarding the other toxic effects of this material for humans.

Section 11: Toxicological Information (Continued)

Special Remarks on Toxicity to Animals: Not available.

Special Remarks on Chronic Effects on Humans:  Not Available.

Special Remarks on other Toxic Effects on Humans:  Not Available.

Section 12: Ecological Information

Ecotoxicity: Not available.

BOD5 and COD: Not available.

Products of Biodegradation:

Possibly hazardous short term degradation products are not likely. However, long term degradation products may arise.

Toxicity of the Products of Biodegradation:  The products of degradation are more toxic.

Special Remarks on the Products of Biodegradation: Not available.

Section 13: Disposal Considerations

Waste Disposal :

Section 14: Transport Information

Land transport (ADR-RID)

General information : Not regulated.

Section 14: Transport Information (Continued)

Sea transport (IMDG) [English only]

General information : Not regulated.

Air transport (ICAO-IATA) [English only]

General information : Not regulated.

Section 15: Other Regulatory Information

Federal and State Regulations: TSCA 8(b) inventory: No products were found.

Other Regulations:  Not available.

Other Classifications:

WHMIS (Canada): Not controlled under WHMIS (Canada).

DSCL (EEC):  This product is not classified according to the EU regulations.

HMIS (U.S.A.):

Health Hazard: 0

Fire Hazard: 1

Reactivity: 0

Personal Protection: a

National Fire Protection Association (U.S.A.):

Health: 0

Flammability: 1

Reactivity: 0

Specific hazard:

Protective Equipment:

Not applicable. Lab coat. Not applicable. Safety glasses.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.