MATERIAL SAFETY DATA SHEET

COOMASSIE BRILLIANT BLUE R-250

(Brilliant Blue R-250)

MSDS CAS :- 6104-59-2

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: COOMASSIE BRILLIANT BLUE R-250

CAS#: 6104-59-2

C.I. No.: 42660

Synonym: Not available.

Chemical Name: Not available.

Chemical Formula: C45H44N3NaO7S2

Molecular Weight: 825.99

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet:

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| COOMASIE BRILLIANT BLUE R 250 | 6104-59-2 |  |

Section 3: Hazards Identification

Classification of the substance or mixture:

Classification EC 67/548 or EC 1999/45: Not classified.

Hazard Class and Category Code(s), Regulation (EC) No 1272/2008 (CLP): Not classified.

Label elements:

Labelling EC 67/548 or EC 1999/45: Not classified.

S Phrase(s): S22 : Do not breathe dust.

S24/25 : Avoid contact with skin and eyes.

Labelling Regulation EC 1272/2008 (CLP): Not classified.

Other hazards: The substance does not fulfil the criteria to be identified as PBT substance or vPvB

substance according to Annex XIII of Regulation REACH.

Section 4: First Aid Measures

Description of first aid measures:

Inhalation: Assure fresh air breathing. Allow the victim to rest.

Skin contact: Remove affected clothing and wash all exposed skin area with mild soap and

water, followed by warm water rinse.

Eye contact: Rinse immediately with plenty of water. Obtain medical attention if pain, blinking or redness persist.

Ingestion: Obtain emergency medical attention. Do NOT induce vomiting. Rinse mouth.

Most important symptoms and effects, both acute and delayed:

Symptoms relating to use: Not expected to present a significant hazard under anticipated conditions of normal use.

Section 4: First Aid Measures (Continued)

Indication of any immediate medical attention and special treatment needed:

General information: Never give anything by mouth to an unconscious person. If you feel unwell, seek medical advice (show the label where possible).

Section 5: Fire and Explosion Data

Extinguishing media:

Suitable extinguishing media: Foam. Dry powder. Carbon dioxide. Water spray. Sand.

Unsuitable extinguishing media: Do not use a heavy water stream.

Surrounding fires: Use water spray or fog for cooling exposed containers.

Special hazards arising from the substance or mixture:

Hazardous combustion products: Under fire conditions, hazardous fumes will be present.

Advice for fire-fighters:

Protection against fire: Do not enter fire area without proper protective equipment, including respiratory

protection.

Special procedures: Exercise caution when fighting any chemical fire. Avoid (reject) fire-fighting water to

enter environment

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

For emergency responders: Equip cleanup crew with proper protection.Ventilate area.

Section 6: Accidental Release Measures (Continued)

For non-emergency personnel: Evacuate unnecessary personnel.

Environmental precautions: Prevent entry to sewers and public waters. Notify authorities if product enters sewers or public waters.

Methods and material for containment and cleaning up:

Clean up methods: On land, sweep or shovel into suitable containers. Minimize generation of dust.

Store away from other materials.

Reference to other sections: See section 8. Exposure controls/personal protection

Section 7: Handling and Storage

Precautions for safe handling:

Handling: Wash hands and other exposed areas with mild soap and water before eat, drink or smoke and when leaving work.

Technical protective measures: Provide good ventilation in process area to prevent formation of vapour.

Conditions for safe storage, including any incompatibilities:

Storage: Keep only in the original container in a cool, well ventilated place. Keep container

closed when not in use.

Storage : Away from: Strong bases. Strong acids. Sources of ignition. Direct sunlight.

Specific end use(s): None.

Section 8: Exposure Controls/Personal Protection

Exposure controls:

Personal protection: Avoid all unnecessary exposure.

Respiratory protection: Wear approved mask.

Hand protection: Wear protective gloves.

Eye protection: Chemical goggles or safety glasses.

Others: When using, do not eat, drink or smoke.

Control parameters:

Occupational Exposure Limits: No data available.

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties:

Physical state at 20°C				: Solid.

Colour						: Brownish purple.

Odour						: Not available.

Odour threshold					: Not available.

pH value						: Not available.

Melting point [°C]				: Not available.

Decomposition point [°C]			: Not available.

Critical temperature [°C]			: Not available.

Auto-ignition temperature [°C]		: Not available.

Flammability (solid, gas)			: Not available.

Flash point [°C]					: Not available.

Boiling point [°C]					: Not available.

Initial boiling point [°C]			: Not available.

Section 9: Physical and Chemical Properties (Continued)

Final boiling point [°C]				: Not available.

Evaporation rate					: Not available.

Vapour pressure [20°C]				: Not available.

Vapour pressure mm/Hg			: Not available.

Vapour density					: 28,5

Density [g/cm3]					: Not available.

Relative density, gas (air=1)			: Not available.

Relative density, liquid (water=1)		: Not available.

Solubility in water [% weight]		: Soluble in water

Section 10: Stability and Reactivity Data

Reactivity: Not established.

Chemical stability: Stable under recommended storage conditions.

Possibility of hazardous reactions:

Hazardous reactions: Not established.

Conditions to avoid: Direct sunlight. Extremely high or low temperatures.

Incompatible materials:

Materials to avoid: Strong acids. Strong bases.

Hazardous decomposition products: Fumes. Carbon monoxide. Carbon dioxide.

Section 11: Toxicological Information

Information on toxicological effects:

Acute toxicity:

Inhalation: Based on available data, the classification criteria are not met.

Dermal: Based on available data, the classification criteria are not met.

Ingestion: Based on available data, the classification criteria are not met.

Corrosion: Based on available data, the classification criteria are not met.

Irritation: Based on available data, the classification criteria are not met.

Sensitization: Based on available data, the classification criteria are not met.

Mutagenicity: Based on available data, the classification criteria are not met.

Carcinogenicity: Based on available data, the classification criteria are not met.

Toxic for reproduction: Based on available data, the classification criteria are not met.

STOT-single exposure: Based on available data, the classification criteria are not met.

STOT-repeated exposure: Based on available data, the classification criteria are not met.

Aspiration hazard: Based on available data, the classification criteria are not met.

Section 12: Ecological Information

Toxicity: Not established.

Persistence - degradability: Biodegradable.

Section 12: Ecological Information (Continued)

Bioaccumulative potential: Not established.

Mobility in soil: Not established.

Results of PBT and vPvB assessment: The substance does not fulfil the criteria to be identified as PBT substance or vPvB substance according to Annex XIII of Regulation REACH.

Other adverse effects:

Environmental precautions: Avoid release to the environment.

Section 13: Disposal Considerations

Waste treatment methods:

General: Avoid release to the environment. Dispose in a safe manner in accordance with

local/national regulations.

Section 14: Transport Information

Land transport (ADR-RID):

General information : Not regulated.

Sea transport (IMDG) [English only]:

General information : Not regulated.

Air transport (ICAO-IATA) [English only]:

General information : Not regulated.

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture:

Safety, health and environmental: Ensure all national/local regulations are observed.

regulations/legislation specific for thesubstance or mixture.

REACH Restrictions - Annex XVII: The components of this product are not subject to restrictions.

REACH Authorisation - Annex XIV: The components of this product are not subject to authorization.

Chemical Safety Assessment: It has not been carried out.

Section 16 - Additional Information

Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.