MATERIAL SAFETY DATA SHEET

COPPER AAS STANDARD SOLUTION

(1000mg/L Cu IN DILUTED HNO3)

MSDS CAS:

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: COPPER AAS STANDARD SOLUTION-1000mg/L Cu IN DILUTED HNO3

CAS#:

Chemical Name: COPPER AAS STANDARD SOLUTION-1000mg/L Cu IN DILUTED HNO3

Chemical Formula:

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| COPPER FINE POWDER AR (325 mesh) 99.7% | 7440-50-8 | > 0.01 < 1 % |
| Nitric acid | 7697-37-2 | 2.2 % |
| Water | 7732-18-5 | > 75 < 99 % |

Toxicological Data on Ingredients: Not available.

Section 3: Hazards Identification

Classification of the substance or mixture

Classification EC 67/548  or EC 1999/45

Classification        :  Xi; R36/38

Hazard Class and Category Code(s), Regulation (EC) No 1272/2008 (CLP)

Health hazards     :  Skin irritation - Category 2 - Warning  (CLP : Skin Corr. 2) H315

Eye irritation - Category 2A - Warning  (CLP : Eye Irrit. 2) H319

Other hazards       :  The substance does not fulfil the criteria to be identified as PBT substance or vPvB substance according to Annex XIII of Regulation REACH.

Section 4: First Aid Measures

Description of first aid measures:

Inhalation:  Assure fresh air breathing. Allow the victim to rest.

Skin contact:  Specific treatment (see on this label). Get medical advice. If skin irritation occurs: Wash with plenty of soap and water. Wash contaminated clothing before reuse.

Eye contact:  If eye irritation persists: Get medical advice. Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

Ingestion:  Obtain emergency medical attention. Rinse mouth. Do NOT induce vomiting.

Most important symptoms and effects, both acute and delayed:

Symptoms relating to use:  Causes serious eye irritation.

Causes skin irritation.

Indication of any immediate medical attention and special treatment needed:

General information:  Never give anything by mouth to an unconscious person. If you feel unwell, seek medical advice (show the label where possible).

Section 5: Fire and Explosion Data

Extinguishing media:

Suitable extinguishing media         : Foam. Dry powder. Carbon dioxide. Water spray. Sand.

Unsuitable extinguishing media     : Do not use a heavy water stream.

Surrounding fires                            : Use water spray or fog for cooling exposed containers.

Special hazards arising from the substance or mixture:

Hazardous combustion products: Under fire conditions, hazardous fumes will be present.

Advice for fire-fighters:

Protection against fire                     : Do not enter fire area without proper protective equipment,             including respiratory protection.

Special procedures                           : Exercise caution when fighting any chemical fire. Avoid (reject)                            fire-fighting water to enter environment.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

For emergency responders        :Equip cleanup crew with proper protection.

:Ventilate area.

For non-emergency personnel  :Evacuate unnecessary personnel.

Environmental precautions

Environmental precautions       :Prevent entry to sewers and public waters. Notify authorities if product           enters sewers or public waters.

Methods and material for containment and cleaning up:

Clean up methods                       : Soak up spills with inert solids, such as clay or diatomaceous earth as           soon as possible. Collect spillage. Store away from other materials.

Section 7: Handling and Storage

Precautions:

Wash thoroughly after handling. Wash thoroughly after handling. Provide good ventilation in process area to prevent formation of vapour.

Section 7: Handling and Storage (Continued)

Storage:  Keep only in the original container in a cool, well ventilated place. Keep container closed when not in use. Storage - away from: Strong bases. Strong acids. Sources of ignition. Direct sunlight.

Section 8: Exposure Controls/Personal Protection

Exposure controls:

Personal protection: Avoid all unnecessary exposure.

• Respiratory protection: Wear approved mask.

• Hand protection: Wear protective gloves.

• Skin protection: Wear suitable protective clothing.

• Eye protection: Chemical goggles or safety glasses.

• Others: When using, do not eat, drink or smoke.

Control parameters:

Occupational Exposure Limits: No data available.

Section 9: Physical and Chemical Properties

Physical state and appearance	: Liquid.

Odor						: Characteristic.

Taste						: Not available.

Molecular Weight			: Not available.

Color						: light blue.

pH (1% soln/water)			: 6 - 8 at 25 °C

Boiling Point				: 100°C

Melting Point				: Not available.

Critical Temperature			: Not available.

Specific Gravity				: Not available.

Vapor Pressure				: Not applicable.

Vapor Density				: Not available.

Volatility					: Not available.

Odor Threshold				: Not available.

Water/Oil Dist. Coeff.			: Not available.

Ionicity (in Water)			: Not available.

Dispersion Properties			: Not available.

Solubility					: Not available.

Section 10: Stability and Reactivity Data

Stability: The product is stable.

Instability Temperature: Not available.

Conditions to avoid:  Direct sunlight. Extremely high or low temperatures.

Incompatibility with various substances: Not available.

Materials to avoid:  Strong acids. Strong bases.

Special Remarks on Reactivity: Not available.

Special Remarks on Corrosivity: Not available.

Hazardous decomposition products:  Fumes. Carbon monoxide. Carbon dioxide.

Polymerization: No.

Section 11: Toxicological Information

Information on toxicological effects:

Toxicity information : The product has been not fully tested. The calculated risk has been done under the requirements of the EU regulations.

Acute toxicity:

• Inhalation: Based on available data, the classification criteria are not met.

• Dermal: Based on available data, the classification criteria are not met.

• Ingestion: Based on available data, the classification criteria are not met.

Corrosion: Based on available data, the classification criteria are not met.

Irritation: Causes serious eye irritation.

Causes skin irritation.

Sensitization: Based on available data, the classification criteria are not met.

Mutagenicity: Based on available data, the classification criteria are not met.

Carcinogenicity: Based on available data, the classification criteria are not met.

Toxic for reproduction: Based on available data, the classification criteria are not met.

STOT-single exposure: Based on available data, the classification criteria are not met.

STOT-repeated exposure: Based on available data, the classification criteria are not met.

Aspiration hazard: Based on available data, the classification criteria are not met.

Section 12: Ecological Information

Toxicity information:  The product has been not fully tested. The calculated risk has been done under the requirements of the EU regulations.

BOD5 and COD: Not available.

Persistence - degradability:  Biodegradable.

Bioaccumulative potential:  Not established.

Toxicity of the Products of Biodegradation: Not available.

Special Remarks on the Products of Biodegradation: Not available.

Results of PBT and vPvB assessment:  The substance does not fulfil the criteria to be identified as PBT substance or vPvB substance according to Annex XIII of Regulation REACH.

Environmental precautions:  Avoid release to the environment.

Section 13: Disposal Considerations

Waste Disposal: Avoid release to the environment. Dispose in a safe manner in accordance with local/national regulations.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name : CORROSIVE LIQUID, ACIDIC, INORGANIC, N.O.S.

UN N° 		        : 3264

ADR - Class 	        : 8

Labelling – Transport :  8 : Corrosive substance.

Section 14: Transport Information (Continued)

Sea transport (IMDG) [English only]

Proper shipping name         : CORROSIVE LIQUID, ACIDIC, INORGANIC, N.O.S.

UN N° 		                : 3264

IMO-IMDG - Class or division  : 8 : Corrosive substance.

IMO-IMDG - Packing group      : III

Air transport (ICAO-IATA) [English only]

Proper shipping name           : CORROSIVE LIQUID, ACIDIC, INORGANIC, N.O.S.

UN N°                                      : 3264

IATA - Class or division        : 8 : Corrosive substance.

IATA - Packing group           : III

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture:

Safety, health and environmental      : Ensure all national/local regulations are observed. regulations/legislation specific for

the substance or mixture.

REACH Restrictions - Annex XVII   : The components of this product are not subject to restrictions.

REACH Authorisation - Annex XIV : The components of this product are not subject to authorization.

Chemical Safety Assessment:

Chemical Safety Assessment               : It has not been carried out.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.