MATERIAL SAFETY DATA SHEET

COTTON BLUE LACTOPHENOL SOLUTION

MSDS CAS:

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: COTTON BLUE LACTOPHENOL SOLUTION

CAS#:

Synonym: Lactophenol Cotton Blue Solution

Chemical Name: 	Not Applicable.

Chemical Formula:  Not Applicable.

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Water | 7732-18- 5 | 25-35 |
| 1,2,3-Propanetriol(glycerin) | 56-81-5 | 25-35 |
| Lactic acid | 50-21-5 | 15-25 |
| Phenol | 108-95-2 | 15-25 |

Section 3: Hazards Identification

Risk advice to man and the environment:

Toxic by inhalation, in contact with skin and if swallowed. Causes burns. Harmful: danger of serious damage to health by prolonged exposure through inhalation, in contact with skin and if swallowed. Possible risk of irreversible effects.

Section 4: First Aid Measures

General advice:

Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled:

If breathed in, move person into fresh air. If not breathing give artificial respiration Consult a physician.

In case of skin contact:

Take off contaminated clothing and shoes immediately.

Wash off with soap and plenty of water.

Take victim immediately to hospital. Consult a physician.

In case of eye contact:

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed:

Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Section 5: Fire and Explosion Data

Suitable extinguishing media:

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special protective equipment for fire-fighters:

Wear self-contained breathing apparatus for firefighting if necessary.

Section 6: Accidental Release Measures

Personal precautions:

Wear respiratory protection.

Avoid breathing vapors, mist or gas. Ensure adequate ventilation. Evacuate personnel to safe areas.

Environmental precautions:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Methods for cleaning up:

Soak up with inert absorbent material and dispose of as hazardous waste. Keep in suitable, closed containers for disposal.

Section 7: Handling and Storage

Handling:

Avoid contact with skin and eyes. Avoid inhalation of vapour or mist.

Provide appropriate exhaust ventilation at places where dust is formed. Normal measures for preventive fire protection.

Storage:

Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Containers which are opened must be carefully resealed and kept upright to prevent leakage.

Section 8: Exposure Controls/Personal Protection

Personal protective equipment

Respiratory protection:

Where risk assessment shows air-purifying respirators are appropriate use a dust mask type N95 (US) or type P1 (EN 143) respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Hand protection:

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it. Handle with gloves.

Eye protection: Tightly fitting safety goggles. Faceshield (8-inch minimum).

Section 8: Exposure Controls/Personal Protection (Continued)

Skin and body protection:

Choose body protection according to the amount and concentration of the dangerous substance at the work place.

Hygiene measures:

Avoid contact with skin, eyes and clothing. Wash hands before breaks and immediately after handling the product.

Section 9: Physical and Chemical Properties

Appearance Form		: Liquid.

Molecular Weight		: No data available.

pH					: No data available.

Melting point			: No data available.

Boiling point			: No data available.

Flash point				: Not Applicable.

Ignition temperature		: No data available.

Lower explosion limit		: No data available.

Upper explosion limit		: No data available.

Density				: 1,16 g/mLat 20 °C

Water solubility			: No data available.

Section 10: Stability and Reactivity Data

Storage stability:

Stable under recommended storage conditions.

Section 11: Toxicological Information

Acute toxicity: No data available.

Irritation and corrosion: No data available.

Sensitisation: No data available.

Chronic exposure:

IARC: 3 - Group 3: Not classifiable as to its carcinogenicity to humans ()

Potential Health Effects:

Inhalation Toxic if inhaled. Material is extremely destructive to the tissue of the mucous membranes and upper respiratory tract.

Skin Toxic if absorbed through skin. Causes skin burns.

Eyes Causes eye burns.

Ingestion Toxic if swallowed. Causes burns.

Target Organs Central nervous system, Kidney, Liver, Pancreas, Spleen.

Section 12: Ecological Information

Elimination information (persistence and degradability)

Ecotoxicity effects

Further information on ecology

Section 13: Disposal Considerations

Product:

Observe all federal, state, and local environmental regulations. Contact a licensed professional waste disposal service to dispose of this material. Dissolve or mix the material with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and scrubber.

Contaminated packaging:

Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name    : PHENOL SOLUTION

UN N°			 : 2821

ADR - Class 		 : 6.1

ADR - Group       		 : II

Sea transport (IMDG) [English only]

Proper shipping name    : PHENOL SOLUTION

UN N° 			 : 2821

IMO-IMDG - Class or division:  6.1

IMO-IMDG - Packing group: II

Air transport (ICAO-IATA) [English only]

Proper shipping name 	 : PHENOL SOLUTION

UN N°		         	 :  2821

IATA - Class or division : 6.1

IATA - Packing group    : II

Section 15: Other Regulatory Information

Labelling according to EC Directives

R-phrase(s):

R23/24/25 Toxic by inhalation, in contact with skin and if swallowed.

R34 Causes burns.

R48/20/21/22 Harmful: danger of serious damage to health by prolonged exposure through inhalation, in contact with skin and if swallowed.

R68 Possible risk of irreversible effects.

S-phrase(s):

S26 In case of contact with eyes, rinse immediately with plenty of water and seek medical advice.

S36/37/39 Wear suitable protective clothing, gloves and eye/face protection.

S45 In case of accident or if you feel unwell, seek medical advice immediately (show the label where possible).

Hazardous components which must be listed on the label:

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.