MATERIAL SAFETY DATA SHEET

CRYOLITE POWDER 98.5%

(Sodium Hexafluoroaluminate)

Extra Pure

MSDS CAS: 15096-52-3

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: CRYOLITE POWDER

CAS#:   15096-52-3

Synonym: Sodium Hexafluoroaluminate

Chemical Name: 	Cryolite Powder

Chemical Formula:  Na3AlF6

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Cryolite Powder | 15096-52-3 | 98.5 |

Section 3: Hazards Identification

Classification of the substance or mixture

Classification EC 67/548 or EC 1999/45

Classification: T; R48/23/25, Xn; R20/22, N; R51-53

Hazard Class and Category Code(s), Regulation (EC) No 1272/2008 (CLP)

Health hazards:

Specific Target Organ Toxicity - Repeated exposure - Category 1 - Danger (CLP: STOT RE 1) H372

Acute toxicity, Inhalation - Category 4 - Warning (CLP: Acute Tox. 4) H332

Acute toxicity, Oral - Category 4 - Warning (CLP: Acute Tox. 4) H302

Environmental hazards: Hazardous to the aquatic environment - Chronic hazard - Category 2 (CLP :

Aquatic Chronic 2) H411.

Label elements

Labelling EC 67/548 or EC 1999/45

R Phrase(s):

R20/22: Harmful by inhalation and if swallowed.

R48/23/25: Toxic: danger of serious damage to health by prolonged exposure through inhalation and if swallowed.

R51/53: Toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

S Phrase(s):

S22: Do not breathe dust.

S37: Wear suitable gloves.

S45: In case of accident or if you feel unwell, seek medical advice immediately (show the label when possible).

S61: Avoid release to the environment. Refer to special instructions/Safety datasheets.

Labelling Regulation EC 1272/2008 (CLP)

Hazard statements:

H302: Harmful if swallowed.

H332: Harmful if inhaled.

H372: Causes damage to organs through prolonged or repeated exposure.

H411: Toxic to aquatic life with long lasting effects.

Precautionary statements

Prevention:

P271: Use only outdoors or in a well-ventilated area.

P260: Do not breathe dust, fume, gas, mist, vapours, and spray.

P273: Avoid release to the environment.

P264: Wash thoroughly after handling.

P270: Do not eat, drink or smoke when using this product.

Section 3: Hazards Identification (Continued)

Response:

P301+P312: IF SWALLOWED: Call a POISON CENTER or doctor if you feel unwell.

P391: Collect spillage.

P304+P340: IF INHALED: Remove to fresh air and keep at rest in a position comfortable for breathing.

P314: Get medical advice if you feel unwell.

Disposal considerations:

P501: Dispose of this material and its container to hazardous or special waste collection point, in accordance with local, regional, national and/or international regulation.

Other hazards

Other hazards: The substance does not fulfil the criteria to be identified as PBT substance or vPvB substance according to Annex XIII of Regulation REACH.

Section 4: First Aid Measures

Description of first aid measures

Inhalation:

Assure fresh air breathing. Allow the victim to rest. Remove to fresh air and keep at rest in a position comfortable for breathing. Call a POISON CENTER or doctor if you feel unwell.

Skin contact:

Remove affected clothing and wash all exposed skin area with mild soap and water, followed by warm water rinse.

Eye contact:

Rinse immediately with plenty of water. Obtain medical attention if pain, blinking or redness persist.

Ingestion:

Obtain emergency medical attention. Do NOT induce vomiting. Rinse mouth. Call a POISON CENTER or doctor if you feel unwell.

Most important symptoms and effects, both acute and delayed

Symptoms relating to use:

Causes damage to organs through prolonged or repeated exposure.

Danger of serious damage to health by prolonged exposure through inhalation.

Swallowing a small quantity of this material will result in serious health hazard.

Section 4: First Aid Measures (Continued)

Indication of any immediate medical attention and special treatment needed

General information:  Never give anything by mouth to an unconscious person. If you feel unwell, seek

medical advice (show the label where possible).

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media: Foam. Dry powder. Carbon dioxide. Water spray. Sand.

Unsuitable extinguishing media: Do not use a heavy water stream.

Surrounding fires: Use water spray or fog for cooling exposed containers.

Special hazards arising from the substance or mixture

Hazardous combustion products: Under fire conditions, hazardous fumes will be present.

Advice for fire-fighters

Protection against fire: Do not enter fire area without proper protective equipment, including respiratory protection.

Special procedures: Exercise caution when fighting any chemical fire. Avoid (reject) fire-fighting water to enter environment.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures

For emergency responders: Equip cleanup crew with proper protection. Ventilate area.

For non-emergency personnel: Evacuate unnecessary personnel.

Environmental precautions

Environmental precautions: Prevent entry to sewers and public waters. Notify authorities if product enters sewers or public waters. Avoid release to the environment.

Section 6: Accidental Release Measures (Continued)

Methods and material for containment and cleaning up

Clean up methods: On land, sweep or shovel into suitable containers. Minimize generation of dust.

Store away from other materials.

Section 7: Handling and Storage

Precautions for safe handling

Handling: Use only outdoors or in a well-ventilated area. Do not breathe dust, fume, gas, mist, vapours, and spray. Wash thoroughly after handling. Do not eat, drink or smoke when using this product.

Technical protective measures: Provide good ventilation in process area to prevent formation of vapor.

Conditions for safe storage, including any incompatibilities

Storage: Keep only in the original container in a cool, well ventilated place. Keep container closed when not in use.

Storage - away from: Strong bases. Strong acids. Sources of ignition. Direct sunlight.

Section 8: Exposure Controls/Personal Protection

Exposure controls

Personal protection: Avoid all unnecessary exposure.

Respiratory protection: In case of insufficient ventilation, wear suitable respiratory equipment.

Hand protection: Wear protective gloves.

Skin protection: Wear suitable protective clothing.

Eye protection: Chemical goggles or safety glasses.

Others: When using, do not eat, drink or smoke.

Control parameters

Occupational Exposure Limits: No data available.

Section 9: Physical and Chemical Properties

Physical state at 20 °C			: Solid.

Colour					: White powder.

Odour					: Not Available.

Odour threshold				: No data available.

Molecular Weight			: 209.94 g/mole.

pH value					: Not applicable.

Melting point [°C]			: No data available.

Decomposition point [°C]		: Not Available.

Critical temperature [°C]		: Not Available.

Auto-ignition temperature [°C]	: Not Available.

Flammability (solid, gas)		: Not Available.

Flash point [°C]				: Not Available.

Boiling point [°C]				: Not Available.

Initial boiling point [°C]		: Not Available.

Final boiling point [°C]			: Not Available.

Evaporation rate				: Not Available.

Vapour pressure [20°C]			: Not Available.

Vapour pressure mm/Hg		: Not Available.

Vapour density				: Not Available.

Density [g/cm3]				: Not Available.

Relative density, gas (air=1)		: Not Available.

Relative density, liquid (water=1)	: Not Available.

Solubility in water [% weight]	: Not Available.

Solubility in water			: Not Available.

Log Pow octanol / water at 20°C	: No data available.

Solubility					: Not Available.

Viscosity at 40°C [mm2/s]		: Not Available.

Other information

Explosive properties			: Not Available.

Explosion limits - upper [%]		: Not Available.

Explosion limits - lower [%]		: Not Available.

Oxidizing properties			: No data available.

Section 10: Stability and Reactivity Data

Reactivity

Reactivity: Not established.

Chemical stability

Chemical stability: Stable under recommended storage conditions.

Possibility of hazardous reactions

Hazardous reactions: Not established.

Conditions to avoid

Conditions to avoid: Direct sunlight. Extremely high or low temperatures.

Incompatible materials

Materials to avoid: Strong acids. Strong bases.

Hazardous decomposition products

Hazardous decomposition products: Fumes. Carbon monoxide. Carbon dioxide

Section 11: Toxicological Information

Information on toxicological effects

Acute toxicity

Inhalation: Harmful if inhaled.

Dermal: Based on available data, the classification criteria are not met.

Ingestion: Harmful if swallowed.

Corrosion: Based on available data, the classification criteria are not met.

Irritation: Based on available data, the classification criteria are not met.

Sensitization: Based on available data, the classification criteria are not met.

Mutagenicity: Based on available data, the classification criteria are not met.

Carcinogenicity: Based on available data, the classification criteria are not met.

Toxic for reproduction: Based on available data, the classification criteria are not met.

STOT-single exposure: Based on available data, the classification criteria are not met.

STOT-repeated exposure: Causes damage to organs through prolonged or repeated exposure.

Aspiration hazard: Based on available data, the classification criteria are not met.

Section 12: Ecological Information

Toxicity

Toxicity information: Toxic to aquatic life with long lasting effects.

Persistence - degradability

Persistence - degradability: May cause long-term adverse effects in the environment.

Bioaccumulative potential

Bioaccumulative potential: Not established.

Mobility in soil

Mobility in soil: Not established.

Results of PBT and vPvB assessment

Results of PBT and vPvB assessment: The substance does not fulfil the criteria to be identified as PBT substance or vPvB substance according to Annex XIII of Regulation REACH.

Other adverse effects

Environmental precautions: Avoid release to the environment.

Section 13: Disposal Considerations

Waste treatment methods

General: Avoid release to the environment. Dispose in a safe manner in accordance with local/national regulations.

Dispose of this material and its container to hazardous or special waste collection point, in accordance with local, regional, national and/or international regulation.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name: ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID, N.O.S.

UN N°: 3077

Section 14: Transport Information (Continued)

H.I. nr: 90

ADR - Class: 9

Labelling - Transport: 9: Miscellaneous dangerous substances and articles.

ADR - Group: III

Sea transport (IMDG) [English only]

Proper shipping name: ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID, N.O.S.

UN N°: 3077

IMO-IMDG - Class or division: 9: Miscellaneous dangerous substances and articles.

IMO-IMDG - Packing group: III

Air transport (ICAO-IATA) [English only]

Proper shipping name: ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID, N.O.S.

UN N°: 3077

IATA - Class or division: 9: Miscellaneous dangerous substances and articles.

IATA - Packing group: III

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture

Safety, health and environmental regulations/legislation specific for the substance or mixture

: Ensure all national/local regulations are observed.

REACH Restrictions - Annex XVII: The components of this product are not subject to restrictions.

REACH Authorization - Annex XIV: The components of this product are not subject to authorization.

Chemical Safety Assessment

Chemical Safety Assessment: It has not been carried out.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.