MATERIAL SAFETY DATA SHEET

CUMENE HYDROPEROXIDE 70% SOLUTION

MSDS CAS: 80-15-9

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: CUMENE HYDROPEROXIDE SOLUTION

CAS#:80-15-9

Synonym:a,a-Dimethylbenzylhydroperoxide

Chemical Name:	CumeneHydroperoxideSolution

Chemical Formula:C9H12O2

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Substance name | Contents | CAS No | EC No | Annex No | REACH Ref. | Classification |
| --- | --- | --- | --- | --- | --- | --- |
| Cumenehydroperoxide | > 50 < 75 % | 80-15-9 | 201-254-7 | ----- | ---- | T; R23 Xn; R20 C; R34 N; R51 ------------------------------- Acute Tox. 4 (skin) _ H312 Acute Tox. 4 (inhal) _ H332 Acute Tox. 4 (oral) _ H302 |

Section 3: Hazards Identification

Classification of the substance or mixture

Classification EC 67/548 or EC 1999/45

Classification: T; R23 Xn; R20 C; R34 N; R51

Hazard Class and Category Code(s), Regulation (EC) No 1272/2008 (CLP)

Health hazards:

Acute toxicity, Oral - Category 4 - Warning (CLP: Acute Tox. 4) H302

Acute toxicity, Inhalation - Category 4 - Warning (CLP: Acute Tox. 4) H332

Acute toxicity, dermal - Category 4 - Warning (CLP: Acute Tox. 4) H312

Label elements

Labelling EC 67/548 or EC 1999/45

R Phrase(s):

R21: Harmful in contact with skin.

R22: Harmful if swallowed.

R23: Toxic by inhalation.

R34: Causes burns.

R48: Danger of serious damage to health by prolonged exposure.

R20: Harmful by inhalation.

R22: Harmful if swallowed.

R51: Toxic to aquatic organisms.

R53: May cause long-term adverse effects in the aquatic environment.

S Phrase(s):

S14: Keep away from.

S36: Wear suitable protective clothing.

S37: Wear suitable gloves.

S39: Wear eye/face protection.

S45: In case of accident or if you feel unwell, seek medical advice immediately (show the label when possible).

S50: Do not mix with.

S61: Avoid release to the environment. Refer to special instructions/Safety datasheets.

Contains:Cumenehydroperoxide.

Labelling Regulation EC 1272/2008 (CLP)

Hazard statements:

H302: Harmful if swallowed.

H312: Harmful in contact with skin.

H332: Harmful if inhaled.

Section 3: Hazards Identification (Continued)

Precautionary statements

Prevention:

P280: Wear protective gloves, protective clothing, eye protection, face protection.

P271: Use only outdoors or in a well-ventilated area.

P261: Avoid breathing dust, fume, gas, mist, vapours, and spray.

P264: Wash thoroughly after handling.

P270: Do not eat, drink or smoke when using this product.

Response:

P302+P352: IF ON SKIN: Wash with plenty of soap and water.

P301+P312: IF SWALLOWED: Call a POISON CENTER or doctor if you feel unwell.

P304+P340: IF INHALED: Remove to fresh air and keep at rest in a position comfortable for breathing.

P363: Wash contaminated clothing before reuse.

Disposal considerations:

P501: Dispose of this material and its container to hazardous or special waste collection point, in accordance with local, regional, national and/or international regulation.

Contains:Cumenehydroperoxide.

Other hazards

Other hazards:The substance does not fulfil the criteria to be identified as PBT substance or vPvBsubstance according to Annex XIII of Regulation REACH.

Section 4: First Aid Measures

Description of first aid measures

Inhalation:

Assure fresh air breathing. Allow the victim to rest. Remove to fresh air and keep at rest in a position comfortable for breathing. Call a POISON CENTER or doctor if you feel unwell.

Skin contact:

Remove affected clothing and wash all exposed skin area with mild soap and water, followed by warm water rinse. Immediately call a POISON CENTER or doctor. Specific treatment (see on this label). Wash with plenty of soap and water. Wash contaminated clothing before reuse.

Eye contact:

Rinse immediately with plenty of water. Obtain medical attention if pain, blinking or redness persist.

Section 4: First Aid Measures (Continued)

Ingestion:

Obtain emergency medical attention. Call a POISON CENTER or doctor if you feel unwell. Do NOT induce vomiting. Rinse mouth.

Most important symptoms and effects, both acute and delayed

Symptoms relating to use: Repeated exposure to this material can result in absorption through skin causing significant health hazard.

Danger of serious damage to health by prolonged exposure through inhalation. Swallowing a small quantity of this material will result in serious health hazard.

Indication of any immediate medical attention and special treatment needed

General information: Never give anything by mouth to an unconscious person. If you feel unwell, seekmedical advice (show the label where possible).

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media: Foam. Dry powder. Carbon dioxide. Water spray. Sand.

Unsuitable extinguishing media: Do not use a heavy water stream.

Surrounding fires: Use water spray or fog for cooling exposed containers.

Special hazards arising from the substance or mixture

Hazardous combustion products: Under fire conditions, hazardous fumes will be present.

Advice for fire-fighters

Protection against fire: Do not enter fire area without proper protective equipment, including respiratory protection.

Special procedures: Exercise caution when fighting any chemical fire. Avoid (reject) fire-fighting water to enter environment.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures

For emergency responders: Equip cleanup crew with proper protection. Ventilate area.

For non-emergency personnel: Evacuate unnecessary personnel.

Section 6: Accidental Release Measures (Continued)

Environmental precautions

Environmental precautions: Prevent entry to sewers and public waters. Notify authorities if product enterssewers or public waters.

Methods and material for containment and cleaning up

Clean up methods: Soak up spills with inert solids, such as clay or diatomaceous earth as soon as possible. Store away from other materials. Collect spillage.

Section 7: Handling and Storage

Precautions for safe handling

Handling:

Wash thoroughly after handling. Do not eat, drink or smoke when using this product. Avoid breathing dust, fume, gas, mist, vapours, and spray. Use only outdoors or in a well-ventilated area.

Technical protective measures: Provide good ventilation in process area to prevent formation of vapour.

Conditions for safe storage, including any incompatibilities

Storage: Keep only in the original container in a cool, well ventilated place. Keep container closed when not in use.

Storage - away from:Strong bases. Strong acids. Sources of ignition. Direct sunlight.

Section 8: Exposure Controls/Personal Protection

Exposure controls

Personal protection: Avoid all unnecessary exposure.

Respiratory protection: In case of insufficient ventilation, wear suitable respiratory equipment.

Hand protection: Wear protective gloves.

Eye protection: Chemical goggles or safety glasses.

Others: When using, do not eat, drink or smoke.

Control parameters

Occupational Exposure Limits: No data available.

Section 9: Physical and Chemical Properties

Physical state at 20 °C		: Liquid.

Colour				: Colorless

Odour				: Characteristic.

Odour threshold			: No data available.

Molecular Weight		:152.19 g/mole

pH value				: No data available.

Melting point [°C]		: No data available.

Decomposition point [°C]	: Not Applicable.

Critical temperature [°C]	: No data available.

Auto-ignition temperature [°C]: No data available.

Flammability (solid, gas)	: Flammable.Flammable liquid and vapour.

Flash point [°C]			: 79 °C - closed cup

Boiling point [°C]			: Not Applicable.

Initial boiling point [°C]	: No data available.

Final boiling point [°C]		: 100 - 101 °C at 11 hPa - lit

Evaporation rate			: Not Applicable.

Vapour pressure [20°C]		: < 0,04hPa at 20 °C

Vapour pressure mm/Hg	: Not Applicable.

Vapour density			: 5,25 - (Air = 1.0)

Density [g/cm3]			: Not Applicable.

Relative density, gas (air=1)	: Not Applicable.

Relative density, liquid (water=1): 1,03 g/mL

Solubility in water [% weight]: Not Applicable.

Solubility in water		: No data available.

Log Pow octanol / water at 20°C: No data available.

Solubility				: No data available.

Viscosity at 40°C [mm2/s]	: No data available.

Section 10: Stability and Reactivity Data

Reactivity

Reactivity: Not established.

Chemical stability

Chemical stability: Stable under recommended storage conditions.

Section 10: Stability and Reactivity Data (Continued)

Possibility of hazardous reactions

Hazardous reactions: Not established.

Conditions to avoid

Conditions to avoid: Direct sunlight. Extremely high or low temperatures.

Incompatible materials

Materials to avoid: Strong acids. Strong bases.

Hazardous decomposition products

Hazardous decomposition products:Fumes. Carbon monoxide. Carbon dioxide.

Section 11: Toxicological Information

Information on toxicological effects

Toxicity information: The product has been not fully tested. The calculated risk has been done under therequirements of the EU regulations.

Acute toxicity:  LD50 Oral - rat - 382 mg/kg Remarks: Kidney, Ureter, Bladder: Hematuria

Inhalation: LC50 Inhalation - rat - 4 h - 220 ppm Remarks: Lungs, Thorax, or Respiration: Dyspnea

Dermal: LD50 Dermal - rat - 500 mg/kg Remarks: Behavioral:Convulsions or effect on seizure threshold. Kidney, Ureter, Bladder:Hematuria

Ingestion: Harmful if swallowed

Corrosion: Based on available data, the classification criteria are not met.

Irritation: Based on available data, the classification criteria are not met.

Sensitization: Based on available data, the classification criteria are not met.

Mutagenicity: Based on available data, the classification criteria are not met.

Carcinogenicity: Based on available data, the classification criteria are not met.

Toxic for reproduction: Based on available data, the classification criteria are not met.

STOT-single exposure: Based on available data, the classification criteria are not met.

STOT-repeated exposure: Based on available data, the classification criteria are not met.

Aspiration hazard: Based on available data, the classification criteria are not met.

Section 12: Ecological Information

Toxicity

Toxicity information: The product has been not fully tested. The calculated risk has been done under the requirements of the EU regulations. Toxicity to fish LC50 - Oncorhynchus mykiss (rainbow trout) - 3,9 mg/L - 96 h Toxicity to daphnia and other aquatic invertebrates: LC50 - Daphnia magna (Water flea) - 16 mg/L - 24h

Persistence - degradability

Persistence - degradability: Biodegradable.

Bioaccumulative potential

Bioaccumulative potential: Not established.

Mobility in soil

Mobility in soil: Not established.

Results of PBT and vPvB assessment

Results of PBT and vPvB assessment: The substance does not fulfil the criteria to be identified as PBT substance or vPvB substance according to Annex XIII of Regulation REACH.

Other adverse effects

Environmental precautions: Avoid release to the environment.

Section 13: Disposal Considerations

Waste treatment methods

General: Avoid release to the environment. Dispose in a safe manner in accordance with local/national regulations.

Dispose of this material and its container to hazardous or special waste collection point, in accordance with local, regional, national and/or international regulation.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name: ORGANIC PEROXIDE TYPE F, LIQUID

UN N°: 3109

H.I. nr: 539

ADR - Class: 5.2

Labelling - Transport: 5.2: Organic peroxide.

Sea transport (IMDG) [English only]

Proper shipping name: ORGANIC PEROXIDE TYPE F, LIQUID

UN N°: 3109

IMO-IMDG - Class or division: 5.2: Organic peroxide.

Air transport (ICAO-IATA) [English only]

Proper shipping name: ORGANIC PEROXIDE TYPE F, LIQUID

UN N°: 3109

IATA - Class or division: 5.2: Organic peroxide.

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture

Safety, health and environmental regulations/legislation specific for the substance or mixture

: Ensure all national/local regulations are observed.

REACH Restrictions - Annex XVII:The components of this product are not subject to restrictions.

REACH Authorisation - Annex XIV:The components of this product are not subject to authorization.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.