MATERIAL SAFETY DATA SHEET

DIACETYL 98%

(For Synthesis) (2,3-butanedione)

MSDS CAS: 431-03-8

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: DIACETYL

CAS#:   431-03-8

Synonym: 2,3-Butanedione

Chemical Name: Diacetyl

Chemical Formula:  C4H6O2

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS NO. | % by Weight |
| --- | --- | --- |
| Diacetyl | 431-03-8 | 100 |

Section 3: Hazards Identification

Appearance: yellow to green liquid. Flash Point: 6 deg C.

Warning!

Flammable liquid and vapor. Causes eye, skin, and respiratory tract irritation. Harmful if inhaled or swallowed. May cause central nervous system depression. May cause lung damage. Marine pollutant.

Target Organs:

Lungs, respiratory system, eyes, skin.

Potential Health Effects

Eye: Causes eye irritation. Causes redness and pain. May cause chemical conjunctivitis and corneal damage.

Vision may become blurred.

Skin:

Causes moderate skin irritation. Causes redness and pain. May be harmful if absorbed through the skin.

Ingestion:

Harmful if swallowed. May cause gastrointestinal irritation with nausea, vomiting and diarrhea.

Ingestion of large amounts may cause CNS depression.

Inhalation:

Causes respiratory tract irritation. May cause lung damage. Vapors may cause dizziness or Suffocation. Harmful if inhaled. Inhalation of vapors may cause drowsiness and dizziness. High exposure to but an edione may cause headache, drowsiness, lack of coordination and seizures. May cause a feeling of tightness in the chest.

Chronic:

Chronic exposure may cause lung damage. Repeated exposure may damage the red blood cells causing anemia.2,3-Butanedione has been linked to bronchiolitis obliterans, a disease characterized by inflammation and scarring in the smallest airways of the lungs.

Section 4: First Aid Measures

Eyes:

Immediately flush eyes with plenty of water for at least 15 minutes, occasionally lifting the upper and lower eyelids. Get medical aid immediately.

Skin:

Get medical aid. Flush skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes.

Section 4: First Aid Measures (Continued)

Ingestion:

If victim is conscious and alert, give 2-4 cupfuls of milk or water. Never give anything by mouth to an unconscious person. Get medical aid.

Inhalation:

Remove from exposure and move to fresh air immediately. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical aid.

Notes to Physician: Treat symptomatically and supportively.

Section 5: Fire and Explosion Data

General Information:

As in any fire, wear a self-contained breathing apparatus in pressure-demand,

MSHA/NIOSH (approved or equivalent), and full protective gear. Vapors may form an explosive mixture with air. During a fire, irritating and highly toxic gases may be generated by thermal decomposition or combustion. Containers may explode in the heat of a fire. Flammable liquid and vapor. Vapors are heavier than air and may travel to a source of ignition and flash back. Vapors can spread along the ground and collect in low or confined areas. Hazardous polymerization may occur under fire conditions.

Extinguishing Media:

Water may be ineffective. Water may spread fire. If water is the only media available, use in flooding amounts. Do NOT use straight streams of water. Use water fog, dry chemical, and carbon dioxide or alcohol type foam.

Flash Point: 6 deg C (42.80 deg F) Autoignition.

Temperature: 365 deg C (689.00 deg F)

Explosion Limits, Lower: 2.4% Upper: 13.0%

NFPA Rating : (estimated) Health: 2; Flammability: 3; Instability: 1

Section 6: Accidental Release Measures

General Information:

Use proper personal protective equipment as indicated in Section.

Section 6: Accidental Release Measures (Continued)

Spills/Leaks:

Absorb spill with inert material (e.g. vermiculite, sand or earth), then place in suitable container. Avoid runoff into storm sewers and ditches which lead to waterways. Clean up spills immediately, observing precautions in the Protective Equipment section. Remove all sources of ignition. Use a spark-proof tool. A vapor suppressing foam may be used to reduce vapors.

Section 7: Handling and Storage

Handling:

Wash thoroughly after handling. Remove contaminated clothing and wash before reuse. Ground and bond containers when transferring material. Use spark-proof tools and explosion proof equipment. Avoid contact with eyes, skin, and clothing. Do not breathe dust, mist, or vapor. Empty containers retain product residue, (liquid and/or vapor), and can be dangerous. Do not pressurize, cut, weld, braze, solder, drill, grind, or expose empty containers to heat, sparks or open flames. Keep away from heat, sparks and flame. Use only with adequate ventilation or respiratory protection.

Storage:

Keep away from sources of ignition. Store in a tightly closed container. Refrigerator/flammables.

Section 8: Exposure Controls/Personal Protection

Engineering Controls:

Facilities storing or utilizing this material should be equipped with an eyewash facility and a safety shower. Use process enclosure, local exhaust ventilation, or other engineering controls to control airborne levels.

Exposure Limits

Chemical Name ACGIHNIOSHOSHA- Final PELs

2,3-Butanedione none listed none listed none listed

OSHA Vacated PELs: 2,3-Butanedione: No OSHA

Vacated PELs are listed for this chemical.

Personal Protective Equipment

Eyes:

Wear chemical splash goggles.

Section 8: Exposure Controls/Personal Protection (Continued)

Skin:

Wear appropriate protective gloves to prevent skin exposure.

Clothing:

Wear appropriate protective clothing to prevent skin exposure.

Respirators:

Follow the OSHA respirator regulations found in 29 CFR 1910.134 or European Standard

EN 149. Use a NIOSH/MSHA or European Standard EN 149 approved respirator if exposure limits are exceeded or if irritation or other symptoms are experienced.

Section 9: Physical and Chemical Properties

Physical State			: Liquid

Colour				: Deep yellow

Molecular Weight		: 86.09 g/mole

Odor					: strong rancid - chlorine-like - butter-like

pH					: 3.2

Vapor Pressure			: 39 mm Hg @ 20 deg C

Vapor Density			: 3 (air=1)

Evaporation Rate			: Not available.

Viscosity				: Not available.

Boiling Point			: 88 deg C @ 760 mm Hg

Freezing/Melting Point		:-4 deg C

Decomposition Temperature	: > 110 deg C

Solubility				: Soluble.

Specific Gravity/Density	: 9850

Section 10: Stability and Reactivity Data

Chemical Stability:

Stable under normal temperatures and pressures. Material is heat sensitive and may decompose at elevated temperatures.

Section 10: Stability and Reactivity Data (Continued)

Conditions to Avoid:

Light, ignition sources, excess heat.

Incompatibilities with Other Materials:

Metals, oxidizing agents, reducing agents, acids, strong bases.

Hazardous Decomposition Products: Carbon monoxide, carbon dioxide.

Hazardous Polymerization: Has not been reported.

Section 11: Toxicological Information

Epidemiology:

2,3-Butanedione has been associated with lung disease, bronchiolitis obliterans, in workers at a microwave-popcorn plant. Lung damage was so severe a double lung transplant is needed in one of those afflicted.

Teratogenicity: No information found.

Reproductive Effects: No information found.

Mutagenicity: Mutagenic effects have occurred in experimental animals.

Neurotoxicity: No information found.

Section 12: Ecological Information

Ecotoxicity: No data available. No information available.

Environmental: Biodegradable.

Physical: No information available.

Other: Marine pollutant.

Section 13: Disposal Considerations

Chemical waste generators must determine whether a discarded chemical is classified as a hazardous waste. US EPA guidelines for the classification determination are listed in 40 CFR Parts 261.3. Additionally, waste generators must consult state and local hazardous waste regulations to ensure complete and accurate classification.

RCRAP-Series: None listed.

RCRAU-Series: None listed.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name: BUTANEDIONE

UN N°: 2346

H.I. nr: 33

ADR - Class: 3

Labelling - Transport: 3 : Flammable liquid.

ADR - Group: II

Sea transport (IMDG) [English only]

Proper shipping name: BUTANEDIONE

UN N°: 2346

IMO-IMDG - Class or division: 3 : Flammable liquid.

IMO-IMDG - Packing group: II

Air transport (ICAO-IATA) [English only]

Proper shipping name: BUTANEDIONE

UN N°: 2346

IATA - Class or division: 3 : Flammable liquid.

IATA - Packing group: II

Section 15: Other Regulatory Information

TSCA:

CAS# 431-03-8 is listed on the TSCA inventory.

Health & Safety Reporting List:

None of the chemicals are on the Health & Safety Reporting List.

Chemical Test Rules:

None of the chemicals in this product are under a Chemical Test Rule.

Section 12b:

None of the chemicals are listed under TSCA Section 12b.

TSCA Significant New Use Rule:

None of the chemicals in this material have a SNUR under TSCA.

CERCLA Hazardous Substances and corresponding RQs:

None of the chemicals in this material have an RQ.

SARA Section 302 Extremely Hazardous Substances:

None of the chemicals in this product have a TPQ.

SARA Codes:

CAS # 431-03-8: immediate, fire.

Section 313:

No chemicals are reportable under Section 313.

Clean Air Act:

This material does not contain any hazardous air pollutants.

This material does not contain any Class 1 Ozone depletors.

This material does not contain any Class 2 Ozone depletors.

Clean Water Act:

None of the chemicals in this product are listed as Hazardous Substances under the CWA.

None of the chemicals in this product are listed as Priority Pollutants under the CWA.

None of the chemicals in this product are listed as Toxic Pollutants under the CWA.

OSHA:

None of the chemicals in this product are considered highly hazardous by OSHA.

Section 15: Other Regulatory Information (Continued)

STATE

CAS# 431-03-8 can be found on the following state right to know lists: New Jersey, Pennsylvania, and Massachusetts.

Risk Phrases:

R 11 Highly flammable.

R 20/22 Harmful by inhalation and if swallowed.

R 36/37/38 Irritating to eyes, respiratory system and skin.

Safety Phrases:

S 16 Keep away from sources of ignition - No smoking.

S 26 In case of contact with eyes, rinse immediately with plenty of water and seek medical advice.

S 33 Take precautionary measures against static discharges.

S 36/37/39 Wear suitable protective clothing, gloves and eye/face protection.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.